PIT TERMINAL CONFIG v4.16

Written by James Berry

Introduction

~~~~~~~~~~~~

The PIT Terminal configuration allows the user to control the modem

and hardware settings used by the Pit Term.

The Pit Term accepts the com port and the baud rate as parameters

when it is run. The settings in this config program allow the user to

control the comport addresses, baud rate, flow control, and sound

card settings.

Settings

~~~~~~~~

1. Com Port Addressing

~~~~~~~~~~~~~~~~~~~

This section will allow you to change the default settings

for your com ports. Normally these do not require any

changes, as they are already set to the default addresses

used by 99% of all systems.

Notice that these settings are used internally by only our

terminal. They will have no effect on your communication

software nor DOS.

2. Baud Rate Setting

~~~~~~~~~~~~~~~~~

We provide the ability to lock the baud rate of your modem to

almost all of the baud rates possible for a modem.

For most systems the best selection is "Auto-Detect".

This option forces the Pit Terminal to run at the baud rate

at which the BBS is already connected to the caller.

Any other option results in THE PIT setting the baud rate on

the modem to the rate specified.

3. Flow Control

~~~~~~~~~~~~

This selection has only three options, "Auto-Detect", "None"

or "CTS/RTS Flow Control". "Auto-Detect" is the default

setting, and should work in most cases. The program detects

baud rate the modem is running at, and turns on the CTS/RTS

flow control if a speed greater than 2400 is detected.

If you are running a modem that is faster than 2400 you are

REQUIRED to have CTS/RTS turned on. Under this setting, the

game uses the CTS and RTS pins on the modem to regulate the

flow of data across the phone line.

Without the CTS/RTS set on, the result would be that no data

would flow at all.

2

4. 16550 FIFO Buffer

~~~~~~~~~~~~~~~~~

This setting allows you to select the size of the "First-In

First-Out" buffer on your 16550 chip. This buffer is used

primarily by high speed modems to achieve an effective

transfer rate greater than the normal 19,200 baud rate.

The safest setting is "Use default FIFO Settings". However,

if you know what your BBS is using, you can always change it

to 1, 4, 8, or 14 byte buffers.

5. Music        -<NOT CURRENTLY USED>-

~~~~~

MIDI music will be played through a sound card, the pc

speaker, or not at all. You must choose the best choice for

your system.

Notice that the Sound Blaster Pro 16 is not given as a

choice. At the time this game was written we did not own a SB

16. We believe that it will only function if "Sound Blaster

v1.5" is chosen as the sound card type.

6. Sound FX

~~~~~~~~

Sound Effects will be played through a sound card, the pc

speaker, or not at all. You must choose how your sound

effects will be played. (See the notes about Sound Blaster

Pro 16 above.)

7. Sound Blaster Card Settings

~~~~~~~~~~~~~~~~~~~~~~~~~~~

This selection will allow you to change your Sound Blaster

settings from the defaults. If you have your sound card dip

switches set to alternate settings, you will have to change

these settings to match it. We provide you the standard

choices of IRQ's and Addresses provided by the manufacturer.

8. Adlib Card Settings

~~~~~~~~~~~~~~~~~~~

This section will allow you to change your Adlib settings

from the defaults. If you have your sound card dip switch set

to alternate settings, you will have to change these settings

to match it. We have provided you with the standard choices

of IRQ's and Addresses available from the manufacturer.

9. Mouse

~~~~~

This option allows us to either auto-detect the existant of a

mouse and utilize it, or claim to have none.

3

10. Joystick    -<NOT CURRENTLY USED>-

~~~~~~~~

This option allows us to either activate the use of a

joystick or not activate it.

4