PCAD2APR   Version 1.5

This converter will convert from a PCAD .APR aperture file to GraphiCode's

.APR aperture file (although the files have the same extension, they have

very different formats).  The converter can be used to convert single

files or groups of files.

Version 1.5 adds support for thermals PCAD version 7.  Since PCAD only

gives an O.D., we calculate as ID=.8*OD, Gap = .15*OD.

Version 1.4 adds support for PCAD version 7.

Version 1.3 adds support for PCAD version 6.

usage: PCAD2APR <aperture file> [<outfile>]

Example:

To convert "ALIST1.APR" to "NEWALIST.APR" type:

PCAD2APR ALIST1 NEWALIST

For PCAD version 5 and earlier, for laser plotting type:

PCAD2APR -L ALIST1 NEWALIST

This creates the 255 position "Laser" aperture list.

To convert several files at once type:

PCAD2APR *.APR

GraphiCode aperture files will then be produced with the extension

.AP_  to distinguish them from the original PCAD .APR files.

Special apertures, those other than ROUND, DRAW, and SQUARE are

declared as custom apertures using "TRIANGLE.CAP".  For PCAD

version 6, we convert to TARGET to preserve any diameter info.  If

you are using one of these apertures, you will need to use APER to

create the desired custom aperture, or choose from the other aperture

types available in our system.

This is the older format for the PCAD aperture list file:

V004

PCAD.APR

1,5,6,0

2,10,6,0

3,12,6,0

4,25,6,0

5,50,6,0

6,75,6,0

7,100,6,0

8,1,0,1

9,60,6,1

10,60,7,1

11,70,6,1

12,70,7,1

13,75,6,1

14,75,7,1

15,50,6,1

16,55,6,1

17,80,6,1

18,85,6,1

19,90,6,1

This is a newer format:

#$#     P-CAD Gerber Aperture Table

UNIT = METRIC (mm)

#

#                       Outside           Inside

#                   ---------------   ---------------

#                   Sides/                                       Type/

#Apr D-             Width/   Height   Width/   Height   Rotation Polygon

#No. code Shape     Diameter          Diameter         (degrees) point

1    10   ROUND     0.5080   N/A      0        0        N/A      FLASH

2    11   ROUND     1.1176   N/A      0        0        N/A      FLASH

3    12   ROUND     1.3208   N/A      0        0        N/A      FLASH

4    13   ROUND     1.3970   N/A      0        0        N/A      FLASH

5    14   ROUND     1.4986   N/A      0        0        N/A      FLASH

6    15   ROUND     1.6002   N/A      0        0        N/A      FLASH

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.