REP2APR   Version 1.7

This converter will convert from a PADS-PCB .REP or .PHO aperture file to a

GraphiCode .APR aperture file.  The converter can be used to convert single

files or groups of files.

usage: REP2APR <aperture file> [<outfile>]

Example:

To convert the aperture file "ALIST1.REP" for use with GraphiCode

products, type:

REP2APR ALIST1

This creates the GraphiCode aperture wheel file "ALIST1.APR".

To convert "ALIST1.REP" to "NEWALIST.APR" type:

REP2APR ALIST1 NEWALIST

To convert several aperture files type:

REP2APR *.REP

Version 1.7 allows uninitialized shape codes - formerly it would

stop reading if it encountered a blank shape name.

Version 1.5 adds the capability to work with PADS 2000

report files that are in METRIC or INCH rather than the

default of MILS.  To convert a metric file type:

To convert a metric file from PADS 2000 type:

REP2APR MM *.REP

To convert a inch file from PADS 2000 type:

REP2APR IN *.REP

This new version of REP2APR allows the conversion of the

aperture list as output by PADS DEFAULTS, PHO-PLOT routine.

This capability provides for the easy generation of one

single .APR file for each board loaded in GC-PREVUE. Using

this approach will make it much easier to load several

boards in GC-PREVUE even though each board may use many

different apertures.

For different a .APR list for each .PHO type:

REP2APR filename

or to do them all at once type:

REP2APR *.PHO

This command will convert each PADS .REP file as entered in

the filename to a GC-PREVUE .APR file (for example, foo.REP

creates foo.APR).

To generate a master GC-PREVUE .APR file for a whole job:

1. Generate Gerbers for the job so that the batch.dat for

the job contains all the apertures needed.

2. Leave PCB (or PADS2000) and then restart the program

with the printer turned OFF. Restart the program and go into

the CAM routine and enter the directory for the job needing

the .APR file.

3. Enter into the DEFAULTS, PHO-PLOT routine. At this point

you should see the apertures listed for the job, click on

PROCEED. Answer NO to updating the DEFAULTS.PAD file unless

you want to store these apertures for use in future jobs.

Answer YES to PRINT APERTURE LIST?. This will store the

aperture list in the PCB.ERR (or P2000.ERR) file.

If you did not turn of your printer the list will just print.

4. Exit the program and go into the \PADS\FILES directory

(or use PEDIT for this directory). Examine the .ERR file

and be sure that the ONLY thing it contains is the aperture

list output by the DEFAULTS, PHO-PLOT routine. Any other data

will probably confuse REP2APR. Any aperture positions that

contain zeros will not cause problems provided that they are

not BETWEEN positions that are used. Save this file and exit

the editor.

5. At the DOS prompt enter:

REP2APR PCB.ERR filename.APR

This will generate a .APR file that will contain all the

apertures needed by this job.

It is recommended that the file name used for the .APR file

match that of the job so that it is easy to know what .APR

goes with which .PHO groups when using GC-PREVUE.

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.