Personal Communications Library

For Visual BASIC (Windows)

(PCLVBW)

USERS MANUAL

Version  1.0

Feb 11, 1996

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1996

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice 205-881-4630

FAX   205|880|0925

BBS   205-880-9748

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

PCLVBW Users Manual                                               Page 1

C O N T E N T S

Chapter                                                   Page

1.0 Introduction................................................3

1.1 User Support............................................4

1.2 ASP Ombudsman...........................................4

1.3 Disabling Windows Ports.................................5

1.4 Installation............................................6

2.0 Library Organization........................................7

2.1 Configuration...........................................7

2.2 Initialization & Termination............................7

2.3 Modem Control & Status..................................8

2.4 Serial I/O..............................................8

2.5 Error Detection.........................................9

2.6 General Support.........................................9

3.0 Library Overview...........................................10

3.1 Using the Library......................................10

3.2 Example Programs.......................................10

3.3 Door Programs..........................................11

3.4 BBS Programs...........................................11

3.5 Compiling & Linking....................................11

4.0 Talking to Your Modem......................................12

4.1 Modem Standards........................................12

4.2 Flow Control...........................................13

4.3 Modem I/O (MIO)........................................14

4.3.1 MIO Introduction.................................14

4.3.2 MIO Functions....................................15

4.3.2.1 mioDriver................................15

4.3.2.2 mioBreak.................................15

4.3.2.3 mioSendTo................................15

4.3.2.4 mioWaitFor...............................15

4.3.2.5 mioQuiet.................................16

4.4.3 MIO Summary......................................16

4.4 Modem Initialization...................................17

5.0 Problems...................................................18

6.0 Serial Communications......................................19

6.1 Communications Basics..................................19

6.2 Standard Port Addresses................................20

6.3 Running 3 or 4 Ports Concurrently......................21

6.4 Using Multiport Cards..................................22

6.4.1 The DigiBoard....................................22

6.4.2 The BOCA Board...................................22

6.5 Transmitter Interrupts.................................23

6.6 RS232 Signals..........................................24

6.7 National 8250, 6450, and 16550 UARTs...................25

6.8 Register Summary.......................................26

7.0 [reserved section number]

8.0 Legal Issues...............................................28

8.1 Registration...........................................28

8.2 License................................................28

8.3 Warranty...............................................28

9.0 Summary....................................................30

9.1 Revision History.......................................30

9.2 Function Summary.......................................30

9.3 Further Reading........................................31

10.0 Other MarshallSoft Computing Products .....................31

PCLVBW Users Manual                                               Page 2

1.0 Introduction

The  Personal  Communications Library for Visual BASIC / Windows (PCLVBW) is

an asynchronous communications library  designed  for  experienced  software

developers programming in Windows using Visual BASIC.

PCLVBW  controls  the  serial  port  hardware directly.  It does not use the

Windows serial communications API functions. This allows up to 20 ports with

a maximum baud rate of 115,200 baud. The PCL features:

o 36 communications and support functions.

o Supports the high performance 16550 UART.

o Use IRQ2 to IRQ15 with any UART address.

o Supports the PC/4 and PC/8 DigiBoard.

o Supports the BOCA BB1004, BB1008, and BB2016 boards.

o Supports hardware (RTS/CTS) flow control.

o Interrupt driven receiver & (optionally) transmitter.

o Supports 300 baud to 115,200 baud.

o Supports COM1 through COM8 (through COM20 with multiport boards)

o Adjustable receive queues from 8 bytes to 32 KB.

o Control-BREAK error exit.

o 18 communications error conditions trapped.

o Allows 4 ports to run concurrently (more with multiport boards).

o Complete modem control & status.

o Written in assembly language for small size & high speed.

o Does NOT use the Windows 3.X communications driver.

Why should you buy PCLVBW ?  Several good reasons are:

COMPLETE - PCLVBW is complete since it provides absolute  control  of  the

serial ports (including the high performance 16550).

COMPACT - PCLVBW  is  very  compact at less than 8 KB.  Your application

doesn't carry a lot of excess code.

FAST - PCLVBW is fast since it runs at 38400 baud on even slow 286

PCs and at 115200 baud on most everything else.

SUPPORT - If you get stuck, you talk to the  programmer  that  wrote the

code, not a person hired to answer the phone.

BBS  - A BBS is available (2400 to 14,400  baud,  N81)  in  order  to

provide immediate support as necessary.

NEWSLETTER - Our newsletter discusses "COMM  TALK"  is  published  quarterly

(available  online) which discusses communications problems and

solutions.

PRICE - You get PCLVBW for a very reasonable price!

UPGRADES - Once you buy PCLVBW, you can always update to  the  most recent

version very inexpensively ($20 plus shipping).

Be  sure  to  read  the  README.DOC file. It contains last release notes and

other last minutes changes and/or additions.

PCLVBW Users Manual                                               Page 3

1.1 User Support

We want you to be successful in developing your applications  using  PCLVBW!

We depend on our customers to let us know what they need in a communications

library.   This  means we are committed to providing the best communications

library that we can. If you have any suggestions or comments, please let  us

know!

We  provide  customer  support  for registered customers by voice, FAX, BBS,

email, and US mail.  We provide limited support for  unregistered  users  by

voice, email, and BBS only.

If  you  are  having a problem using PCLVBW, call us at 205-881-4630 between

1:30 PM and 9:30 PM (CST) Monday through Friday. You can also call at  other

times and leave a message, and call back later for a reply. Registered users

(ONLY) can also FAX us at 205-880-0925 at any time (24 hours).

However,  we  can  only  answer  questions  with respect to using the PCLVBW

library.  We cannot help you program your application, but we'll be glad  to

discuss it with you.

You  may  also  call  our User Support BBS (2400 to 14400 baud, no parity, 8

data bits, 1 stop bit) at 205-880-9748 and leave a message  (address  it  to

the SYSOP).  We will usually have a reply ready for you within 24 hours.

The BBS is available 24 hours per day. All files are in standard ZIP format.

The BBS will contain  the  latest  shareware  version  of  all  MarshallSoft

Computing products as well as related files such as:

BUGS.ZIP     -  Bug report.

PRODUCTS.ZIP -  List of all shareware products.

The  MarshallSoft  Computing,  Inc.   newsletter  "Comm  Talk"  is published

quarterly.  It discusses various communications problems and solutions using

PCLVBW as well as related information.

The latest copy of our newsletter can be found on our support BBS, anonymous

FTP site, or on our Web site.

BBS      : 205-880-9748            (look in file area "Newsletters")

Anon FTP : ftp.traveller.com       (directory  /pub/users/msc)

Web site : www.traveller.com/~msc/

1.2 ASP Ombudsman

MarshallSoft  Computing,  Inc.   is a member of the Association of Shareware

Professionals (ASP).  ASP wants to make sure that  the  shareware  principle

works  for  you.   If  you are unable to resolve a shareware-related problem

with an ASP member by contacting the member directly, ASP  may  be  able  to

help.  The  ASP  Ombudsman can help you resolve a dispute or problem with an

ASP member, but does not provide technical support  for  members'  products.

Please  write  to  the  ASP  Ombudsman  at 545 Grover Road, Muskegon, MI USA

49442-9427, Fax 616-788-2765, or send a CompuServe  message  via  CompuServe

Mail to ASP Ombudsman 70007,3536.

PCLVBW Users Manual                                               Page 4

1.3 Disabling Windows Ports

The  Windows operating system virtualizes all serial ports that it controls.

This  means  that  Windows  intercepts  reads  and writes to hardware ports,

resulting in significan slowdown. And, when Windows virtualizes a port,  the

UART FIFO control register (FCR) can not be modified.

Since PCLVBW also controls the hardware directly, it is best  (although  not

absolutely necessary) to disable Windows control of the serial ports and the

IRQs that will be used by PCLVBW applications.

To find which ports Windows wants to control, choose the "386 Enhanced" icon

in the "Control Panel" in the "Main Group" when running Windows.

The four standard COM ports which Windows typically controls are:

Port  Address  IRQ

COM1   &H3F8    4

COM2   &H2F8    3

COM3   &H3E8    4

COM4   &H2E8    3

Since COM1 and COM3 share an IRQ, if either is to be  used  by  PCLVBW  then

both should be disabled from Windows control. Similarly for COM2 and COM4.

To begin, exit from Windows completely. To disable the  serial  ports,  edit

the  [386Enh] section of the WINDOWS.INI file in the WINDOWS directory.  For

example, to disable COM1 and COM3,  add  the  following  to  the  [386  Ehn]

section:

COM1Base=0

COM1Irq=-1

COM3Base=0

COM3Irq=-1

After modifying SYSTEM.INI, Windows must be restarted (type WIN).

The  above  procedure does not apply to multiport boards or any serial ports

that Windows does not know about.

16550 UARTS are really necessary when running under Windows, particularly at

baud  rate  above  9600.  The Microsoft Diagnostics (MSD) program can be run

which will report the type of UARTs installed on your machine.

If  a 16550 port MUST be shared with Windows, and you want the FIFO enabled,

look for statements such as

COM1FIFO=0

in the [386Ehn] section of SYSTEM.INI and delete it.  However,  the  Windows

communications  driver may have trouble with the FIFO, which is why there is

a statement to disable it. This problem does not occur with PCLVBW.

PCLVBW Users Manual                                               Page 5

1.4 Installation

(1) Before installation of PCLVBW, your Visual BASIC compiler should already

be installed on your system and tested. It is recommended that  you  disable

Windows's  control  of  the  serial  ports  that you will be using. Refer to

section 1.3 "Disabling Windows Ports".

(2) Make a backup  copy  of  your  distribution  disk.   Put  your  original

distribution disk in a safe place.

(3) Create a work directory on your work disk (normally your harddisk).  For

example, to create a work directory named PCLVBW, we first log onto the work

disk and then type:

MKDIR PCLVBW

(4)  Copy  all  the  files from your backup copy of the distribution disk to

your work directory.  For example, to copy from the A: drive  to  your  work

directory, we type:

CD PCLVBW

COPY A:*.*

(5)  Copy  the  DLL  (dynamic link libraries) PCLVBW.DLL and MIO.DLL and the

icon file MSC.ICO, to the directory from which Visual Basic is running. This

is the directory reported by selecting FILES, and  then  PROPERTIES  on  the

Windows menu bar after clicking (once) on the Visual Basic icon.

(6)  Start  Visual  Basic,  and  load  the SIMPLE example program by opening

project SIMPLE.MAK. It can then be run by choosing RUN on the  Visual  Basic

menu bar.  If Visual Basic cannot load the DLLs, it means that they have not

been copied into the Visual Basic directory as outlined in (5) above.

(7) Also load and compile LOGIN.

(8)  The  easiest way to test SIMPLE is to run it on two computers connected

by a null modem  cable.   Whatever  is  typed  on  one  computer  should  be

displayed on the other. SIMPLE can also be tested by connecting your port to

a modem.

(9)    LOGIN   can   be   tested   by   connecting   to   a   modem.   Select

MODEM/START/HANDSHAKE on the dial menu bar to send an AT  to  the  connected

modem.  An  "OK" is expected in return. Selecting MODEM/START/DIAL will dial

the MarshallSoft Computing BBS.

PCLVBW Users Manual                                               Page 6

2.0 Library Organization

The PCLVBW library is organized into six categories of functions.  Refer  to

the PCL Reference Manual (PCLVBW.REF) for details on individual functions.

2.1 Configuration

There are three functions in the configuration category.  SioPorts sets the

number  of PC and DigiBoard (or BOCA board) ports. SioUART is used to change

the UART base address for a communications port to a  non-standard  address,

while  SioIRQ  is  used  to  assign  a nonstandard IRQ line to a port.  (See

chapter 6 Serial Communications for more details on standard UART  addresses

and IRQ lines).

The  configuration  functions  SioPorts,  SioUART  and SioIRQ must be called

before calling any other library functions.  Be very careful in using  these

functions.  Remember that your serial hardware must support the UART and IRQ

that you specify.  Always test any new configuration immediately.

SioPorts -  Sets number of PC and DigiBoard (or BOCA board) ports.

SioUART  |  Sets the UART base address.

SioIRQ   -  Assigns an IRQ line to a port.

THE  IRQ  GOLDEN  RULE:  You  may  open (via SioReset) only one port per IRQ

(except for the DigiBoard and BOCA board).

Be  sure  to read section 1.3 "Disabling Windows Ports" since it is best not

to share ports with Windows.

2.2 Initialization & Termination

There are eight functions in the initialization  and  termination  category.

Together,  SioParms,  SioFIFO,  SioRxBuf,  SioTxBuf, and SioReset initialize

your serial communications system.  Your application must call SioParms  and

SioRxBuf  before  calling  SioReset,  and SioReset must be called before any

serial I/O processing can be done.

After initialization, SioParms and SioBaud can be called again to change the

communications parameters without resetting the serial port. SioFlow can  be

called to enable hardware flow control.

Before  exiting  from  your  application, SioDone must be called. Failure to

call SioDone can crash your system later.

SioRxBuf   - Sets up receive buffer.

SioTxBuf   | Sets up transmitter buffer.

SioFIFO    | Sets the interrupt level for the 16550.

SioParms   | Sets parity, stop bits, and word length.

SioReset   | Initialize a serial port for processing.

SioDone    | Terminates further serial processing.

SioBaud    | Sets the baud rate of the selected port.

SioFlow    - Enables / disables flow control.

PCLVBW Users Manual                                               Page 7

2.3 Modem Control & Status

There are nine functions in the modem  control  and  status  category  which

provide  your  application with complete control over the status and control

bits of your modem.

There are two modem control bits, "Data Terminal Ready" (DTR)  and  "Request

To  Send"  (RTS).   These  bits  can  be read, set, or cleared by SioDTR and

SioRTS.

There are four modem status bits, "Data Set Ready" (DSR),  "Clear  To  Send"

(CTS),  "Ring Indicator" (RI), and "Data Carrier Detect" (DCD). SioModem can

read any of the modem status bits.  SioDSR, SioCTS, SioRI,  and  SioDCD  can

only  read  their respective modem status bit. SioGetDiv reads the baud rate

divisor register so the baud rate can be determined.

Refer to the chapter entitled "RS232 Signals" for a discussion  of  each  of

the control and status bits.

SioDTR    - Set, clear, or read the Data Terminal Ready (DTR) bit.

SioRTS    | Sets, clears, or reads the Request to Send (RTS) line.

SioModem  | Reads the modem status register.

SioDSR    | Reads the Data Set Ready (DSR) modem status bit.

SioCTS    | Reads the Clear to Send (CTS) modem status bit

SioDCD    | Reads the Data Carrier Detect (DCD) modem status bit.

SioRI     | Reads the Ring Indicator (RI) modem status bit.

SioRead   | Reads the contents of the 7 UART registers.

SioGetDiv - Reads the baud rate divisor registers.

2.4 Serial I/O

There are eleven library functions in the serial  I/O  category.   Together,

these functions give the programmer complete control over serial I/O. Higher

level  functions  such  as  protocols  and smart modem communications can be

completely implemented in terms of these functions.  Refer  to  the  example

code.

SioGetc  and  SioPutc  read  and  write  single  bytes from the serial line.

SioPuts and SioGets read and write blocks of characters. SioUnGetc  "ungets"

the  last  serial  byte  read.   SioRxClear  clears  the receive queue while

SioTxClear clears the transmit queue. SioLine can be used to test  for  UART

errors.   SioTxFlush  completes  transmission  of  all bytes in the transmit

buffer before returning.  SioRxQue  returns  the  number  of  bytes  in  the

receive  queue  while  SioTxQue  returns the number of bytes in the transmit

queue.

SioGetc    - Reads the next character from the serial line.

SioGets    | Reads a block of characters from the serial line.

SioPutc    | Transmit a character over a serial line.

SioPuts    | Transmit a block of characters over a serial line.

SioUnGetc  | "Un-gets" (puts back) a specified character.

SioRxClear | Clears the receive  buffer.

SioRxQue   | Returns the number of characters in the RX queue.

SioTxClear | Clears the transmit buffer.

SioTxFlush | Flushes the transmit buffer.

SioTxQue   | Returns the number of characters in the TX queue.

SioLine    - Reads the line status register.

PCLVBW Users Manual                                               Page 8

2.5  Error Detection

There  are two functions in the error detection category. They are concerned

with detecting or reporting communications errors.  Use of  these  functions

can make your application significantly more robust.

SioBrkSig can read or modify  the  UART  break  bit.   This  is  useful  for

signalling   the   remote  system  that  a  fatal  condition  has  occurred.

SioLoopBack can be used to  test  the  integrity  of  your  UART.

SioBrkSig   - Asserts, cancels, or detects the RS232 BREAK  signal.

SioLoopBack - Performs a UART loopback test.

2.6 General Support

There  is  just  one  functions  in  the  general  support category. SioInfo

returns a host of information, such  as  the  PCLVBW  library  version,  the

number of transmitter and receiver interrupts, etc.

SioInfo     - Returns the library version, and various other info.

PCLVBW Users Manual                                               Page 9

3.0 Library Organization

3.1 Using the Library

Please  examine the PCLVBW.BAS file. Note that COM1 is defined as port zero,

not port one.  The user  must  assume  the  responsibilty  for  passing  the

correct information when calling PCLVBW functions.

If there are any conflicts between PCLVBW definitions  and  those  in  other

libraries,  the  PCLVBW definitions can be changed in the PCLVBW.BI file and

any file that uses the definition. There is  no  change  necessary  for  the

library code itself.

The  PCLVBW  libraries contain no references to any runtime libraries.  Only

Windows API functions are called.

3.2 Example Programs

Two  communications  programs are provided as a demonstration of the PCLVBW

library - SIMPLE and LOGIN.

SIMPLE is a very simple communications programming using PCLVBW.  Everything

that  is  typed  on  the  keyboard is sent to the serial port, and everthing

incoming from the serial port is displayed on the screen.

Load  SIMPLE.MAK  from  Visual  Basic.  Select "Change" from the menu bar to

change the port, baud  rate,  parity,  data  bits,  or  stop  bits.   Select

"OnLine" from the "Line" menu item to go online.

Test  SIMPLE  by  connecting  the port it controls to a modem and issuing AT

commands, or connect to another computer using a null modem cable.

LOGIN is the same program as SIMPLE but with the addition of "Modem" on  the

menu  bar.  Under "Modem", select "Start", then "Handshake" in order to send

an "AT" to the connected modem, or "Dial" to  send  a  dial  string  to  the

modem  (which  dials  our user support BBS). Once the dial sequence is sent,

the program will wait for up to 60 seconds for the "CONNECT" string from the

modem. This wait can be terminated at any time  by  choosing  "BREAK"  under

"Modem".

LOGIN serves as an example of the use of the MIO.BAS (modem I/O)  functions.

Refer to section 4.3 "Modem I/O" for more information on MIO functions.

To  test LOGIN, you need a AT command set compatible modem and a BBS to call

up. LOGIN will dial our BBS at 205-880-9748, or  edit  the  dial  string  in

LOGLINE.BAS to call up a local BBS.

PCLVBW Users Manual                                               Page 10

3.3 Door Programs

In  order  to  write a door program which "takes over" a serial port without

resetting the port or changing the baud rate, call SioReset()  with  NORESET

as  the  second  argument rather than the baud rate. Call SioGetDiv() to get

the baud rate divisor if the baud rate must be determined. Be sure  to  call

SioDone() before returning to the invoking program.

3.4 BBS Programs

If you are designing a BBS program (also known as HOST  programs),  consider

using  16550  UARTS.   You  should  also choose a multiport card such as the

DigiBoard or BOCA board if you wish to run more than 4 ports simultaneously.

If  you  are using an error correcting modem, then you should be sure to set

flow control and fix your baud rate at the highest possible transfer  rates.

For  14,400  modems, this means 19200 or 38400. You may need a 16550 UART in

order to run at the higher speed.

If you are using an older multi-speed modem (say  1200,  2400,  4800,  9600)

that doesn't use flow control, you should change your baud rate to match the

baud rate as reported by the modem with the CONNECT message if necessary.

3.5 Compiling and Linking

Registered users may wish to recreate PCLVBW.DLL.  The Microsoft  assembler

(MASM) and the Microsoft C compiler are required. Type

NMAKE PCL4W.MAK

RENAME PCL4W.DLL PCLVBW.DLL

Similiarly, MIO.DLL can be recreated.

PCLVBW Users Manual                                               Page 11

4.0 Talking to Your Modem

A modem is used to extend the  distance  over  which  you  may  communicate.

Without  a  modem, your RS232 cable is limited to a maximum of approximately

50 feet.  But with a modem, you can communicate literally around the world.

Also  refer  to  Section  4.3  "MODEM I/O  functions" for details on MODEM I/O

functions. These functions faciliate communications with modems.

4.1 Modem Standards

Two modems can communicate over a telephone line only if they are both using

the  same  signaling frequencies and modulation, which are determined by the

the modem standards used.  Modem standards can be divided into  three  sets:

(1) speed, (2) data compression used, and (3) error control.

The  Bell  standards  (103  &  212A)  are  those  of  AT&T.   The CCITT (The

International Consultative Committee for Telephone and Telegraph)  standards

are designated as "V. ".

Speed

Bell 103  -   300 baud

Bell 212A |  1200 baud

V.21      |   300 baud

V.22bis   |  1200 & 2400 baud

V.32      |  4800 & 9600 baud

V.32bis   |  4800, 7200, 9600, 12000, and 14400 baud

V.34      -  28800 baud.

Data Compression

MNP 5     -  Microcom Networking Protocol (proprietary).

V.42bis   -  International data compression standard.

Error Control

MNP 2,3,4 -  Three level error correction (public domain).

V.42      -  International error correction standard.

Most of the newer high speed modems use  several  of  the  above  standards.

However,  not  all  combinations of modem makes communicate easily with each

other, especially at high speeds (9600 and up).

PCLVBW Users Manual                                               Page 12

4.2 Flow Control

With modems using data compression, the modem to modem connection  will  run

at  various  speeds  depending  on the quality of the line.  The computer to

modem connection will be at a fixed baud rate. Therefore, a  protocol  (flow

control)  is  necessary to synchronize the data flow between a modem and the

computer to  which  it  is  connected.   Refer  to  your  modem  manual  for

information on flow control protocols supported.

Two flow control protocols are used by most all modems  which  require  flow

control.   Software  flow  control is called "XON/XOFF" (other software flow

control character pairs are defined but operate the same  as  XON/XOFF)  and

hardware  flow  control is called "RTS/CTS".  Most modems which require flow

control enable hardware flow control by default.

In XON/XOFF (software) flow control, the computer suspends transmitting data

if it receives a XOFF character (13  hex)  from  the  modem,  and  continues

transmitting  when  it  receives  a XON character (11 hex).  Similiarly, the

computer can signal the modem not to send any more data  by  transmitting  a

XOFF  to  it,  and  can tell the modem to continue transmission be sending a

XON.

In RTS/CTS (hardware) flow control, the RTS line is used by the computer  to

signal  the  modem  ,  while the CTS line is used by the modem to signal the

computer.  The RTS line is set OFF by the computer  to  tell  the  modem  to

suspend  transmission,  and  set  to  ON  to  tell  the  modem  to  continue

transmission.  The CTS line is set to OFF by the modem to tell the  computer

to  stop  transmitting,  and  set  to  ON  to  tell the computer to continue

transmitting.

Given the choice, always choose hardware flow  control  over  software  flow

control  so  that  all  data  transmission is transparent.  If hardware flow

control is not the default (which it almost always is),  you  should  modify

your modem initialization string to turn hardware flow control on.

You will not need to use flow control when exchanging data between computers

connected  by  a  null  modem  cable  (no  modems) as long as each packet is

acknowledged (as in XMODEM and YMODEM) and the buffer size is  at  least  as

large as the packet size.

PCLVBW Users Manual                                               Page 13

4.3 MODEM I/O (MIO)

4.3.1 MIO Introduction

The file MIO.BAS contains declarations for using the Modem I/O  DLL.   These

functions  ease  communicating  with  modems  using  AT  commands. The LOGIN

example program code has examples of using the MIO functions.

The   Windows   3.X  "operating  system"  is  what  is  called  "cooperative

multitasking". This means that the executing Windows task  must  voluntarily

"give  up"  control  before  another  Windows  task  (or Windows itself) can

execute.

The  difficulty  is  how  to  wait  for  a  number  of  seconds  while still

relinquishing control periodically before the wait is up. One solution is by

making functions that need to wait (such as the MIO functions) into  "state"

machines. A function is broken down into parts called states, and control is

returned to Windows between executing each state.

For  example,  to  send  the string "ATDT1,205,880,9748" to the  modem  with

one  tenth  second  delay  between  transmitted  characters,  the  following

sequence is used:

(1) Send the string to the MIO driver by executing:

Code = mioSendTo(Port,100,"!ATDT1,205,880,9748")

The '!' characters are converted to carriage returns.  The  text  string  is

copied into the driver's data area.

(2)  Call mioDriver (typically based on a Visual BASIC timer) until MIO_IDLE

is returned.  Each time mioDriver is called, it will send another  character

to  the  modem provided the required delay (since the previous character was

sent) has passed.  If the delay has not passed, the  driver  simply  returns

MIO_RUNNING,  but  without  actually sending a character to the modem.  Once

all characters have been sent, mioDriver will return MIO_IDLE, indicating it

is done and is ready to accept another function.

mioDriver will return MIO_RUNNING if it is still processing.  Anything  else

received  indicates  that it is still processing and the returned value is a

character from the modem that can be displayed if wanted.

(3) Once mioDriver returns MIO_IDLE, call mioResult to get the result of the

mioSendTo call.

The above is rather straight forward in practice.  See the LOGIN application

for an example of its use.

PCLVBW Users Manual                                               Page 14

4.3.2 MIO Functions

4.3.2.1 mioDriver

Syntax: mioDriver(ByVal Port As Integer) As Integer

mioDriver  executes  the  next  state of any previously started MIO function

such as mioSendTo, mioWaitFor, and mioQuiet. It returns:

MIO_IDLE    : if the driver is ready for the next mioSendTo, mioWaitFor, or

mioQuiet.

MIO_RUNNING : if the driver is not idle.

<otherwise> : if the driver is not idle,  and  the  returned  character  was

received from the modem.

4.3.2.2 mioBreak

Forces the MIO driver to the IDLE state, abandoning any work in progress.

4.3.2.3 mioSendTo

Syntax:  FUNCTION mioSendTo(ByVal Port As Integer,

ByVal Pace As Long,

ByVal Text As STRING) As Integer

The mioSendTo function sends the characters in the string 'Text'  to  serial

output. There is a delay of 'Pace' milliseconds between characters.

4.3.2.4 mioWaitFor

Syntax: FUNCTION mioWaitFor(ByVal Port As Integer,

ByVal Wait As Long,

ByVal Case As Integer,

ByVal Text As String) As Integer

The mioWaitFor function waits for characters from serial  input  that  match

the  string 'Text'. A total of 'Wait' milliseconds are allowed before timing

out and returning FALSE (0).  If the 'Case' flag is TRUE,  then  the  string

comparison is NOT case sensitive.

For  example,  to  wait  up  to  one  minute for "CONNECT", "NO CARRIER", or

"BUSY" from the modem after dialing a number (on COM1):

Code = mioWaitFor(COM1,60000,1,"CONNECT|NO CARRIER|BUSY")

The  function  mioDriver()  must be called until MIO_IDLE is returned.  Then

mioResult() is called to get the result of the mioWaitFor.   A  value  of  0

indicates that neither "CONNECT", "BUSY", nor "NO CARRIER" was received. A A

non-zero  value  indicates that one of the three sub-strings was received. A

ASC("0") is returned if "CONNECT" was seen,  ASC("1")  is  returned  if  "NO

CARRIER" was seen, and ASC("2") is returned if "BUSY" was seen.

PCLVBW Users Manual                                               Page 15

4.3.2.5 mioQuiet

Syntax: FUNCTION mioQuiet(Port,Wait As Long)

The mioQuiet function waits for continuous quiet [no incoming  serial  data]

of  'Wait'  milliseconds  before  returning.  Any  incoming  character while

mioQuiet us running is flushed.

4.3.3 MIO Summary

mioDriver   : Allows  the  execution  of  mioSendTo,  mioWaitFor, or

mioQuiet once they have been  started.  Returns  MIO_IDLE

(defined in MIO.BAS) if ready not running, MIO_RUNNING if

running,  and anything else is a character from the modem

that can be displayed if wanted.

mioBreak    : Forces the MIO driver to IDLE state.

mioSendTo   : Sends string (including control chars) to the modem.

mioWaitFor  : Waits for a particular string from the modem, passing

all else through.

mioQuiet    : Waits for continuous quiet of specified duration.

mioBreak    : Breaks further modem I/O activity.

PCLVBW Users Manual                                               Page 16

4.4 Modem Initialization

If  your  application uses a modem (as opposed to using a null modem cable),

then you should always send an initialization string to your modem if it  is

a  programmable  modem  such as those made by Hayes.  Communication programs

such as PROCOMM and TELIX always send such a string automatically as soon as

they start up.

The particular initialization string depends on the make of your modem.  For

Hayes and Hayes AT command  set  compatible  modems,  the  following  string

(followed by a carriage return) may work:

AT E1 S7=60 S11=60 V1 X1 Q0 S0=0

Refer to your Modem User's Guide for a full discussion of these commands.  A

brief description is as follows:

AT     Modem attention command.

E1     Modem will echo what you send to it.

S7=60  Wait 60 seconds for carrier and/or dial tone.

S11=60 Use 60 milliseconds for tone dialing duration & spacing.

V1     Display result code as words (not numbers).

X1     Use the extended result message (CONNECT XXXX) set.

Q0     Modem returns result codes.

S0=0   Do not answer RING.

If  your application will answer incoming calls, then set the S0 register to

the ring on which to automatically answer.

If you send the above codes by using SioPutc (as opposed to typing them from

the keyboard), then follow these guidelines:

(1) Send an initial carriage return before the initialization string.

(2) Pause at least a  tenth  of  a  second  (100  milliseconds)  after  each

character  sent  as  your  modem  needs the time to perform its own internal

processing. Pause a little longer  if  your  modem  is  not  accepting  your

initialization string.

(3)  Pause  one  and a half seconds after sending any initialization command

such as ATZ or AT&F since your modem must do quite a bit of processing.

If you experience any problems in initializing your Hayes modem, you  should

first reset it to factory settings by sending:

AT&F

Your modem may require additional initialization than presented above. Refer

to your modem manual for details. It you have a comm program such as ProComm

or Telix that is known to initialize your modem correctly, then you may wish

to use the same initialization string.

PCLVBW Users Manual                                               Page 17

5.0 Problems

If you cannot get your application to run properly, first  compile  and  run

the  example programs SIMPLE and LOGIN. Both SIMPLE and LOGIN can be used to

call up any BBS.

If  your  application  does not run but SIMPLE runs correctly, then you have

most likely made a programming mistake in  your  application.   MarshallSoft

Computing  cannot  debug  your  application,  especially over the telephone!

However, consider each of the following when searching for an error in  your

application.

1.   Are  you  connected  to a device that requires flow control? Be sure to

turn on flow control with SioFlow if connected to a modem  or  other  serial

device that uses flow control.

2.  Loosing  data  at  high  baud  rates? Turn on port FIFOs with SioFIFO if

16550 UARTS are being used. Also disable Windows control of the serial ports

you are using. See section 1.3 "Disabling Windows Ports".

3.  Is your receive buffer large enough? In  general,  your  receive  buffer

should be twice as large as the largest incoming block of data.

4.   If you are running two COM ports simultaneously, are you using separate

transmit and receive buffers? (you should).

5.  Did SioReset return a zero value?  If not, then you must call  SioReset

again.

6.  Did you send the proper initialization string to your modem? See section

4.4 "Modem Initialization".

7.  Did you set DTR and RTS? Most serial devices require  that  DTR  be  set

before they will respond.

8.  Do you have more than one COM1 port, etc.  For example, if  you  have  a

COM1  port  on  your  motherboard, you cannot add another COM1 port or modem

board that uses COM1 without first disabling COM1 on the motherboard.

9.  Are you passing the proper segment of the receive (or transmit) buffer?

See SIMPLE or LOGIN for an example.

Registered  users  can  call 205-881-4630 from 1:30 PM to 9:30 PM CST Monday

through Friday for help.

PCLVBW Users Manual                                               Page 18

6.0 Serial Communications

6.1 Communications Basics

The heart of serial  communications  is  the  UART  (Universal  Asynchronous

Receiver  Transmitter).   The  IBM  PC/XT/AT  and  compatibles use the 8250,

16450, or the 16550 UART.  The purpose of the UART is:

(1) To convert bytes from the CPU (Central Processing Unit), into  a  serial

format  by  adding  the  necessary start, stop, and parity bits to each byte

before transmission, and to then transmit each bit at the correct baud rate.

(2) To convert the incoming stream (at a specified baud rate) of serial bits

into bytes by removing the start, stop, and parity bits  before  being  made

available to the CPU.

The  UART  is part of the serial interface circuitry which allows the CPU to

send and receive signals over the RS232 lines. This  can  be  diagrammed  as

follows:

Serial Interface

+-------------------+

|                   |

+-----+  Data Bus  |     +------+      |    RS232 Signals

| CPU +------------+     | UART |      +----------------*

+-----+            |     +------+      |

|                   |

+-------------------+

The  8250/16450/16550  UART  is  capable  of  operating in one of two modes,

"polled" and "interrupt driven".  The serial communications functions in the

BIOS uses the polled method.  In this approach, the CPU is  typically  in  a

loop asking the UART over and over again if it has a byte ready. If it does,

the  polling  code  returns the byte.  But, if the next byte comes in before

the polling code is executing again, then that byte is lost.

In the interrupt driven approach (used by PCLVBW for incoming data), when  a

byte  is  received by the UART, an interrupt is generated and the "Interrupt

Service Routine"  (ISR)  is  executed  immediately,  suspending  temporarily

whatever  else is executing. The ISR then moves the byte to a buffer so that

your application program can later read it. Refer to  Section  6.6  and  6.7

entitled  "RS232  Signals"  and  "National  8250,  16450, and 16550

UARTs" for further information on these topics.

PCLVBW Users Manual                                               Page 19

6.2 Standard Port Addresses

There  are  a  few  things to know about how serial communications ports are

used by IBM PC/XT/AT and compatible computers.  The  standard  IBM  PC/XT/AT

configuration values are as follows:

Port    Reg.   IRQ  Vector

COM1    3F8H    4     12

COM2    2F8H    3     11

COM3    3E8H    4     12

COM4    2E8H    3     11

(Refer to your DigiBoard manual for DigiBoard addresses, or your BOCA  board

manual for BOCA port addresses).

PCLVBW assumes the above values.  If necessary, the UART base address can be

changed by SioUART, and IRQ lines can be re-assigned  by  SioIRQ.   Remember

that  each  port to be used concurrently must have a unique IRQ line.  Refer

to the PCLVBW Reference Manual for specific details.

When installing new  communications  cards,  the  following  guidelines  are

recommended:

(1)  Be  sure to read the documentation for the hardware you are installing.

Pay special attention to UART base addresses and IRQ lines, particularly  if

trying to set up a non-standard configuration.

(2)  If  you  have  a  choice in base addresses and IRQ lines, always choose

standard values as defined above.

(3) The first port should be COM1, the second COM2, etc.  Do NOT  skip  over

any port.

(4)  Use SioUART to zero all unused ports (for example, call SioUART(COM4,0)

if there is no COM4 port installed).

(5) Be carefull not to configure two ports for the  same  address.  This  is

easier to do than you may believe.

(6)  Choose  an  external  modem over an internal one.  It is much easier to

debug problems with an external modem than an internal one.

(7) Select hardware flow control (RTS/CTS) if flow control is  required  and

hardware flow control is not the default.

(8)  Always  test your port as soon as it is installed. Try several programs

that use the communications ports.

PCLVBW Users Manual                                               Page 20

6.3 Running 3 or 4 Ports Concurrently

PCLVBW supports up to 4 serial ports running concurrently (more if you  have

a  DigiBoard  or BOCA board).  One free interrupt for each port is required.

Refer to the next section if you have a DigiBoard or BOCA board.

Interrupts  IRQ4  and  IRQ3  are  dedicated to the communications ports in a

standard IBM PC/XT/AT configuration.  IRQ4 is shared between COM1  and  COM3

while IRQ3 is shared between COM2 and COM4.  This means that you can run two

ports simultaneously provided that they don't share an interrupt.

Suppose that you wish to run 3 ports simultaneously. To begin, you must have

3  serial  UARTs  installed  on your computer.  Assume, for purposes of this

discussion, that COM1 is installed on your motherboard, and  that  you  have

purchased a new 2 port serial communications board.

You  should  be able to configure the first serial board port as COM2, which

uses IRQ3.  Refer to the manual that came with your serial board.

In order to run the third serial port concurrently with the  first  two,  an

unused  interrupt  must be found.  If your serial card can use only IRQ3 and

IRQ4, then there is no way to run a third line since IRQ4 and IRQ3 are  used

for COM1 and COM2.

However,  many serial cards can use other IRQs, typically IRQ2 through IRQ5.

Since IRQ5 is normally used  for  a  second  printer  port,  it  is  a  good

candidate  for  COM3.  To use IRQ5 for the third serial port, first set your

serial card to use IRQ5 for COM3 (refer to your serial card manual) and then

add the following line to your applications code before calling SioReset:

SioIRQ(COM3,IRQ5);

Don't forget to disable any device that might use IRQ5,  such  as  a  second

printer  port  or  a  music  card.   Unfortunately,  there is no easy way to

determine that you have no conflicts until you actually attempt to  use  the

IRQ.  If  there  are  conflicts, your system will probably hang and you will

have to reboot.

To run a fourth serial port, another  free  IRQ  must  be  found.   On  some

systems, IRQ7 can be used. To use IRQ7 for the fourth serial port, first set

your serial card to use IRQ7 for COM4 and then add:

SioIRQ(COM4,IRQ7);

To summarize, your serial card must be able to  generate  the  correct  IRQ,

which  is not already being used. Refer to the entry for the SioIRQ function

in the PCLVBW Reference Manual.

PCLVBW Users Manual                                               Page 21

6.4 Using Multiport Cards

The PCLVBW library supports the dumb Digiboard (PC/4 & PC/8)  and  the  dumb

BOCA board (BB1004, BB1008, and BB2016).

6.4.1 The DigiBoard

In  order  to  use  the  DigiBoard,  you  must  configure  PCLVBW  using the

SioPorts(), SioUART(), and SioIRQ() functions.

Your PCs ports must be partitioned into "standard" PC ports  and  dumb  card

ports.  Remember that standard PC ports cannot share IRQs like the DigiBoard

(or  BOCA  board)  can. If you are using IRQ4 and IRQ3 for standard PC ports

COM1 and COM2, then you cannot use either for DigiBoard ports (try  IRQ5  or

IRQ7).

Suppose that COM1 through COM2 are standard PC ports (using IRQ4  and  IRQ3)

and  you  have  installed  a  PC/8  DigiBoard  that you wish to use for COM3

through COM10 using interrupt line IRQ5.  You choose to use the  recommended

DigiBoard UART addresses starting at 0x100:

DEFINT A-Z

RC = SioPorts(8,COM1,&H140,DIGIBOARD)  'COM1 = 1st BOCA port

Address = &H100                        '1st BOCA UART address

FOR Port=COM1 TO COM8                  'look at each port

RC = SioUART(Port,Address)           'set the UART address

Address = Address + 8                'compute next address

RC = SioIRQ(Port,IRQ5)               'set the BOCA IRQ

NEXT I

The DigiBoard uses 0x140 for the status address for odd interrupts and 0x141

for even interrupts.

Digiboard  may  be  contacted  at  6400 Flying Cloud Drive, Eden Prairie, MN

55344.  Telephone 612-943-9020 or FAX 612-943-5398.

6.4.2 The BOCA Board

As with the DigiBoard, you must  configure  PCLVBW  before  using  the  BOCA

board.

For  example,  to  configure the BOCA BB2016 to use COM1 to COM16, with base

addresses starting at 0x100 and IRQ5:

DEFINT A-Z

RC = SioPorts(16,COM1,&H107,BOCABOARD) 'COM1 = 1st BOCA port

Address = &H100                        '1st BOCA UART address

FOR Port=COM1 TO COM16                 'look at each port

RC = SioUART(Port,Address)           'set the UART address

Address = Address + 8                'compute next address

RC = SioIRQ(Port,IRQ5)               'set the BOCA IRQ

NEXT I

BOCA may be contacted at BOCA Research, Inc., 6413  Congress  Avenue,  Suite

130, Boca Raton, FL 33487.  Phone 407-241-8088, FAX 407-997-0918.

PCLVBW Users Manual                                               Page 22

6.5 Transmitter Interrupts

Transmitter interrupts are supported by the library.  Separate libraries are

provided,  one  with  transmitter  interrupts enabled and one without.  When

transmitter interrupts are NOT enabled, the following logic occurs everytime

you call SioPutc():

1. Wait for transmit buffer to become empty. The transmit

buffer may not be empty if the previous transmit is not

completed (the UART breaks down the byte & sends 1 bit

at a time).

2. When the transmit buffer is empty, the byte from the

SioPutc() call is loaded into the transmit buffer and

control is returned to the caller.

Note that you can not write to the UART any faster than the UART baud rate.

When transmitter interrupts are enabled, the byte from SioPutc() is put into

a previously prepared (by SioTxQue) transmitter queue. The interrupt service

routine fetches bytes from this queue as soon as the previous byte has  been

sent.

While  you can now call SioPutc() faster than the baud rate, bytes are still

transmitted at the given baud rate.

Transmitter  interrupts  are generally preferrable in Windows , although not

necessary. Transmitter interrupts are better not enabled if  an  application

will be doing little or no serial transmission.

An   application  can  determine  at  runtime  if  the  PCLVBW  library  has

transmitter interrupts enabled by  calling  SioInfo(ASC("I")).   A  non-zero

return value indicates that transmitter interrupts are enabled.

A transmit buffer must be set up using SioTxBuf  if  transmitter  interrupts

are  enabled,  otherwise a transmit buffer is not required. See the programs

SIMPLE and LOGIN for an example.

The  default  library  (PCLVBW.DLL)  has  transmitter  interrupts  enabled.

PCLVBW1.DLL is the library with transmitter interrupts disabled.

PCLVBW Users Manual                                               Page 23

6.6 RS-232 Signals

RS-232  is  the  name  of the serial data interface standard used to connect

computers to modems.  Most IBM compatible computers are built with at  least

one serial port and use either DB9 (9 pin) or DB25 (25 pin) connectors.

A  summary  of  these  pins  and  their function follows.  For more detailed

information, refer to one of the many books dealing with RS-232 interfacing.

Signal Ground Pin 7 (DB25), Pin 5 (DB9)

The SG line is used  as  the  common  signal  ground,  and  must  always  be

connected.

Transmit Data Pin 2 (DB25), Pin 3 (DB9)

The TX line is used to carry data from the computer to the modem.

Receive Data Pin 3 (DB25), Pin 2 (DB9)

The RX line is used to carry data from the modem to the computer.

Data Terminal Ready Pin 20 (DB25), Pin 4 (DB9)

The  DTR  line is used by the computer to signal the modem that it is ready.

DTR should be set high when talking to a modem.

Data Set Ready Pin 6 (DB25), Pin 6 (DB9)

The DSR line is used by the modem to signal the computer that it is ready.

Request to Send Pin 4 (DB25), Pin 7 (DB9)

The RTS line is used to "turn the line around" in half  duplex  modems,  and

for hardware flow control in most modems that require flow control.

Clear to Send Pin 5 (DB25), Pin 8 (DB9)

The  CTS  line  is used to "turn the line around" in half duplex modems, and

for hardware flow control in most modems that require flow control.

Data Carrier Detect Pin 8 (DB25), Pin 1 (DB9)

The DCD line is used by the modem to signal the computer that a data carrier

signal is present.

Ring Indicator Pin 22 (DB25), Pin 9 (DB9)

The RI line is asserted when a 'ring' occurs.

PCLVBW Users Manual                                               Page 24

6.7 National 8250, 16450, and 16550 UARTs

The Personal Communications  Library  is  based  on  the  standard  National

8250,  16450,  and 16550 UARTs. The 8250 was the original UART used

in the IBM PC, whereas the 16450 is a faster version found on most 286 &  up

machines. The 16550 contains a 16 byte FIFO to further reduce communications

overhead. These UARTs consists of 8 register ports as follows:

Offset    R/W   Register

0       R/W   Receiver (read) / Transmitter (write)

1       R/W   Interrupt Enable (read)

2       R     Interrupt Identification

2       W     FIFO control (16550 only)

3       R/W   Data Format (Line Control)

4       R/W   RS-232 (Modem) Control

5       R/W   Line Status

6       R/W   RS-232 (Modem) Status

7       R/W   Not used.

For  the standard PC ports (not DigiBoard or BOCA ports), the UART registers

are based at 3F8h (COM1), 2F8h (COM2), 3E8h (COM3), and 2E8h  (COM4).   COM1

and COM3 share interrupt request line IRQ4 while COM2 and COM4 share request

line  IRQ3.   This  means  that  COM1  and  COM3 can't be used concurrently.

Similarly for COM2 and COM4.

If you have a DigiBoard (or BOCA board) installed, you will have 4  or  more

additional  ports  using  16450 or 16550 UARTS.  The default DigiBoard

and BOCA board ports are located at 100h, 108h, 110h,  etc.  Refer  to  your

DigiBoard (or BOCA board) manual.

Four  sources  of  interrupts  are  possible  with  the  8250 and 16550: (1)

receiver error or BREAK, (2) receiver data ready, (3) ready to transmit, and

(4) RS232 input.   These  four  sources  of  interrupts  are  summarized  as

follows:

Source of Interrupt        Action Required to Clear

Receiver error or BREAK.   Read Line Status register.

Receiver data.             Read data from data register.

Transmitter Buffer Empty.  Write to data register or read IID reg.

RS232 input.               Read Modem Status register.

However,  PCLVBW  only enables the receiving data interrupt. This means that

interrupts can only be caused by incoming data.

If  you  are  not  familiar with the 8250, several good books are available.

Refer to Section 6.0 Serial Communications for recommendations.  Although  a

knowledge of the 8250 is not necessary to use PCLVBW, a general knowledge of

the theory of asynchronous serial communications is recommended.

PCLVBW Users Manual                                               Page 25

6.8 Register Summary

REG 0 : Data Register

Reading  from  the  data  register  fetches  the next input byte, once it is

ready.  Writing to the data register transmits the byte written to  it  over

the serial line.

REG 1 : Interrupt Enable

The  Interrupt Enable register enables each of four types of interrupts when

the appropriate bit is set to a one.

bit 3 : Enable interrupt on RS232 input.

bit 2 : Enable interrupt on receiver error or break.

bit 1 : Enable interrupt on transmitter buffer empty (TBE).

bit 0 : Enable interrupt on received data (RxRDY).

REG 2 : Interrupt Identification (IID)

Reading  the Interrupt Identification (read only) register once an interrupt

has occurred identifies the interrupt as follows:

Bit 2  Bit 1  Bit 0  Priority    Interrupt

0      0      1      none      none

1      1      0      0 (high)  Serialization or break.

1      0      0      1         Received data.

0      1      0      2         Transmitter Buffer Empty.

0      0      0      3 (low)   RS232 Input.

In the INS16650, REG 2 (write only)  is  also  the  FIFO  control  register.

Writing bits 6 & 7 will set the FIFO trigger level (number of bytes received

before an interrupt is generated).

Bit 7  Bit 6   Trigger             Bit 7  Bit 6   Trigger

0      0      1 byte               1      0      8 bytes

0      1      4 bytes              1      1      14 bytes

REG 3 : Line Control

RS232 line parameters are selected by writing to this register.

bit 7   : DLAB = 0

bit 6   : BREAK on(1), off(0).

bits 5-3: Parity None(000),ODD(001),EVEN(011),MARK(101),SPACE(111)

bit 2   : One stop bit(0), two stop bits(1).

bits 1-0: Data bits = 5 (00), 6(01), 7(10), 8(11).

When  the Divisor Latch Access Bit (DLAB) is 1, registers 0 and 1 become the

LS and MS bytes of the Baud Rate Divisor registers.

Baud   Divisor      Baud  Divisor      Baud  Divisor

300    0180        4800   0018       38400   0003

1200    0060        9600   000C       57600   0002

2400    0030       19200   0006      115200   0001

PCLVBW Users Manual                                               Page 26

REG 4 : Modem Control

RTS, DTR, loopback testing, and  General  Purpose  Outputs  #1  and  #2  are

controlled by the Modem Control register as follows:

bit 4 : Enable local loopback.

bit 3 : Enable GP02. Necessary for 8250 interrupts.

bit 2 : Enable GP01.

bit 1 : Set / clear RTS.

bit 0 : Set / clear DTR.

REG 5 : Line Status

Reading  the  Line Status register provides status information as follows (1

for TRUE, 0 for FALSE) :

bit 6 : Transmitter Empty.

bit 5 : Transmitter Buffer Empty (TBE).

bit 4 : BREAK detect.

bit 3 : Framing error.

bit 2 : Parity error.

bit 1 : Overrun error.

bit 0 : Data Ready.

REG 6 : Modem Status

Reading the Modem Status register provides the following status  information

(1 for TRUE, 0 for FALSE) :

bit 7 : DCD status.

bit 6 : RI status.

bit 5 : DSR status.

bit 4 : CTS status.

bit 3 : Delta DCD status.

bit 2 : Delta RI status.

bit 1 : Delta DSR status.

bit 0 : Delta CTS status.

The  delta  bits  (bits 0 through 3) are set whenever one of the status bits

(bits 4 through 7) changes (from 0 to 1 or from 1 to 0) since the last  time

that  the  Modem Status register was read. Reading the Modem Status register

clear the delta bits.

REG 7 : Scratch Register

There is no function associated with register 7.  It does not exist in early

versions of the 8250.

PCLVBW Users Manual                                               Page 27

8.0 Legal Issues

8.1 Registration

If you wish to register the PCLVBW library, please send $75 plus $5 S&H ($10

outside of North America) to:

MarshallSoft Computing, Inc.

Post Office Box  4543

Huntsville AL 35815

Multiple copies are available: The first copy is $75. Copies 2 through 5 are

$60 each. Copies 6 through 10 are $45 each. Copies 11 and up are $30 each. A

site license is  also  available  for  $495  (includes  5  sets  of  printed

documentation). We pay shipping on multiple copy orders.

We accept American Express, VISA, and Mastercard (account number, expiration

date,  exact  name  on  your  card,  and complete billing address required),

checks in US dollars  drawn  on  a  US  bank,  purchase  orders  (POs)  from

recognized  US  schools  and  companies  listed in Dun & Bradstreet, and COD

(street address and phone number required) within the USA (plus a $4.50  COD

charge).

You  can  also order PCLVBW from The Public Software Library (PSL) with your

MC, Visa, AmEx, or Discover card by  calling  800-242-4PSL  (from  overseas:

713-524-6394)  or  by  FAX  at 713-524-6398 or by CompuServe at [71355,470].

THESE NUMBERS ARE FOR ORDERING ONLY. The product number for PCLVBW is 11499.

(Specify VisualBASIC/Windows when  you  call).  Please  have  your  shipping

address and credit card billing address ready.

If  you wish to update from an older version of PCLVBW, send $20 plus $5 S&H

($10 outside of North  America).   Updates  must  be  ordered  directly  from

MarshallSoft Computing.

The registered package includes:

o  Assembler source code for the communications library.

o  C source code for the modem I/O library.

o  Printed Users and Reference Manuals.

o  Telephone, email, FAX, and BBS support for one year.

Print the file INVOICE.DOC if an invoice is needed. The registered user will

receive  the  latest  version  of  PCLVBW  shipped  by two day priority mail

(packet airmail overseas).  A 3.5"  diskette  is  provided  unless  a  5.25"

diskette is requested.

PCLVBW Users Manual                                               Page 28

8.2 License

MarshallSoft Computing, Inc. grants the registered user of PCLVBW the  right

to  use one copy of the PCLVBW library (in object form) on a single computer

in the development of any software product (other  than  libraries  such  as

PCLVBW).   The user may not use the library on more than one computer at the

same time.  The source code for the library (PCLVBW.ASM) is  copyrighted  by

MarshallSoft Computing and may not be released in whole or in part.

Products  developed  using PCLVBW can include the object form of the library

and may be distributed without any royalty.

8.3 Warranty

MARSHALLSOFT COMPUTING, INC.  DISCLAIMS  ALL  WARRANTIES  RELATING  TO  THIS

SOFTWARE,  WHETHER  EXPRESSED  OR  IMPLIED, INCLUDING BUT NOT LIMITED TO ANY

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR  PURPOSE,

AND  ALL  SUCH WARRANTIES ARE EXPRESSLY AND SPECIFICALLY DISCLAIMED. NEITHER

MARSHALLSOFT COMPUTING, INC. NOR ANYONE ELSE WHO HAS BEEN  INVOLVED  IN  THE

CREATION,  PRODUCTION,  OR DELIVERY OF THIS SOFTWARE SHALL BE LIABLE FOR ANY

INDIRECT, CONSEQUENTIAL, OR INCIDENTAL DAMAGES ARISING OUT  OF  THE  USE  OR

INABILITY  TO  USE  SUCH  SOFTWARE EVEN IF MARSHALLSOFT COMPUTING, INC.  HAS

BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR CLAIMS. IN NO EVENT SHALL

MARSHALLSOFT COMPUTING, INC.'S LIABILITY FOR ANY SUCH  DAMAGES  EVER  EXCEED

THE  PRICE  PAID FOR THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM

OF THE CLAIM. THE PERSON USING THE SOFTWARE BEARS ALL RISK AS TO THE QUALITY

AND PERFORMANCE OF THE SOFTWARE.

Some states do not allow  the  exclusion  of  the  limit  of  liability  for

consequential  or  incidental damages, so the above limitation may not apply

to you.

This agreement shall be governed by the laws of the  State  of  Alabama  and

shall  inure  to  the  benefit  of  Marshallsoft  Computing,  Inc.   and any

successors, administrators, heirs and  assigns.  Any  action  or  proceeding

brought  by either party against the other arising out of or related to this

agreement shall be brought only in a STATE or  FEDERAL  COURT  of  competent

jurisdiction  located in Madison County, Alabama. The parties hereby consent

to in personam jurisdiction of said courts.

PCLVBW Users Manual                                               Page 29

9.0 Summary

9.1 Revision History

Version 1.0 - 10 February 1996

This  is  the  first  release  of  the Visual BASIC / Windows version of our

library. It uses the common assembler language library used in  all  of  our

Windows and DOS communications library products.

9.2 Function Summary

Refer to the PCLVBW Reference Manual (PCLVBW.REF) for  detailed  information

on  the  communications  and  support functions.  A one line summary of each

function follows:

SioBaud     Sets the baud rate of the selected port.

SioBrkSig   Asserts, cancels, or detects BREAK signal.

SioCTS      Reads the Clear to Send (CTS) modem status bit.

SioDCD      Reads the Data Carrier Detect (DCD) modem status bit.

SioDone     Terminates further serial processing.

SioDSR      Reads the Data Set Ready (DSR) modem status bit.

SioDTR      Set, clear, or read the Data Terminal Ready (DTR) bit.

SioFIFO     Sets the interrupt level for the 16550.

SioFlow     Enables / disables hardware flow control.

SioGetc     Reads the next character from the serial line.

SioGetDiv   Reads the baud rate divisor registers.

SioKeyPress Detects if keyboard has been pressed.

SioKeyRead  Reads the keyboard.

SioInfo     Returns library version number & memory model.

SioIRQ      Assigns an IRQ line to a port.

SioLine     Reads the line status register.

SioLoopBack Performs a UART loopback test.

SioModem    Reads the modem status register.

SioParms    Sets parity, stop bits, and word length.

SioPorts    Sets # ports, 1st DigiBoard / BOCA port & status reg.

SioPutc     Transmit a character over a serial line.

SioRead     Reads any of 7 UART ports.

SioReset    Initialize a serial port for processing.

SioRI       Reads the Ring Indicator (RI) modem status bit.

SioRTS      Sets, clears, or reads the Request to Send (RTS) line.

SioRxBuf    Sets up receive buffer.

SioRxClear  Clears the receive buffer.

SioRxQue    Returns the number of characters in the receive queue.

SioTxBuf    Sets up transmit buffer.

SioTxClear  Clears the transmit buffer.

SioTxFlush  Flushes the transmit buffer.

SioTxQue    Returns the number of characters in the transmit queue.

SioUART     Sets the UART base address.

SioUnGetc   "Un-gets" (puts back) a specified character.

PCLVBW Users Manual                                               Page 30

9.3 Further Reading

The best way to learn about serial communications is to read a good book  on

the subject. Several good texts are available.  Two that I like are:

(1) C  Programmers's  Guide to Serial Communications by Joe Campbell (SAMS)

(2) Mastering Serial Communications by Peter Gofton (SYBEX).

10.0 Other MarshallSoft Computing Products

MarshallSoft Computing also makes communication libraries for Visual BASIC /

DOS  (PCL4VB),  Power  BASIC  (PCL4PB),  C/DOS  (PCL4C),  C/Windows (PCL4W),

Pascal/DOS (PCL4P), Delphi (PCL4D).

We  also  will  be releasing a communications library that uses the build-in

Windows  communications  functions.  It  will be limited to 9 ports at 19200

baud.

PCLVBW Users Manual                                               Page 31