The CTS Serial Port Utilities (TM)

User Guide

Computer Telecommunication Systems, Inc

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

(c) Copyright 1993-1994 Computer Telecommunication Systems, Inc.

All Rights Reserved.

No part of this User Guide may be reproduced, transmitted,

transcribed, stored in a retrieval system, or translated into any

language (natural or computer), in any form or by any means,

without the prior written permission of Computer

Telecommunication Systems, Inc. See the accompanying files

VENDOR.DOC, LICENSE.DOC, and SYSOP.DOC for specific information

pertaining to distribution of this shareware version of the User

Guide.

Information in this User Guide is subject to change without

notice and does not represent a commitment on the part of

Computer Telecommunication Systems, Inc.

Computer Telecommunication Systems, Inc. may make improvements

and/or changes in the User Guide and/or in Serial Port Utilities

at any time.

Trademarks and Copyrights

IBM(R)  and PS/2(R)  are registered trademarks of International

Business Machines Corporation.

Hayes(R) is a registered trademark of Hayes Microcomputer

Products, Inc.

The CTS Serial Port Utilities(TM), SwapIRQ(TM), and AT Command

Test(TM) are trademarks of Computer Telecommunication Systems,

Inc.

MSD is an application distributed by Microsoft Corporation

T/Port(TM) and Telcor(TM) are trademarks of Telcor Systems

Corporation.

Other names listed in this User Guide are the proprietary

trademarks of their respective companies.

Computer Telecommunication Systems, Inc.

3847 Foxwood Road, Suite 1000

Duluth, Georgia USA 30136

404-263-8623 (voice)

404-263-0124 (fax)

CompuServe ID:76662,2315

ii

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

TABLE OF CONTENTS

Registration Options  . . . . . . . . . . . . . . . . . . . .  vi

Personal . . . . . . . . . . . . . . . . . . . . . . . .  vi

Personal PLUS  . . . . . . . . . . . . . . . . . . . . .  vi

Professional . . . . . . . . . . . . . . . . . . . . . . vii

Professional PLUS  . . . . . . . . . . . . . . . . . . . vii

Association Of Shareware Professionals Member . . . . . . . .   x

Acknowledgments . . . . . . . . . . . . . . . . . . . . . . .  xi

1.0 Introducing The CTS Serial Port Utilities . . . . . . . .   1

PORTINFO . . . . . . . . . . . . . . . . . . . . . . . .   1

BUFFER . . . . . . . . . . . . . . . . . . . . . . . . .   2

COM_BPS  . . . . . . . . . . . . . . . . . . . . . . . .   3

COM_FMT  . . . . . . . . . . . . . . . . . . . . . . . .   3

DOS_COM  . . . . . . . . . . . . . . . . . . . . . . . .   3

DOS_SWAP . . . . . . . . . . . . . . . . . . . . . . . .   3

DTR  . . . . . . . . . . . . . . . . . . . . . . . . . .   3

IRQ  . . . . . . . . . . . . . . . . . . . . . . . . . .   3

RESETCOM . . . . . . . . . . . . . . . . . . . . . . . .   3

RTS  . . . . . . . . . . . . . . . . . . . . . . . . . .   4

2.0 Short Instructions (Quick-Start)  . . . . . . . . . . . .   5

3.0 The Command Line  . . . . . . . . . . . . . . . . . . . .   7

Serial Port Number Switches  . . . . . . . . . . . . . .   8

On / Off Switches  . . . . . . . . . . . . . . . . . . .   9

Parameter Switches . . . . . . . . . . . . . . . . . . .   9

Option Switches  . . . . . . . . . . . . . . . . . . . .   9

4.0 Defining Non-Standard Serial Ports  . . . . . . . . . . .  11

5.0 Testing Your System With PortInfo . . . . . . . . . . . .  13

Test All Serial Ports  . . . . . . . . . . . . . . . . .  14

Test One Serial Port . . . . . . . . . . . . . . . . . .  15

Quick Test . . . . . . . . . . . . . . . . . . . . . . .  15

Log File . . . . . . . . . . . . . . . . . . . . . . . .  16

Network Considerations . . . . . . . . . . . . . . . . .  17

The PortInfo Summary Screen  . . . . . . . . . . . . . .  18

Port Configuration  . . . . . . . . . . . . . . . .  18

Port Address . . . . . . . . . . . . . . . . .  18

Interrupt (IRQ)  . . . . . . . . . . . . . . .  19

IRQ (Out2) Enabled . . . . . . . . . . . . . .  19

DOS uses port as . . . . . . . . . . . . . . .  19

Type of Port . . . . . . . . . . . . . . . . .  20

Buffer - Trigger . . . . . . . . . . . . . . .  20

Port Settings . . . . . . . . . . . . . . . . . . .  21

Speed (BPS)  . . . . . . . . . . . . . . . . .  21

Bits / Character . . . . . . . . . . . . . . .  21

iii

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

Parity . . . . . . . . . . . . . . . . . . . .  22

Stop Bits  . . . . . . . . . . . . . . . . . .  22

Hardware Signals  . . . . . . . . . . . . . . . . .  22

Clear to Send  . . . . . . . . . . . . . . . .  22

Data Set Ready . . . . . . . . . . . . . . . .  23

Ring Indicator . . . . . . . . . . . . . . . .  23

Data Carrier Detect  . . . . . . . . . . . . .  24

Data Terminal Ready  . . . . . . . . . . . . .  24

Request to Send  . . . . . . . . . . . . . . .  25

6.0 The Other Utilities . . . . . . . . . . . . . . . . . . .  27

BUFFER . . . . . . . . . . . . . . . . . . . . . . . . .  27

COM_FMT  . . . . . . . . . . . . . . . . . . . . . . . .  28

COM_BPS  . . . . . . . . . . . . . . . . . . . . . . . .  28

DOS_COM  . . . . . . . . . . . . . . . . . . . . . . . .  29

DOS_SWAP . . . . . . . . . . . . . . . . . . . . . . . .  29

DTR  . . . . . . . . . . . . . . . . . . . . . . . . . .  29

IRQ  . . . . . . . . . . . . . . . . . . . . . . . . . .  29

RESETCOM . . . . . . . . . . . . . . . . . . . . . . . .  30

RTS  . . . . . . . . . . . . . . . . . . . . . . . . . .  30

7.0 PortInfo FAILURE and WARNING Messages . . . . . . . . . .  31

Interrupt Conflicts  . . . . . . . . . . . . . . . . . .  31

Message PI-2010 . . . . . . . . . . . . . . . . . .  31

Message PI-2020 . . . . . . . . . . . . . . . . . .  32

Message PI-2100 . . . . . . . . . . . . . . . . . .  32

Message PI-2110 . . . . . . . . . . . . . . . . . .  32

You have multiple ports at the same address  .  33

You have another device using the same

interrupt . . . . . . . . . . . . . . . .  34

Your system will not let you share this

interrupt . . . . . . . . . . . . . . . .  34

Message PI-2200 . . . . . . . . . . . . . . . . . .  35

Message PI-2300 . . . . . . . . . . . . . . . . . .  35

Port Conflicts . . . . . . . . . . . . . . . . . . . . .  35

Message PI-3000 . . . . . . . . . . . . . . . . . .  35

Message PI-3100 . . . . . . . . . . . . . . . . . .  36

Mouse Conflicts  . . . . . . . . . . . . . . . . . . . .  36

Message PI-4000 . . . . . . . . . . . . . . . . . .  36

DOS Port Assignments . . . . . . . . . . . . . . . . . .  36

Message PI-5000 . . . . . . . . . . . . . . . . . .  37

Message PI-5010 . . . . . . . . . . . . . . . . . .  37

Message PI-5020 . . . . . . . . . . . . . . . . . .  38

Message PI-5030 . . . . . . . . . . . . . . . . . .  38

Message PI-5040 . . . . . . . . . . . . . . . . . .  38

Other CTS Software Detected  . . . . . . . . . . . . . .  38

Message PI-6000 . . . . . . . . . . . . . . . . . .  39

8.0 Customer Support Policy . . . . . . . . . . . . . . . . .  41

9.0 About Computer Telecommunication Systems  . . . . . . . .  43

AT Command Test  . . . . . . . . . . . . . . . . . . . .  43

iv

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

SwapIRQ  . . . . . . . . . . . . . . . . . . . . . . . .  43

MouseOFF and MouseON . . . . . . . . . . . . . . . . . .  43

Serial Cards (Comm Ports)  . . . . . . . . . . . . . . .  43

Cables / Adapters  . . . . . . . . . . . . . . . . . . .  44

Custom Programming Services  . . . . . . . . . . . . . .  44

New Products . . . . . . . . . . . . . . . . . . . . . .  44

v

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

Registration Options

====================

Four different registration options are available:

** Personal **

--------------

This is the most economical registration for a single user. If you need

to use (or used) the SPU to diagnose a problem with your personal

computer, then this is the license for you. By registering, you obtain

personalized support to assist you with trouble-shooting your computer.

Details are provided in the User Guide section titled "Customer Support

Policy". As part of this license, you also receive:

*  30 minutes of personal assistance with your serial port problems.

*  The most current version of the CTS Serial Port Utilities.

*  A FREE CompuServe IntroPak which gets you online with CompuServe

without the regular signup fee plus a $15 usage credit.

*  FREE minor version updates.

*  Announcements about major updates. You will receive a FREE upgrade

to any major update within 6 months of your registration.

The license provided with this option allows the registered user to use

the SPU on any computer personally owned by the registered user.

** Personal PLUS **

-------------------

This registration option provides all of the benefits above plus:

*  A printed copy of the attractive 85 page, 8.5 x 11" User Guide,

including 30 pages of introductory and reference information not

included in the electronic manual. This additional information

includes serial port basics, types of serial ports, when and why

you need a 16550A UART, how and when you can share interrupts

between serial ports, details of the RS-232 interface, cable

wiring, and how IRQ 2 and IRQ 9 are used.

*  A FREE upgrade to any major update within *12* months of your

registration.

vi

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

*  up to *60* minutes of personal assistance with your serial port

problems.

*  The MouseOn and MouseOff utilities. The "mouse utilities" allow a

mouse and a modem to share the same interrupt when running under

DOS.

The license provided with this option allows the registered user to use

the SPU on any computer personally owned by the registered user.

** Professional **

------------------

This license is for consultants and corporate support specialists who

routinely deal with a multiple computers. Treat the SPU like your

screwdriver - that is, as long as it is in your possession and not

"left" (installed) at any other machine, you are in compliance with the

license. Only one individual is authorized to call for support.

This license option provides all of the benefits described above, plus:

*  A *new* bonus utility "FindCOM" which locates any serial port

installed in the computer system. This utility does a passive

search of the I/O address area to find any 8250 compatible serial

port at *any* address. A very handy utility for those "unknown"

machines.

*  Up to *90* minutes of technical support (one authorized caller per

license)

*  Allows use of the SPU on multiple computers AS LONG AS the SPU is

ONLY used on ONE COMPUTER AT A TIME. Site licenses are available

for situations where multiple installed copies are required.

** Professional PLUS **

-----------------------

Includes everything in the Professional License, PLUS:

*  A "toolkit copy" of the CTS SwapIRQ(tm) utility. SwapIRQ is our

$29.95 (retail) DOS utility that lets DOS applications use any IRQ

supported by a serial port or modem. A very handy utility for those

situations where an IRQ conflict could be resolved *if* a software

application could just use a different IRQ. Now it can!

SwapIRQ is a TSR which uses 784 bytes of memory and redirects

hardware-generated IRQs to the IRQ the software uses. SwapIRQ runs in

DOS, or in a DOS box under Windows and OS/2.

Note: each installed copy of SwapIRQ requires an additional license.

Multicopy discounts and site licenses are available if required. Contact

CTS for details.

vii

SPU Version 1.3 LICENSE REGISTRATION FORM - 114

Name: _________________________________________________________

Daytime telephone: __________________  Evening: _______________

Fax Number: _________________    CompuServe ID: _______________

Company Name: _________________________________________________

Address: ____________________________________________________

____________________________________________________

ALL ORDERS - Choose format: 5.25" ____ 3.5" ____

QTY   Description                            Unit Price   Subtotal

===   ===========                            ==========   ========

___   Personal                                  $20.00   $________

___   Personal PLUS                             $35.00   $________

___   Professional                              $59.00   $________

___   Professional PLUS                         $79.00   $________

Sales Tax:  Georgia residents add 5%   $________

Shipping:   $    4.00

TOTAL (US dollars, please)   $________

PAYMENT METHOD:      Cash __         Check __     Money Order __

Visa __    MasterCard __        Discover __

=================================================================

CREDIT CARD INFORMATION

=================================================================

Name as it appears on the card: _________________________________

Card No. ________  ________  ________  ________  Expires  ___/___

Cardholder signature: ___________________________________________

=================================================================

SEND TO

=================================================================

Computer Telecommunication Systems, Inc.

3847 Foxwood Road, Suite 1000

Duluth, Georgia  30136-6100 USA

404-263-8623 (Voice)

404-263-0124 (FAX)

CompuServe: 76662,2315

Internet:76662.2315@compuserve.com

NOTE: Prices are subject to change without notice

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

DISCLAIMER OF WARRANTY

USERS OF THIS SOFTWARE MUST ACCEPT THIS DISCLAIMER OF WARRANTY:

THIS SOFTWARE IS SUPPLIED AS IS. COMPUTER TELECOMMUNICATION

SYSTEMS DISCLAIMS ALL WARRANTIES, EITHER EXPRESS OR IMPLIED,

INCLUDING BUT NOT LIMITED TO IMPLIED WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE, WITH

RESPECT TO THE SOFTWARE AND THE ACCOMPANYING WRITTEN MATERIALS.

NO LIABILITY FOR CONSEQUENTIAL DAMAGES. IN NO EVENT SHALL

COMPUTER TELECOMMUNICATION SYSTEMS OR ITS SUPPLIERS BE LIABLE FOR

ANY DAMAGES WHATSOEVER (INCLUDING, WITHOUT LIMITATION, DAMAGES

FOR LOSS OF BUSINESS PROFITS, BUSINESS INTERRUPTION, LOSS OF

BUSINESS INFORMATION, OR OTHER PECUNIARY LOSS) ARISING OUT OF THE

USE OF OR INABILITY TO USE THIS COMPUTER TELECOMMUNICATION

SYSTEMS PRODUCT, EVEN IF COMPUTER TELECOMMUNICATION SYSTEMS HAS

BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES. BECAUSE SOME

STATES DO NOT ALLOW THE EXCLUSION OR LIMITATION OF LIABILITY FOR

CONSEQUENTIAL OR INCIDENTAL DAMAGES, THE ABOVE LIMITATION MAY NOT

APPLY TO YOU.

ix

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

Association Of Shareware Professionals Member

=============================================

_______

____|__     |                (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

This program is produced by a member of the Association of

Shareware Professionals (ASP). ASP wants to make sure that the

shareware principle works for you. If you are unable to resolve a

shareware-related problem with an ASP member by contacting the

member directly, ASP may be able to help. The ASP Ombudsman can

help you resolve a dispute or problem with an ASP member, but

does not provide technical support for members' products.  Please

write to the ASP Ombudsman at 545 Grover Road, Muskegon, MI

49442-9427 USA, FAX 616-788-2765 or send a CompuServe message via

CompuServe Mail to ASP Ombudsman 70007,3536.

x

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

Acknowledgments

I wish to express my very special thanks to the following

individuals who made this application possible. The CTS Serial

Port Utilities would not exist today without their suggestions,

assistance, and, most importantly, the time they invested in

testing the thirty or so iterations of the SPU during the six

month BETA test program.

David Bittner, who received, tested, and reported back on

more iterations of the SPU than I had a right to expect of

anyone!

Dan Landiss, who identified an internal modem which the SPU

didn't like...and hung in there running tests until I got it

right!

Earle Robinson, who really has the knack for product

testing. He broke some of the early versions of the SPU more

often and faster than anyone!

And, finally, Tom Wason, who wrote a six page letter filled

with suggestions and comments. I had to agree with

everything he said! Tom...if you mentioned it, it's here!

In addition, there were individuals too numerous to list who

tested at least one version of the SPU and reported back

suggestions or verified that known problems were fixed. To each

of you, my thanks!

xi

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

1.0 Introducing The CTS Serial Port Utilities

Serial ports (also known as comm ports) are painless when they

work, and a pain in the neck when they don't. They have plenty of

options and limitations. They need a unique address, an inactive

IRQ, and software which is properly configured to use it. No

wonder nearly everyone gets confused by serial ports.

The CTS Serial Port Utilities (SPU) is a collection of DOS

software utilities that provide you with control over the serial

ports in your computer. Most serial port problems can be resolved

by using these utilities. Each of the utilities support all

standard and any user-defined serial ports. User-defined ports

are defined in the command line or in your system environment

variables.

Why "Oh, NO! NOT ANOTHER?" serial port utility? Well, several

good reasons come to mind. The first is that there were a lot of

things still not done by other utilities. Can you name one which

detects multiple serial ports installed at the same address

(other than the SPU?) Or one that tells you when your monitor

will cause problems if you use COM4? Want to know what the

current receiver trigger level is in your 16550A serial port?

Better use the SPU program PortInfo to find out! Nothing else out

there today will tell you! The SPU even detects the T/Port

serial card, and identifies some internal modems that have

"emulated UARTs".

More reasons? We spent a lot of time reviewing the utilities that

were out before SPU, and didn't find one that was both good and

full-featured. There were a few that were very good for limited

applications, but didn't properly test IRQ assignments, or were

limited in the serial ports they supported, or....well, you get

the idea. It just seemed like it was time for a common set of

utilities that tied everything together.

So the CTS Serial Port Utilities came into being. The utilities

all share a common command line interface, and they all use a

common set of environment variables you can set to define non-

standard serial ports. The SPU contains the following programs:

1.1 PORTINFO

PortInfo is a comprehensive utility which tests your system

to determine everything possible about your serial ports,

including interrupt and address conflicts. PortInfo provides

you with FAILURE and WARNING messages about actual and

possible conflicts.

1

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

PortInfo provides details on the following:

*   Multiple serial ports at the same address (eg, 2 Com1

ports)

*   Interrupt conflicts between a bus mouse and a serial

port

*   Shared interrupts (2 serial ports, or a serial port and

a serial mouse sharing an IRQ)

*   Conflicts COM4 and 8514/A compatible monitors

*   Unusual default serial port setups (BIOS and DOS port

assignments)

*   Serial ports that cannot generate interrupts

*   Helps to determine when interrupts can be shared

Other information provided:

*   Actual IRQ used by each port and the current IRQ status

*   DOS port assignments (com1-com4, mouse)

*   Port (UART) Identification (8250, 8250A/16450, 16550,

16550A, the T/Port, and some "emulated UARTs"

*   FIFO status: If it exists, on / off, else 'None'

*   If the FIFO is ON, the current FIFO receiver trigger

level is displayed.

*   All programmable port parameters: speed, format,

parity, current RTS and DTR settings

*   Current status of modem lines: Ring Indicator, Carrier

Detect, Clear to Send, Data Set Ready, with indication

of when the lines are tied together (looped back).

Other features:

*   Ports are COMPLETELY restored to their original

settings

*   Supports standard serial ports 1-4 on PC/AT, 1-8 on

PS/2

*   Supports up to 8 user definable (port address and IRQ)

serial ports

*   Displays detailed messages about conflicts with

reference numbers to assist looking up the detailed

assistance in the section titled "PortInfo FAILURE and

WARNING Messages".

1.2 BUFFER

Buffer allows you to control the FIFO buffers in a 16550A or

compatible UART. You can turn the buffers on and off, and

set the receiver trigger level. See the section titled

"Testing Your Software with PortInfo...Buffer-Trigger" for

more information on UART FIFO receiver trigger levels.

2

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

1.3 COM_BPS

COM_BPS allows you to set a port data rate (in bits per

second, or BPS) to any standard value supported by the

serial port. This utility is normally used as a replacement

for the DOS set mode command.

1.4 COM_FMT

COM_FMT allows you to set a port character format to any

standard value supported by the serial port. This utility is

normally used with COM_BPS as a replacement for the DOS mode

command.

1.5 DOS_COM

You can use DOS_COM to insert a serial port in the list of

ports used by DOS.

1.6 DOS_SWAP

You can use DOS_SWAP to exchange two serial ports in the

list of ports used by DOS.

1.7 DTR

DTR gives you command line control of the Data Terminal

Ready (DTR) line of any serial port.

1.8 IRQ

IRQ allows you to enable and disable the interrupts from any

serial port.

1.9 RESETCOM

ResetCom resets a port to a "power-up" configuration: Port

interrupts are disabled, data rate is set to 2400 bps,

character format to eight data bits, no parity, one stop

bit. If the port is a 16550A family device, then the buffers

(FIFOs) are disabled. This utility will often restore a port

that doesn't work after running an ill-behaved application.

3

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

1.10 RTS

RTS gives you command line control of the Request to Send

(RTS) line of any serial port.

4

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

2.0 Short Instructions (Quick-Start)

Each of the utilities provides a short help screen when you enter

the program name followed immediately by the return key. Users

with a network card installed should refer to the section titled

"Network Considerations" under "Testing Your System With

PortInfo". This help screen includes a brief description of what

each utility does and the valid command line switches used to

control it. For example, if you wanted to see the help screen for

PortInfo, you would enter:

PortInfo <Enter>

After reading the help screen, you might decide to do the

basic system test with the command:

PortInfo /0 <Enter>

NOTE: This only works for standard serial port addresses.

Refer to the following documentation for information on how

to test serial ports located at any address.

We've occasionally observed situations with internal modems that

require the computer to have a hard reset before running PortInfo

to obtain correct results. If you observe a problem where

PortInfo does not detect an internal modem or the IRQ used by the

modem, then either turn your computer off and back on to perform

a hard reset, or, if your computer has a RESET button, press it.

Then try running PortInfo again.

NOTE: a soft reset (control-alt-delete) is not sufficient to

reset these modems to a state where PortInfo can recognize them.

You must do a "hard reset" as described above.

5

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

6

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

3.0 The Command Line

The Serial Port Utilities are controlled by "command line

switches" that you enter on the command line. Command line

switches are program options that you place on the command line.

Each option is preceded by a slash character "/". These

"switches" control the SPU software the way a hardware switch

controls the options on a board.

The standard command line format is:

Program_Name /switch1 /switch2 ... <Enter>

For each of the utilities, you can enter either the program name

by itself or with a "/? command line switch" to display a brief

description of the utility and the switches used to control it.

For example, both of the following commands will display the help

screen for PortInfo:

PortInfo /? <Enter>

or

PortInfo <Enter>

where <Enter> means you should press the Enter or Return key.

There are some general rules for the command line switches:

1. You may enter the command line switches in any order UNLESS a

fixed sequence is defined for a particular utility.

2. You can enter the command line switches as uppercase or

lowercase letters.

3. You may optionally leave one space between different values in

a single command line switch.

7

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

To demonstrate, the following commands all do the same thing (it

sets the data rate to 9600 bps for COM1):

COM_BPS /1 /S9600 <Enter>

com_BPS /s 9600 /1 <Enter>

com_bps /1 /S 9600 <Enter>

com_bps /1/s9600 <Enter>

CoM_BpS/1/s9600 <Enter>

Choose a format to use that is comfortable for you. You will

receive a format error message if you enter a string the utility

doesn't understand.

There are four formats of switches in addition to the help

switch:

3.1 Serial Port Number Switches

Each of the utilities can work on a defined serial port

specified as a number, or on all serial ports that are

standard of defined for the CTS Serial Port Utilities (see

the next chapter for instructions on how to define a non-

standard port.)

The command line switch for port assignments is simply a "/"

(the switch command) followed by the port number. For

example, to run PortInfo on serial port one, you would

enter:

PortInfo /1 <Enter>

Specifying serial port zero (a non-existent port) will cause

the utility, to operate on all of the serial ports. For

example, to run PortInfo on all standard and user-defined

serial ports, you would enter the command:

PortInfo /0 <Enter>

8

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

3.2 On / Off Switches

Some utilities control a serial port function that is turned

on or off. For example, to turn "On" the DTR line for serial

port 2, you enter:

DTR /on /2 <Enter>       -OR-

DTR /ON /2 <Enter>       -OR-

DTR /2 /On <Enter>

Remember, they all do the same thing!

To turn a function off, you use the "/Off" switch rather

than "/On".

3.3 Parameter Switches

There are a number of these switches which you use to enter

a value. One example was shown above when we set the port

data rate to 9600 bps with the "/Sspeed" parameter switch.

Parameter switches are also used to define a non-standard

serial port, to set port formats, receive trigger levels for

a 16550A fifo, and to define a particular interrupt (IRQ) to

use. Check the help screen for each utility to see which

parameter switches are used.

3.4 Option Switches

These are three option switches used in PortInfo to control

how it operates. The switch "/Q" instructs PortInfo to do a

Quick test, and "/L" causes a Log file named  "PORTINFO.LOG"

to be created containing an copy of all messages displayed

to the screen. The "/X" switch allows you to eXclude IRQs

("/XIi") and port addresses ("/XAaaa" or "/XAaaa-bbb") from

the PortInfo test. See the Section "Testing Your System With

PortInfo" for details.

9

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

10

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

4.0 Defining Non-Standard Serial Ports

The CTS Serial Port Utilities will work with any serial port on

any interrupt (IRQ). However, if you have serial ports other than

the standard COM1-COM4, then you have to tell the SPU where the

ports are (that is, what address the port is installed on.) You

have two different ways to do this:

1. you can run each of the utilities using the serial port

address switch "/Aaaa", where "aaa" is the first address location

(the "base address") used by the serial port

2. you can define a system environment variable for each non-

standard serial port that will make the ports available to the

SPU via port numbers.

The CTS SPU recognizes an environment variable with the following

format:

CTSCOMn= /Aaaa

where:

"n" is the serial port number you are assigning to the Com

port

"aaa" is the first address location (the "base address") for

the serial port

NOTE: Make sure there is NO space between the com port number and

the "="

You define an environment variable with the DOS "SET" command,

which you can enter at the DOS command line or place into a batch

file (such as your AUTOEXEC.BAT file.) You DOS manual provides

documentation on the SET command.

Examples:

1. Define a port at base address 1E0 (hexadecimal) as COM5:

CTSCOM5= /A1E0

11

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

2. Define a port at base address 2E0 as COM6:

CTSCOM6= /a 2E0

Using these two examples, you may now access the two non-standard

serial ports as COM5 and COM6 in all utilities of the SPU. These

serial ports will also be used by any utility that is used with

the "/0" (all ports) command line.

NOTE: The CTSCOMn environment variables are not used by any other

applications except SwapIRQ, another CTS product which allows DOS

software to use any IRQ.

12

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.0 Testing Your System With PortInfo

The first thing to do after installing the Serial Port Utilities

is to run PortInfo to do a full system test. If you have a mouse,

make sure that your mouse driver is loaded. PortInfo will

discover many common problems with your serial ports, including:

*  multiple ports installed at the same address

*  interrupt conflicts with other devices

*  conflicts between your mouse and serial ports

*  identifies which serial ports are sharing an interrupt

*  identifies serial ports which are not able to generate an

interrupt

*  possible problems with the DOS definitions for the serial

ports

*  Identifies some displays which conflict with the standard

COM4 serial port

PortInfo will only run from DOS, not from a DOS session within

Windows or OS/2, or under DesqView. These operating environments

prevent the PortInfo utility from directly testing the serial

ports. If you are running Windows, you will need to exit to DOS

before running PortInfo. If you are running OS/2, you will need

to re-boot your system to run DOS before starting PortInfo.

If you have a network card installed, you may temporarily lose

your network connection after running PortInfo. You need to log

out and then back in to the network to re-establish the

connection. See the discussion on "Network Considerations" later

in this section for details on how to avoid this problem.

The summary screen provides information about the type of UART

used in the serial port  and the current settings for the port.

You want to pay particular attention to the FAILURE and WARNING

messages that appear the first time you run PortInfo. All

messages and the summary screen can be captured to a "log file"

for later use or reference.

PortInfo reports a FAILURE message when there is a problem

detected with your system that will prevent correct operation.

Failures are typically reported for conflicts between a serial

port and another serial port or device.

A WARNING message is reported by PortInfo when there is no

immediate problem that you have to fix, but there is a situation

that could lead to a problem if you are not careful. For example,

if you have two different serial ports using the same interrupt,

a WARNING will be reported. If you are careful, you can usually

share the IRQ between the ports - BUT the potential for a problem

exists.

13

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

Both warning and failure messages have a message number. Save the

message number(s) for reference to the section titled "PortInfo

FAILURE and WARNING Messages". You will find each message

explained in more detail with a description of the most common

solutions for the problem.

We've occasionally observed situations with internal modems that

require the computer to have a hard reset before running PortInfo

to obtain correct results. If you observe a problem where

PortInfo does not detect an internal modem or the IRQ used by the

modem, then either turn your computer off and back on to perform

a hard reset, or, if your computer has a RESET button, press it.

Then try running PortInfo again.

NOTE: a soft reset (control-alt-delete) is not sufficient to

reset these modems to a state where PortInfo can recognize them.

You must do a "hard reset" as described above.

5.1 Test All Serial Ports

To start PortInfo to do a complete test of all ports, enter

the following command:

C:> PortInfo /0 <Enter>

This tells PortInfo to do a full test of all standard serial

ports (remember, /0 is the command for all ports), and all

non-standard serial ports you have defined using the CTSCOMn

environment variables for the CTS Serial port Utilities.

NOTE: If you have a non-standard serial port installed, it

will not be tested unless you have defined a CTSCOMn

environment variable.

14

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.2 Test One Serial Port

You can also have PortInfo test only a single serial port.

You can tell PortInfo to test a single serial port in two

different ways. One way is to specify the serial port number

you want to test. For example, the following command will

test COM2:

C:> PortInfo /2 <Enter>

You can also test a serial port by specifying the serial

port address. This is the only way you can test a non-

standard serial port without a CTSCOMn environment variable.

For example, if you want to test the port at address 01E0

(hex), you would enter:

C:> PortInfo /A 1E0 <Enter>

5.3 Quick Test

You may tell PortInfo to do a "quick test" if you are in a

hurry to just check out a port setting. The quick test skips

most of the conflict tests, so it will generally not detect

port conflicts. You can specify the quick test for both a

single port test and the complete system test. To specify

the quick test, add a "/Q" (for Quick) to the command line

as shown below:

Example 1: Do a quick test on all ports:

C:> PortInfo /0 /Q <Enter>

Example 2: Do a quick test on COM3:

C:> PortInfo /3 /Q <Enter>

15

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.4 Log File

PortInfo can create a copy of all messages and the summary

screen on your disk that you can use to refer to when

installing new equipment in your computer. When you use the

"/L" command line switch, PortInfo creates a file named

"PORTINFO.LOG" in your default directory. This file contains

all information that is displayed on the screen while

PortInfo runs.

Example: Do a full system test, and save the results in the

"PORTINFO.LOG" file:

C:> PortInfo /0 /L <Enter>

16

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.5 Network Considerations

While searching for serial port and interrupt (IRQ)

conflicts, PortInfo performs a very intensive test of your

computer system interrupts. One result of this testing is

that it is possible to lose your network connection after

running PortInfo. You can restore your network connection by

just logging out and back in, or you can avoid the problem

entirely if you know what IRQ your network card is using.

PortInfo includes the ability to exclude any IRQ or port

address from the test. By ensuring that your network card

IRQ is excluded, you will avoid interference with the

network. If your network card is installed at an address

that conflicts with a normal serial port address, you should

also exclude the address from the test.

To exclude one or more IRQs from being tested, you can use

the "/XIi" command line switch. The "/XIi" switch causes

PortInfo to eXclude Irq "i" from being tested.

Example: Exclude IRQ 5 (used by the network card) from a

full system test:

C:> PortInfo /0 /Xi5 <Enter>

NOTE: you can exclude multiple IRQs by using multiple "/XIi"

commands

Example: Exclude a network card at address 2E8 using IRQ 5

from a full system test:

C:> PortInfo /0 /Xi5 /XA2E8 <Enter>

This example demonstrates the use of the eXclude Address

switch "/XAaaa". This switch will prevent PortInfo from

testing a port residing in the address block including

address "aaa".

NOTE: an "address block" is defined for this discussion as

eight consecutive addresses starting with an address ending

in "0" or "8".

The example above excludes testing of the eight addresses

located at 2E8-2EF.

17

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.6 The PortInfo Summary Screen

PortInfo completes operation by displaying a summary of all

information it gathered while testing. The following is a

sample display for a system with four or less serial ports

(eight columns are displayed for more serial ports.) Each

item on the display is discussed in the following sections.

PortInfo Summary Screen

Serial 1   Serial 2  Serial 3  Serial 4

Port Address (Hex)           3F8        2F8       3E8       N/A

Interrupt (IRQ)                4          3        10

IRQ (Out2) Enabled            No        Yes        No

DOS uses port as            Com1      Mouse      Com3       N/A

Type of Port              T/Port      16450    16550A   Display

Buffer - Trigger            None       None      On-8

Speed                      19200       1200      2400

Bits / Character               8          7         8

Parity                      None       None      None

Stop Bits                      1          1         1

Clear to Send                 On        Off       Off

Data Set Ready                On        Off       Off

Ring Indicator               Off        Off       Off

Data Carrier Detect           On        Off       Off

Data Terminal Ready          Off         On       Off

Request to Send              Off         On       Off

5.6.1 Port Configuration

5.6.1.1 Port Address

This line shows the "base port address" in

hexadecimal for each serial port detected. The

"base port address" is the first address used by

the serial port. Serial ports normally use eight

address locations, so the term "base port address"

is used to refer to the first address used. In the

example displayed above, the COM1 base port

address is 03F8 (hex). The serial port actually

uses eight consecutive addresses starting at 03F8

and ending at 03FF (hex).

18

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.6.1.2 Interrupt (IRQ)

The Interrupt (IRQ) line of the display shows the

actual hardware interrupt used by the serial port.

This value is determined by testing the serial

port. If a NOTE is displayed in this field, then

there is a possible conflict with this port that

will cause a problem if you try to use the port.

5.6.1.3 IRQ (Out2) Enabled

This line shows the current status of the OUT2

line for the serial port. The OUT2 line is a UART

control line used by some communication software

to enable the IRQ. A serial port cannot generate

interrupts when this line is disabled, OR when

another serial port that uses the same IRQ line is

enabled.

If you have two serial ports sharing an IRQ with

an "Enabled" status, or if you have a problem

using one port when the other shows an "Enabled"

status, then you need to disable the OUT2 line on

the inactive port. You can do this with two

different utilities provided in the SPU.

"ResetCom" will restore a serial port to it's

original power-up status, and "IRQ" provides you

with direct control of the IRQ (Out2) line. These

utilities are described below.

5.6.1.4 DOS uses port as

This line will show how DOS expects to use this

port. If a mouse is detected, the word "Mouse"

will be displayed as shown for above for Serial 2.

Otherwise, the field will contain the DOS default

port assignments as detected by your system bios

at boot-up time. If the letters "N/A" are

displayed, then the port is Not Available for use

by commands like the DOS Mode command. You can

adjust these port assignments with the DOS_COM and

DOS_SWAP utilities provided with PortInfo.

19

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.6.1.5 Type of Port

The Type of Port line describes the type of UART

detected by PortInfo. If no UART was detected for

the port, the letters "N/A" will be displayed.

Also notice that the word "Display" appears on the

COM4 port. There are some display cards, including

the Diamond Stealth, the ATI Ultra Graphics, and

others which emulate IBM 8514/A displays that use

the address space normally reserved for COM4.

PortInfo detects the presence of many of these

displays and identifies the port as such.

NOTE: that there are two types of UARTs which

software cannot tell apart: the 8250A and the

16450. PortInfo chooses to display this UART type

as a "16450" since the 16450 is the most common

UART used today. Other diagnostic software may

identify the UART as an 8250A (or even an 8250,

which is incorrect).

5.6.1.6 Buffer - Trigger

One of the unique features of PortInfo is it's

ability to sense not only the status of the FIFO

buffer on a 16550A, but also to detect what

receiver trigger level is being used. This line

will display "None" for those UARTs that do not

have buffers, Off for 16550A compatible UARTs

whose buffers are not enabled, and On-#, where #

will be 1, 4, 8, or 14 to show that the FIFO

buffer receiver trigger is set to.

The receiver trigger level for a 16550A UART

determines when it will issue an interrupt

request. If the trigger is set to 14, the UART

will issue the interrupt after 14 of the 16 spots

in the buffer are filled. This reduces the number

of interrupts generated by the UART, but only

leaves three character times for the PC to service

the interrupt before data is lost. Reducing the

trigger level to 8 increases the number of

interrupts (1 interrupt for every 8 characters

received), but greatly reduces the possibility of

data loss since the PC has 9 character periods

before an overrun will occur. If you experience

data loss at high speed in a communication

program, try exiting to DOS while connected and

run PortInfo on the port. If the Buffer - Trigger

shows "Off", then your software may not support

the 16550A UART. You can try enabling the buffer

20

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

with the trigger set to one (1) with the "Buffer"

utility and then exit to your communication

software. This change may cause your communication

software to lock up, however. If so, your only

recourse is to contact the software manufacturer

for an update, or change to another communications

application.

If PortInfo displays "On-14", try using the

"Buffer" utility to reset the trigger level to

eight (8). This should reduce or eliminate the

overrun problem. Making this change should not

adversely affect your communications software, but

may eliminate your overrun problems.

5.6.2 Port Settings

The following port settings are normally controlled by

your communication software.

5.6.2.1 Speed (BPS)

This line shows the current port data rate in bits

per second (BPS). You normally control this value

in your communication program, but you can change

this value with the DOS Mode command, or with the

COM_BPS utility provided with PortInfo. The

advantage of the COM_BPS utility is that it allows

setting data rates up to 115,200 bps, and supports

ports not recognized by DOS.

NOTE: to convert to characters per second, you

normally divide the BPS value by 10. That is, a

serial port sending characters at 19,200 bps is

sending 1,920 characters (or bytes) per second.

5.6.2.2 Bits / Character

The bits per character line displays the number of

data bits used for each character. Allowable

values are 5, 6, 7, and 8. You can control the

bits per character with the COM_FMT utility

provided with PortInfo. This setting is normally

controlled by your communications software, but is

provided here for control of non-standard ports

not supported by the DOS Mode command.

21

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.6.2.3 Parity

The Parity line shows the current parity bit

setting for the port. The possible values are

None, Even, Odd, Mark, and Space. You can control

the port format with the COM_FMT utility provided

with PortInfo.

5.6.2.4 Stop Bits

This line shows the number of stop bits sent with

each character. Valid values for the number of

stop bits are 1, 1.5, and 2. You can control the

number of stop bits with the COM_FMT utility

provided with PortInfo.

5.6.3 Hardware Signals

The serial port hardware signals are used by different

modems and devices in different ways. The RTS line is

often used for hardware flow control, or it may be used

to provide power to a portable modem. It's also

possible for a cable, connector, or a serial port board

to provide a means of looping back, or connecting lines

to each other. If you see a hardware line listed as "On

- DTR" or "Off - RTS", then this means that the line

indicated is in the state shown, and it *may* be

connected to the line indicated. These results can be

affected by the devices connected to the serial port.

You can disconnect the device and cables to determine

where the loopback condition occurs.

Common uses for each line are discussed below.

5.6.3.1 Clear to Send

The Clear To Send (CTS) line is commonly used by

modems and other devices for "hardware flow

control". Flow control is used to prevent data

from being sent faster than it can be processed.

The CTS line is input only - it is controlled by

the device attached to your serial port. Your

modem can use the CTS line to start and stop the

flow of data sent by your computer when your

software supports hardware flow control.

The CTS line is normally "Off" when the serial

port has nothing connected to it, or if it is

connected to a device like a serial mouse or a

22

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

scanner. When your serial port is connected to a

Hayes compatible modem, and the modem is turned

on, CTS should be "On" at all times except when

the modem is connected to a remote system, and the

modem has signalled the software to stop sending

information. If you detect that CTS is not "On"

for your modem when there is no connection, then

you may have one of the following conditions: 1)

Your cable between the serial port and the modem

does not contain the CTS line. This means you will

NOT be able to use hardware flow control with your

modem; 2) Your modem is not a true Hayes-

compatible modem; 3) Your modem is not operating

correctly; or 4) Your serial port is not working

correctly.

5.6.3.2 Data Set Ready

The Data Set Ready (DSR) is a line which

historically is used by modems (data sets) to

signal when they are connected to a remote modem

and ready to send data. This line is input only -

it is controlled by the device attached to your

serial port.

Most modems today allow you to configure how this

line is used. The default for Hayes and compatible

modems is to turn this line "On" when the modem is

powered on. On some modems, you can also configure

this line to be "Off" at all times except when

connected with a remote modem.

5.6.3.3 Ring Indicator

The ring indicator line is normally used by modems

to signal that a ring was detected on the phone

line. This line is input only - it is controlled

by the device attached to your serial port.

PortInfo will normally detect this line as "Off"

with a modem unless you manage to get a ringing

signal at the exact moment PortInfo checks the

status of the line.

23

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.6.3.4 Data Carrier Detect

The Data Carrier Detect (DCD) line is normally

used by a modem to signal that the modem is

connected to a remote modem and ready to exchange

information. This line is input only - it is

controlled by the device attached to your serial

port.

Many modems can be configured so that DCD is "On"

all of the time with a command (the &C command is

used by Hayes and true compatible modems) or with

a switch on the modem.

5.6.3.5 Data Terminal Ready

The Data Terminal Ready (DTR) line is a line

output by the serial port to the connected device.

Historically, the DTR line was used by old "dumb

terminals" to signal the modem that they were

ready to receive data.

The DTR line typically has two different uses

today with modems. Most common is to use the DTR

line to end a connection. Your modem can look for

a signal from your communication software to force

a hangup. In some cases, this will also hang up

the modem.

The second use for this line is to provide power

to the "line powered" portable modems introduced

over the last couple of years. Some portable

modems have no power supply or batteries, and

actually use the telephone line, the DTR line, and

the RTS line (see below) to supply their power. If

you have one of these modems, then make sure that

DTR is high if you try to use the modem. Most

communication software does this automatically,

but some utilities that send modem commands from

the DOS prompt may not. You can use the "DTR"

utility that comes with PortInfo to control the

DTR line.

24

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

5.6.3.6 Request to Send

The Request to Send (RTS) line is another line

output by the serial port to the connected device.

This line typically is used for hardware (RTS/CTS)

flow control. When this line is "On", the serial

port is requesting that the modem send data.

A second use for this line is to provide

additional power to the "line powered" modems as

discussed in the paragraph above. You can use the

"RTS" utility provided with PortInfo to control

the RTS line.

25

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

26

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

6.0 The Other Utilities

There are a number of additional utilities provided with

PortInfo. These utilities are very specific in nature, and are

provided to give you a standard user interface and source of

support. You can also use these utilities to test the PortInfo

utility.

6.1 BUFFER

BUFFER controls the internal buffer in a 16550A or

compatible serial port. You can use this utility to turn the

FIFO buffer "On" with any of the four ( 1, 4, 8, and 14)

valid receiver trigger levels, or to turn the buffer off. In

general, application software which can support a 16550A

UART will control the buffer correctly.

The primary use of this utility is to: 1) turn off a 16550A

buffer which was not turned off by an application; and 2) to

provide a way to adjust the buffer setting for a software

application which has the trigger level set too high.

If you have old software which does not support a 16550A,

and you are experiencing an occasional overrun error (this

often appears as a retry during a file transfer with an

"error-free" connection), then you can try turning the

buffer on with different trigger levels. This is an

experimental process, and it may not work. In fact, it's

possible your communication software will "lock up" if you

do this.

From the DOS prompt, type "BUFFER" or "BUFFER /?" to display

the command line options and examples.

NOTE: If no trigger level is specified on the command line,

BUFFER will set the receiver trigger level to 14.

27

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

6.2 COM_FMT

COM_FMT sets the character format on a serial port to the

specified value. You can use this to replace the MODE

command provided with DOS on ports that DOS does not detect.

The following chart shows the three character "format" value

used on the command line and the associated description:

Format         Description

8N1          Eight data, No parity, 1 stop bit

8N2          Eight data, No parity, 2 stop bits

7E1          Seven data, Even parity, 1 stop bit

7E2          Seven data, Even parity, 2 stop bits

7O1          Seven data, Odd parity, 1 stop bit

7O2          Seven data, Odd parity, 2 stop bits

7M1          Seven data, Mark parity, 1 stop bit

7M2          Seven data, Mark parity, 2 stop bits

7S1          Seven data, Space parity, 1 stop bit

7S2          Seven data, Space parity, 2 stop bits

From the DOS prompt, type "COM_FMT" or "COM_FMT /?" to

display the command line options and examples.

NOTE: most communication applications control this parameter

directly and will change whatever you set with this utility.

6.3 COM_BPS

COM_BPS sets a serial port data rate (in bits per second, or

BPS) to any of the common standard values supported by the

serial port. You can use this to replace the MODE command

provided with DOS on ports that DOS does not detect. The

following data rates are supported by COM_BPS: 110, 300,

600, 1200, 2400, 4800, 7200, 9600, 14400, 19200, 38400,

57600, and 115200 bits per second (bps). From the DOS

prompt, type "COM_BPS" or "COM_BPS /?" to display the

command line options and examples.

NOTE: most communication applications control this parameter

directly and will change whatever you set with this utility.

28

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

6.4 DOS_COM

The BIOS of your computer system creates a list of detected

serial ports when it boots. Unfortunately, many computers

will only detect the standard serial ports COM1 and COM2.

Some computers will look for the conventional COM3 and COM4

ports as well. You can use the PortInfo summary screen line

"DOS uses port as" to determine whether the BIOS has

detected the serial port.

DOS will only use ports that are detected in the BIOS list

of serial ports. For example, DOS must know a COM3 port

exists if you want to redirect LPT1 to COM3, where you have

a serial printer. If PortInfo shows "DOS uses port as N/A",

then you need to help it out and add COM3 to the list of DOS

ports. You use DOS_COM to do this. From the DOS prompt, type

"DOS_COM" or "DOS_COM /?" to display the command line

options and examples.

6.5 DOS_SWAP

DOS_SWAP exchanges two port addresses in the list of serial

ports used by DOS. From the DOS prompt, type "DOS_SWAP" or

"DOS_SWAP /?" to display the command line options and

examples. You can use the PortInfo summary screen line "DOS

uses port as" to determine whether how your BIOS has

assigned the serial ports it detected.

6.6 DTR

DTR controls the Data Terminal Ready line for the port

specified. You can raise (turn "On") or lower (turn "Off")

the DTR line with this utility. See the discussion on the

DTR line in the section dealing with the PortInfo summary

screen for applications of this utility. From the DOS

prompt, type "DTR" or "DTR /?" to display the command line

options and examples.

6.7 IRQ

IRQ controls the interrupt request line (Out2) from the

serial port specified. Generally, a serial port should have

IRQs disabled (IRQ Status "Off" on the PortInfo screen) at

all times except when the serial port is actually being

used. The major exception to this rule is when a serial

mouse is attached to the serial port. The mouse driver will

enable the interrupt and keep it enabled at all times.

29

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

If you want to try sharing an interrupt between a mouse and

a modem, you can in some cases manage this with IRQ. For

example, if you have a mouse on COM1 and a modem on COM3,

both using IRQ 4, you can run IRQ to disable interrupts on

COM1. Then run your communication program. When you are

finished with your communication program, you can (but

normally you don't have to) run IRQ to re-enable the COM1

interrupt.

NOTE: this will NOT work reliably with a communication

application that is "mouse aware". If you need to share an

interrupt with a mouse-aware application, then you need to

use a utility similar to the CTS "MouseOff" and "MouseOn"

utilities.

From the DOS prompt, type "IRQ" or "IRQ /?" to display the

command line options and examples.

6.8 RESETCOM

RESETCOM resets a serial port to the DOS default condition.

Interrupts are disabled, the port is set to 2400 bits /

second with a character format of 8 data bits, no parity,

and 1 stop bit, and the Request to Send (RTS) and Data

Terminal Ready (DTR) lines are set low. If the port is a

16550A or compatible, the buffers (FIFOs) are also disabled.

There are a few communication applications which do not

properly restore a serial port after completion. ResetCOM

can be used to completely reset the serial port to a well-

defined state.

From the DOS prompt, type "RESETCOM" or "RESETCOM /?" to

display the command line options and examples.

6.9 RTS

RTS controls the Request to Send line for the port

specified. You can raise (turn "On") or lower (turn "Off")

the RTS line with this utility. See the discussion on the

RTS line in the section dealing with the PortInfo summary

screen for applications of this utility. From the DOS

prompt, type "RTS" or "RTS /?" to display the command line

options and examples.

30

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

7.0 PortInfo FAILURE and WARNING Messages

The different types of failures and conflicts detected by

PortInfo are separated into different categories. Each FAILURE

and WARNING message can be referenced by number to the list

below. Steps you can take to correct each of the problems are

described.

The general rule is to pay the most attention to the first

FAILURE message that displays. Once a FAILURE has been detected,

it should be corrected before looking at other problems that may

be reported.

7.1 Interrupt Conflicts

An interrupt conflict message indicates that one or more of

your serial ports is not always generating interrupts. This

can cause a number of "interesting" problems that sometimes

appear and disappear for no apparent reason. Finding the

cause of the problem requires both a tool like PortInfo to

identify that the problem exists, and patience on your part

in taking a systematic approach to fixing the problem.

Please follow the directions carefully.

7.1.1 Message PI-2010

Symptoms: You may have regular or sporadic lockups

using either of the ports at this address.

Solution: As the message indicates, PortInfo is

detecting two separate interrupts when testing a single

serial port address. The typical cause of this is two

different serial ports at the same address (eg, two

separate UARTs defined as COM1). To fix this problem,

it is necessary disable, remove, or reassign one of the

serial ports to another address location.

31

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

7.1.2 Message PI-2020

Symptoms: You will likely get a system lockup, a high

rate of CRC errors (file transfer retries) if using a

modem, a very jerky motion on a serial mouse, or a

combination of these symptoms if you do not resolve

this problem.

Solution: If your software allows use of another port

or IRQ, then you can change the port settings to avoid

this IRQ. A second approach is to temporarily remove or

disable the serial port, and then to run a general

system diagnostic utility like Microsoft's MSD to

determine which other device is using the IRQ. Then you

can decide which one of the two will actually use the

IRQ.

7.1.3 Message PI-2100

Symptoms: You have a serial port that works sometimes,

but not all of the time. You may have any or all of the

symptoms listed for any message in the PI-2000 to PI-

2999 range.

Solution: The usual causes for this error message are

multiple serial ports at the same address, or another

device using the interrupt. Use the solutions described

for PI-2001 and PI-2002.

7.1.4 Message PI-2110

Symptoms: Most applications will lock up when trying to

use this port.

There are three common causes for this message.

Troubleshooting this problem requires a careful and

methodical approach. The basic idea is to reduce your

system to a state where the conflict is no longer

observed, and then add boards back in one at a time to

determine which board is involved with the conflict.

Often (but not always) the board you have most recently

added to your system is the culprit.

Diagnosing and repairing this problem often involves

opening your computer system and removing / installing

boards. Be sure to follow the instructions provided

with your computer system when adding and removing

boards.

32

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

PortInfo is most useful at diagnosing problems if you

run it after installing each board. By making sure that

the system is ok after each board is added, you can

then quickly identify which board is causing the

problem.

7.1.4.1 You have multiple ports at the same

address

This is a common problem with many computers.

There is often a forgotten or unknown serial port

installed in the computer, and adding another port

at the same address location can cause all sorts

of interesting problems. The best way to approach

this problem is to remove all of the add-in boards

that have serial ports from your system. This

includes internal modems as well. Then run

PortInfo to verify that your system is no longer

displaying this error message. If the error still

exists, then it is likely due to one of the other

problems discussed here. If the problem did go

away, then add each of the boards back into your

computer one at a time to determine which board

causes the problem.

After you have identified which board causes the

problem, you should change the serial port address

(or Com# setting) to a selection which not used on

your system.

33

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

7.1.4.2 You have another device using the same

interrupt

Many different devices compete for interrupts with

your serial ports, including other serial ports

(including internal modems), bus mice, network

cards, sound cards, and so on to name several.

While well-behaved serial ports and software will

allow serial ports to share an IRQ, you generally

cannot share an IRQ between a serial port and

another device. Bus mice often use IRQs 2,3,4,5,

or 12, and sound cards commonly use IRQs 5 and 7.

Try removing these cards from your system and then

run PortInfo again to see if the problem still

appears. If the problem still exists, then leave

the boards out and continue diagnosing the problem

following the steps in (1). When you have the

system to the point where the error is no longer

reported, then begin adding your boards back to

the system one at a time, running PortInfo after

you've added each board to determine which board

is involved in the conflict.

7.1.4.3 Your system will not let you share this

interrupt

There are some computer system motherboards which

simply will not let any other device generate

interrupts for IRQs used on the motherboard. For

example, if you have a serial port on one of these

motherboards set to use Com1 / IRQ 4, no other

device will be able to use IRQ 4.

Your only solution, if you need to share these

interrupts to make everything fit, is to disable

the serial ports on the motherboard, and purchase

an add-in serial port card. CTS offers several

options, so be sure to give us a call.

34

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

7.1.5 Message PI-2200

This is an informational message regarding the Telcor

Systems T/Port card. A newer ROM exists for your

T/Port. If you are experiencing problems with the

T/Port, then you should contact Telcor Systems Inc. for

more information.

NOTE: PortInfo is not able to test the IRQ used by the

T/Port when this message is displayed.

7.1.6 Message PI-2300

Symptoms: If you use well-behaved software, and if your

hardware allows sharing interrupts, then everything

will be fine. If not, then you can have problems

ranging from serial ports which work intermittently to

complete system lockups.

Solution: Make sure the hardware will allow the

interrupt to be shared among multiple devices, and that

your software will make the IRQ inactive on the board

it uses after it has completed use. You may not use two

devices at the same time on the same interrupt.

7.2 Port Conflicts

Port conflicts normally indicate that there are two serial

ports located at the same address. This is commonly done

when installing new serial cards or modems into an older

machine without knowing for certain what equipment is

already installed.

7.2.1 Message PI-3000

Symptoms: System lockup or failure of a communication

application to successfully use a modem.

Solution: You definitely have a conflict between a

serial port and another device, possibly another serial

port, at the same address location. Remove or disable

the port which fails the test, and run PortInfo again.

If another port is reported at the same address, then

you have a conflict between two serial ports. If no

other serial port is detected at that address, then you

have a conflict with another device. Common conflicts

involve local area network cards.

35

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

7.2.2 Message PI-3100

Symptoms: Your system may lock up, or the display may

show meaningless characters ("garbage").

Solution: You cannot use a serial port at the normal

COM4 address location if this message appears. The

display adaptor on your video card is installed at the

address normally used for Com4.

7.3 Mouse Conflicts

PortInfo looks for conflicts between your mouse and your

serial ports. Most conflicts are detected by the interrupt

conflict tests. However, PortInfo also checks for possible

future conflicts.

7.3.1 Message PI-4000

This is a WARNING message that your bus mouse is using

one of the IRQs normally used by the Com1 or Com2

serial ports. If you are not using this port for any

connections, then you will not have any problems. If

you do use Com1 and Com2 for your communications and

they are using the standard IRQs (Com1 / IRQ 4, Com2 /

IRQ 3), then you need to change the IRQ used by the

serial port or the bus mouse to resolve the conflict.

7.4 DOS Port Assignments

The DOS port assignments are determined by a data table that

is created by your computer system BIOS when the system is

powered up or reset. The BIOS detects the ports installed in

your computer and places them in a data area for use by DOS

and application software.

Unfortunately, different BIOS implementations work

differently, so there are often inconsistencies in how the

serial ports are used. Many systems will only look for two

comm ports, and no system looks for more than four. The BIOS

also doesn't follow conventions in assigning a port at a

particular address to the proper Com port number. To further

confuse the issue, many applications do not use this data

area because of the inconsistencies and address the ports

directly by address. This leads to the situation where two

different applications use the same device through

"different" ports.

36

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

The best general rule is to always install ports in

sequential order...that is, do not skip a port number. Make

the first port Com1, the second Com2, and so on.

PortInfo attempts to make sense of what is in your data area

so you can adjust, or at least understand what is going on.

7.4.1 Message PI-5000

Symptom: You've assigned the port speed with the DOS

"MODE" command, but it is changed by a DOS MODE command

to another port.

Solution: You've apparently run a utility to swap or

install additional serial ports into the BIOS data

area. It is usually best to restore the ports to their

normal locations after you use the application which

required the exchange. See the discussion on the

DOS_SWAP or DOS_COM utilities provided with the SPU

will do this.

7.4.2 Message PI-5010

Symptom: PortInfo does not detect a serial port that

your system detected. This can be caused by the

improper use of the DOS_COM (or similar) utility, or

you may have a non-standard serial port the PortInfo

cannot detect (at the time this manual was written,

PortInfo successfully detects all known ports).

Solution: Make sure that you do not have a utility that

is "installing" a non-existent port into the BIOS data

area.

37

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

7.4.3 Message PI-5020

Symptom: You have a device that is accessed through a

serial port in one application, but has to be accessed

through a different serial port (or cannot be accessed)

in another application.

Solution: The problem is that the serial port indicated

is not in the correct location in the BIOS data table.

Causes for this include improper use of port

reassignment or installation utilities like DOS_SWAP

and DOS_COM, or you do not have your serial ports

installed in sequential order. You can fix the problem

by making sure that your ports are installed in

sequential orders, starting with Com1, and then

avoiding the use of the port installation/reassignment

software.

7.4.4 Message PI-5030

Symptom: You've just installed a new serial port, and

one or more applications cannot detect it.

Solution: If your application uses the BIOS data area,

then you will need to install the serial port using the

DOS_COM or similar utility. Many BIOS implementations

do not detect Com3 and Com4 serial ports, so this is a

common problem.

7.4.5 Message PI-5040

Symptom: You have a serial port installed that is not

detected one or more of your applications.

Solution: See the solution for PI-5030.

7.5 Other CTS Software Detected

The CTS Utility SwapIRQ can affect the reporting of IRQs

used by some diagnostic software. PortInfo is "aware" of

SwapIRQ and notifies you that it is still active.

38

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

7.5.1 Message PI-6000

Symptoms: Just an informational message to let you know

that SwapIRQ was detected and temporarily disabled by

PortInfo. The PortInfo reports will not be affected by

the presence of SwapIRQ.

Solution: You should disable SwapIRQ when your

application has finished with it by using the "/D"

command line switch.

39

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

40

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

8.0 Customer Support Policy

Customer support is provided via telephone, fax, and electronic

mail. We provide from 30 to 90 minutes of support (depending on

the license you obtained) for any registered user within the

following guidelines:

1. We are in the office and happy to assist with your problems

during normal business hours (US east coast). We always answer

the phone whenever we are in (other hours). If you are unable to

catch us "live", please leave a detailed message, we will reply

via fax, electronic mail (E-mail), or regular mail.

2. Messages sent to us on CompuServe E-mail will usually receive

a reply within 24 hours.

3. We will be happy to assist with SPU installation problems

before you register. However, we cannot provide technical

assistance until we've actually received your registration. We

can register you over the phone via credit card if you need

immediate assistance.

Be prepared to answer the following questions before you call for

assistance:

*  The version number of your CTS Serial Port Utilities. You

can view this information in the program title by starting

any of the utilities.

*  The brand name and model number of your computer.

*  What equipment you have in your computer. A general purpose

diagnostic program such as Microsoft's MSD can be helpful in

determining what other equipment is installed.

*  The version of DOS you are using (type "VER" at the DOS

prompt).

*  A complete description of the problem.

*  Can you duplicate the series of steps that results in the

problem?

*  Did an error message appear? What did it say?

You may contact CTS via voice phone at 404-263-8623 during normal

business hours (Eastern Time Zone), or 24 hours / day via FAX at

404-263-0124 and on CompuServe at 76662,2315.

41

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

42

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

9.0 About Computer Telecommunication Systems

Computer Telecommunication Systems, Inc. was formed in 1990 to

provide new communication software products, custom programming

services, and consulting on personal computer asynchronous

communications. We currently offer (as of February, 1994) the

following products and services:

9.1 AT Command Test

AT Command Test (ATCT) is a software application that

performs a comprehensive test of the AT command set of any

modem claiming to be "Hayes Compatible". ATCT tests most of

the standard modem commands, all result codes, and many

other characteristics. ATCT provides three different reports

of command set compatibility with nine different Hayes modem

product lines.

ATCT is used today both as a qualification tool for

purchasing modems by major corporations, and as a diagnostic

tool for testing modems returned to corporate support

services prior to obtaining warranty service.

Contact CTS for more information.

9.2 SwapIRQ

A small utility that allows software to use IRQs that are

not supported directly by the software. For example, a

communication application that supports COM3 on IRQ 4 can

now use COM3 on any IRQ. Or, a game that requires a sound

card on IRQ 7 can now use a sound card that generates

interrupts on IRQ 5. Contact CTS for more information.

9.3 MouseOFF and MouseON

MouseOff disables a serial mouse, allowing a modem or other

device using the same interrupt (IRQ) to operate without

interference. MouseOn re-enables the mouse after you have

finished using the other device. MouseOff and MouseOn are

especially useful in sharing an interrupt between a mouse

and a modem when the communications software detects and

tries to use the mouse.

9.4 Serial Cards (Comm Ports)

We carry a variety of serial cards that contain 16550A or

compatible UARTs.

43

Copyright 1993-1994, Computer Telecommunication Systems, Inc.

9.5 Cables / Adapters

We have a variety of serial cables, null modems, and 9 pin /

25 pin adapters.

9.6 Custom Programming Services

Call us to discuss your special communications software

requirements.

9.7 New Products

This manual may be several years old by the time you read

this! Call us to see what's new!

44