File ODIPKT.DOC

NAME

ODIPKT

Translate Open Data-Link Interface to Packet Driver interface.

SYNOPSIS

ODIPKT [ logical_board [ vector ] ]

ODIPKTpn [ logical_board [ vector ] ]

DESCRIPTION

ODIPKT provides a Packet Driver API over Novell's Open Data-Link

Interface (ODI), allowing a single network adapter to service IPX,

TCP/IP, and other protocol stacks which use packet drivers.  ODIPKT

understands Ethernet, Token Ring, and ARCnet frame types.  Ethernet and

Token Ring frames are passed unchanged to packet driver applications, but

ARCnet frames are internally mapped to Ethernet.  Thus, packet driver

applications do not need specific knowledge of ARCnet in order to operate

in such environments.

The logical_board argument indicates which logical adapter ODIPKT should

bind to.  Under ODI, there is one logical adapter per frame type.  There

may be multiple physical adapters, and multiple frame types per physical

adapter.  The logical adapter number starts at zero and increases by one

for each frame type.  For example, if a system contained two physical

adapters with two frame types each, there would be four logical adapters

numbered 0-3.  The frame types are specified in the NET.CFG file which

must be in the current directory (or on the APPEND path) when the ODI

drivers are loaded.

Typically, the frame type used by default by Netware on a given adapter

will not be appropriate for TCP/IP.  The correct frame type for TCP/IP on

Token Ring is TOKEN-RING_SNAP and the correct frame type for TCP/IP on

Ethernet is almost always ETHERNET_II.  Assuming an existing installation

with one adapter running the default Netware frame type, it will usually

be necessary to define a second frame type (as listed above) and to

specify the logical_board argument as 1.  An exception to this rule is

ARCnet which uses only one frame type.  Below is a NET.CFG entry for a

3c501 using the default address and interrupt.  Other Ethernet adapters

would use similar configurations:

Link driver 3c501

envelope type ETHERNET_802.3

envelope type ETHERNET_II

Given the above entry, the logical_board argument of ODIPKT would be 1.

Token Ring uses different frame types:

Link driver token

envelope type TOKEN-RING

envelope type TOKEN-RING_SNAP

Again, the logical_board should be specified as 1 for this nexample.

ARCnet installations require no frame-type definitions and should use a

logical_board of 0.

Because ODIPKT.COM uses buffers provided by the Link Support Layer, the

Link support section of NET.CFG must specify at least one buffer of a

size appropriate to the medium in use.  Some adapters require more

buffers for optimal performance, e.g., the 3c503 can use at least five

during reception of back-to-back packets.  The following lines configure

five buffers large enough for Ethernet:

Link support

Buffers 5 1600

The correct buffer size for Token Ring will vary depending on the

installation.  Specifying too small a buffer size will result in

unpredictable behavior.

Vector is the packet driver interrupt at which to provide service.  The

default is 105 and the parameter is decimal.

ODIPKT.COM should be loaded after lsl.com and the adapter-specific

driver(s).  It must, of course, be loaded before the packet driver

applications which use it.  The Netware shell and ipxodi.com can be

loaded before or after the ODIPKT.COM program.

ODIPKTPN.COM supports an older version of the ODI specification as used

in Farallon's PhoneNET Talk product.  Operation of ODIPKTPN.COM is

identical to that of ODIPKT.COM but configuration of NET.CFG is somewhat

different: no ``envelope'' statements are required or allowed.  Because

multiple frame types are not supported, ODIPKTPN.COM treats all Ethernet

cards as ETHERNET_II.

AUTHOR

Dan Lanciani

185 Atlantic Road

Gloucester, MA 01930

The following copyright notice appears in the ODIPKT.ASM source file:

; ODIPKT.ASM - Adapter provides Packet Driver interface over ODI

;

; (c) Copyright Daniel D. Lanciani 1991-1994.  All rights reserved.

;

; This unmodified source file and its executable form may be used and

; redistributed freely.  The source may be modified, and the source or

; executable versions built from the modified source may be used and

; redistributed, provided that this notice and the copyright displayed by

; the exectuable remain intact, and provided that the executable displays

; an additional message indicating that it has been modified, and by whom.

;

; Daniel D. Lanciani releases this software "as is", with no express or

; implied warranty, including, but not limited to, the implied warranties

; of merchantability and fitness for a particular purpose.

;

; Please send bug reports to:

; Dan Lanciani

; 185 Atlantic Road

; Gloucester, MA  01930

; USA

; Voice: +1 508 283-4974

; Email: ddl@harvard.harvard.edu (Internet)

USING ODIPKT WITH MS-DOS KERMIT

ODIPKT is normally not needed, since MS-DOS Kermit 3.13 and later includes

its own ODI interface, and can run on top of your network board's ODI driver.

Under Microsoft Windows, however, unless Kermit is "locked in memory", Windows

will move it around causing system crashes (or worse) when the ODI driver

goes to write incoming data into Kermit's memory space (this is not a Kermit

problem -- it would happen with any network application).  Therefore we need

to run Kermit over the WINPKT "shim".  But WINPKT goes over a packet driver,

not an ODI driver.  That's where ODIPKT comes in -- it makes the ODI driver

look like a packet driver to WINPKT, which, in turn feeds packets to Kermit.