* ------------------------------------------------------------------------- *

Disect V1.7 -  DMA 1994-5	26.11.94

==========================	========

1) Introduction

2) Disect Request Windows

3) The Program Screen

4) A Disassembler Tutorial

5) Symbol Selection Window

6) Address Functions

7) DataStrings

8) Key Macros

9) Expressions

10) MultiBin Files

11) Screen Icons Reference

12) Keyboard Reference

13) Mouse Reference

14) Menu Reference

15) Process Variables Memory

16) Grabbing An Existing Process

17) Auto Trace Mode

18) Hardware Monitor

19) 68020 Modes/Instructions

20) Installing To Hard Drive

21) Contact Address

* ------------------------------------------------------------------------- *

1) Introduction

===============

Disect is a very powerful combined disassembler/debugger program,

which has been designed to be easy to use. Its power is achieved at the

expense of memory: the minimum requirement is 1Mb; for larger projects, 2Mb

will be required. The programmer neither encourages nor condones its use to

assist in illegal duplication of copyright software. Disect can be used for

two main purposes: development of software; and the gaining of knowledge

(hacking can be very educational).

The debugger allows an executable file, a disk boot-block, a

binary file, or an area of memory to be loaded and created as a Process. It

is even possible to grab an existing memory-resident Process. A Process's

instructions can be executed individually, or the Process can be activated,

with or without breakpoints installed. System exceptions can be trapped

before they crash the machine, and program symbols can be read from execut-

able files.

The disassembler allows a loaded program to be commented, have

blank lines inserted, and have it's numbers and addresses replaced with

constant or program symbols. All the symbols of Release 3 of the system

include files are available to the disassembler. A program which has been

dissected in this way can then be saved as two source files: a program file,

and a support file. The support file will contain EQUs for all required

constant symbols. System constant symbols can be EQUs (grouped by the SI file

where they live), or alternatively the support file can INCLUDE all required

SI files. Disect data files can be saved, to be reloaded later: 'hold on to

your hacks'!

Disect requires Release 2 of the Amiga operating system; it also

requires the ASL library, and GadTools library.

If you've paid your shareware fee then you'll have the full

version. Otherwise, the copy you've got is (hopefully) only a demo. This

has a few differences: you cannot disassemble to file; you cannot extract

program symbols from executable files; auto trace mode is not available.

In addition, you will not have all of the system symbol data files. If this

is so, and since, as I don't doubt, you'll be impressed by this program, then

don't hesitate in registering today, by sending your name and full address

(and a cheque/postal order for 20) to the address which you'll find at the

end of this file. As well as receiving the full version, you'll also be en-

titled to technical support (by mail) if required. If you like this program,

it's well worth the money... this will encourage further versions, which will

only get even more powerful... imagine!

'Disect' is Shareware, Copyright  D.M.Alderson 1994/5, all

rights reserved. Duplication for the purpose of backups is permitted, but

not for the purpose of selling or otherwise distributing the software,

manual, or other related items, which includes '.dsct' data files only for

commercial purposes, without written permission from the author. This does

not apply to the demo version of the software, which in such instances would

include the manual, and other related items, but not including '.dsct' data

files unless for commercial purposes.

The author does not make any warranty, express or implied, with

respect to the software or any related item, their quality, performance, or

fitness for any purpose. It is the responsibility solely of the purchaser

to determine the suitability of the software for any purpose. In no event

will the author be liable for incidental or consequential damages of any kind

in connection with the software product, manual, or other related items and

processes including, but not limited to, any interruption of service, loss of

business, anticipated profit, or other consequential damages. ETC.

Disect was written using a 2 meg A500+ (without a hard drive!).

* ------------------------------------------------------------------------- *

2) Disect Request Windows

=========================

The program takes much of its input through 'request windows'.

These are standard windows containing string and button gadgets, etc. The

more commonly used windows have keyboard shortcuts. Where available, these

consist of the first letter of the text within the gadget (which will be

underlined). One exception is the 'Select Address Base' request: to select

'+ Decimal' you use key 'p'; to select '- Decimal' you use key 'm'.

Many of the windows allow specification of a number of address

ranges (eg: when searching memory). These contain a list of the currently

defined address ranges, and two string gadgets to input the start and end

addresses (inclusive) of a range. You should input the start, and then the

end, after which, if the range is ok, the address range will be stored. Note

that you may not enter zero as an address. For most of the windows, the

ranges list will be initialised to contain the start and end addresses of

any loaded project memory. If you LMB on any address range, it will be

selected, and can then be changed. If you select a range, then enter no

input for the start and end addresses, the range will be deleted from the

list. The RMB can be pressed to un-select a selected address range.

Instead of having to enter the actual end address into any end

address string gadget, it is possible to instead enter the size of a memory

area, by preceeding it with a comma. You can also enter, for example: '+12',

which will result in an address range of 13 bytes (ie: end address = start

address + 12). For an address range of a single byte, you can simply enter

'='.

One last thing: if Disect seems to lock-up, try pressing any

SHIFT key, or any ALT key.

* ------------------------------------------------------------------------- *

3) The Program Screen

=====================

As of V1.2, Disect incorporates menus for most functions. To

prevent contention between menus, and Disect's use of the RMB, in order

to select from a menu, you must first RMB click on the menu bar, in order

to 'activate' the menus. To disable the menus, and allow Disect to respond

to the RMB in the main window, you simply LMB on the window, away from the

menu bar (for example, on the status/message area).

At the top of the screen is displayed the available chip and

fast memory, various function icons, and the program's status/message output.

The status message consists of the current state of any Process (--- none,

-S- Process suspended, -A- Process active, -W- Process Wait()ing), info of

any defined disassembly window markers, and the number of address stack

entries. The icons (from left to right) are: project; INCBIN; address stack

(3 icons); smiley; workbench screen; disassembly window left/right scroll

arrows. See below for more info.

Below this is the disassembly window area. There can be either

one or two disassembly windows open at once: the tab key will toggle between

single and double window mode. The cursor keys can be used to scroll windows,

while pressing shift also will move a window faster. In double window mode,

pressing the left alt key will select the lower disassembly window. This

applies to the cursor keys as well as other disassembly window keys (eg: A,

F1-F9, F10).

In each disassembly window, you'll find up and down arrow gadgets

for scrolling the window. Normally, these work upon the window in which they

are situated. It is possible, however, to use either pair of arrow gadgets

to scroll the memory window, by also pressing the right ALT key.

Next down is a window which displays the PC and SR values, and

the source effective address (SEA) and destination effective address (DEA) of

the instruction at the PC, when applicable. Also displayed is the content

of these addresses. There are two lines of this: the upper line displays

the current values, while the lower line displays the previous values. This

allows the SEA/DEA content to be seen to be affected after execution of an

instruction, which is not possible with programs which display only the

current values. The upper line can be used to enter new values for the PC

and SR, via LMB on the digits of the PC, or the digits/letters of the SR.

To enter a new SR value, a request window will be opened. This allows input

of either an expression (denary/hex/binary/symbols), or any combination of

the letters XNZVC to denote which flags are to be set: any unspecified flags

will be cleared. If no input is received, then the PC/SR is unaffected. If

the Process is active, then attempting to change the PC or SR will have no

actual effect.

The third line in this window is the 'register zoom'. This can

be locked to any of the registers d0-d7 or a0-a7, and displays the register

content, and the content of this address and the next 15 bytes upward in

memory, as denary/hex (depending on the currently selected base), and as

ascii characters. The '' character is displayed for any bytes not within

the ascii range of 32 to 127. The locked register is selected by LMB on the

text of 'd0', 'd1', etc in the register window. The register zoom can also

be locked to the SEA or DEA of the instruction at the PC, if a Process has

been created, by LMB on the text of 'sea=' or 'dea=' in the first line of the

window.

The register window is the lower-left window on the screen, and

this displays the register values as denary/hex. For data registers, the

register content is displayed also as ascii. For address registers, the

register content is also used as an address; the content of this is displayed

as denary/hex and ascii. Any of the registers can be given new values in the

same way as the PC, by LMB on the digits of the register value. A nice little

feature which you may at times find useful is, when entering for an address

register value, instead of an address, you may enter, for example:

>a text string

This has the effect of storing all the input text after the '>' character,

and setting the address register to point to the stored text. Disect contains

eight such text buffers, each of which may contain up to 63 characters.

The lower-right window is the memory dump. This can be scrolled

using cursor keys and the right alt key. The shift key applies for the up

and down cursor keys as it does for the disassembly window. By LMB on the

address digits, you can input a new address for any of the separate lines in

the window. When you do this, the addresses of any lines below the affected

one will be updated, but any above it will remain unchanged. This allows, for

example, the first line to be locked on one memory location, the second to be

locked on another, and the remaining six lines to be used as a memory dump

starting from yet another address. If you use the cursor keys to move the

window, the first two lines will stay where they are in memory, and only the

continuous block of six lines will be moved. Neat eh? Note that a much bigger

memory dump window can be accessed by pressing 'm'.

* ------------------------------------------------------------------------- *

4) A Disassembler Tutorial

==========================

The best way to understand something is to do it, so on the disk

you'll find an executable file called 'KeepASL' (in the Tutorial drawer).

This is a little utility I wrote ages ago since I was fed up with having to

reload the ASL after each time I assembled something. All it does is to open

the ASL library and the clipboard device, preventing them from being removed

from memory. So, load up Disect, and LMB on the project icon. This will open

the project request from which you should select 'Executable', and select the

file via the ASL.

The next request allows you to define whether or not to create a

Process, and extract any symbols from the executable file. Leave these un-

checked (you don't need to create a Process for the disassembler, and there

are no symbols in 'KeepASL'). The string gadget allows you to enter any CLI

arguments to be passed to the executable file (again, not required now). So,

just LMB on 'OK'.

The executable will be loaded, and the disassembly window moved

to it's start address. Note that marker 9 will be defined at the address -

marker 9 is always defined at the start address of any loaded executable/

binary/boot, etc. When you load a binary file, marker 8 will be defined at

its end address, so if you were to save the binary file later, you could

enter 'm9' for the save binary start address, and 'm8' for the end address.

Next, if you haven't already done so, load the SysSymbol data for

the following directories: DOS, EXEC, INTUITION. This is done by pressing the

'Help' key to access the 'Resident SysSymbol Data Preferences', selecting the

required data, then 'Use'. This request also allows you to load and save the

preferences, or to quit, which will restore the preferences to the state they

were when you pressed 'Help'.

In the disassembly window, you'll have something like this:

0000665256		move.l	4.w,665604

0000665264		bsr.w	665504

0000665268		lea.l	665766(pc),a1

0000665272		moveq.l	#0,d0

0000665274		movea.l	665604(pc),a6

0000665278		jsr	-552(a6)

0000665282		move.l	d0,665608

0000665288		beq.w	665406

0000665292		lea.l	665784(pc),a1

0000665296		moveq.l	#0,d0

0000665298		movea.l	665604(pc),a6

0000665302		jsr	-552(a6)

0000665306		move.l	d0,665612

Obviously, the addresses will be different but the methods are the same.

First of all, LMB on the area between the address and the mnem-

onic of the first line, and enter the text 'main'. From the request window,

select 'Instruction'. This will define a program symbol for the first address

of the program, and will log the address as an instruction. Incidentally, if

you ever enter a program symbol name which does not fit into the available

area, the end of the name will not be lost. On the screen, only the first 11

characters will be visible; when you disassemble to a file, the whole of the

name will be output.

Now, LMB on the destination effective address of the first line.

This will open the symbol selection window (see below for specific info).

Activate the string gadget which presently contains the LMB'd address, and

enter the text 'sys_base'. From the request windows which open, you should

define the address type as 'Variable', the address size as 'Long', and the

address base as '+ Decimal'. What this does is to log the DEA of the first

instruction as a long variable, the content of which will be displayed as

positive (unsigned) denary. Just to make sure, RMB on the DEA of the first

line, and you'll see:

0000665604	sys_base	DC.L	0

The previous disassembly window address will have been stacked (look at the

status message), so you can return there either by LMB on the third icon

from the left at the top of the screen, or by pressing F10. Do this now!

You'll notice that as well as having replaced the DEA of the

first line with the program symbol 'sys_base', Disect will also have replaced

all other references to this address. This feature can be disabled by press-

ing 'Del', and unchecking 'AutoSymbol', BUT DON'T DO THIS NOW!

Now, to make the source more readable, we'll add a blank line

after each of the following lines in the disassembly window: the first, the

second, and the eighth line. Do this by LMB on the mnemonic of each of these

lines. This will open the 'Address Functions' request, from which you should

select 'Add Blank Line' (at lower-left of window). For the second and eighth

lines, you'll get the 'Address Type' request, since these addresses have not

yet been logged - obviously, these should be logged as instruction addresses.

This will result in the following:

0000665256	main	move.l	4.w,sys_base

0000665264		bsr.w	665504

0000665268		lea.l	665766(pc),a1

0000665272		moveq.l	#0,d0

0000665274		movea.l	sys_base(pc),a6

0000665278		jsr	-552(a6)

0000665282		move.l	d0,665608

0000665288		beq.w	665406

0000665292		lea.l	665784(pc),a1

0000665296		moveq.l	#0,d0

Next, LMB on the text '-552' (or -$228), then on the text 'exec'

in the lower window. You'll probably have guessed already, but LMB on the

text '_LVOOpenLibrary' in the upper window, then select 'Confirm', and define

the address as 'Instruction'. Now, we all know that a call to exec.Open-

Library() requires a1 pointing to the library name text, so RMB on the SEA of

the third line. What you now see won't look like much, until you've done the

following: define a program symbol at the first line in the disassembly

window, 'name' will do, and define the address as Data/Byte/ASCII/NULL. Now

we know what library it is, we can change the program symbol from 'name' to

'int_name':

0000665766	int_name	DC.B	"intuition.library",0

The last request window ('Select Data Area Size') allows you to

define how many bytes there are in a data area in four ways: you can enter

an expression to define the number of BYTES; you can simply press 'Enter' to

make it consist of one byte, one word, or one long (depending on the address

size already defined); you can select 'NULL' to make the data area end at the

first NULL byte/word/long; or you can select 'Symbol' to make it end at the

byte before the address of the next defined program symbol.

If you now press F10, you'll see that the address reference will

have been replaced with 'int_name'.

Next, press the 'A' key, and enter the following: '[0:$58]'. This

will move the disassembly window to the $58th byte of the first hunk. Here,

you will see a 'jsr' as the third line. LMB on the text '-204' or '-$cc',

and replace it with '_LVOOpenWindow'. So, the first line contains the address

of a NewWindow structure, so RMB on this address to move the disassembly

window to this data. Now press key 'X', (or select 'Define DataString' from

the 'Functions' menu), and from the list of system DataStrings, LMB on the

text 'NewWindow', then select the 'Define' gadget. The NewWindow structure

data area will be logged, and Disect will inform you that it has found an

address pointer within the NewWindow structure, which points to an ASCII

string. Click on the 'Define' gadget, and the ASCII string will also be

defined as a data area. The disassembly window will now be returned to the

address of the NewWindow structure. For further information about DataStrings

I STRONGLY recommend that you read section 7.

Well, we could go on like this for all of the 560 bytes of the

program, but fortunately, you don't have to. Instead, LMB on the project icon

and select 'Load Data', 'Erase', then load the file 'Tutorial/KeepASL.dsct':

; * -------------------------------------------------------------

; Disassembly of 'KeepASL' - DMA 1.10.94

0000665256	main	move.l	(EXEC_BASE).w,sys_base

0000665264		bsr.w	start_up		; WB/CLI start up

0000665268		lea.l	int_name(pc),a1	; open intuition

0000665272		moveq.l	#0,d0

0000665274		movea.l	sys_base(pc),a6

0000665278		jsr	_LVOOpenLibrary(a6)

Much better than the original, eh!

Now, we're going to disassemble the whole program to source, so

press 'd'. This gives the 'Disassemble To File' request. The disassembly

address range will be initialised as the address range of the loaded program.

Note that with executable files, there may be more than one hunk in the file,

so it is possible to disassemble more than one address range. Now, using the

two string gadgets at the bottom of the window, enter the two path/file names

as 'RAM:Program.s' and 'RAM:Support.s', then select either to EQU or INCLUDE

system constant symbols (I suggest EQU), and select 'OK'.

After a short time a request window will open, informing you that

an immediate long has been found. This means that Disect has found a number,

as immediate data, which is within the project memory range. It is possible

for such numbers to be immediate data, or program addresses, but it should

be fairly obvious for you to decide which from the context in which they are

used. For such numbers, Disect will display any matched program symbol name,

or alternatively, the previous program symbol to the address along with the

required offset to match the value of the number. From this request, you have

a choice of leaving the number as immediate data, replacing the number with

the symbol (or symbol+offset), or quitting the disassembly process.

In our case, the number should be replaced (it's actually the

nw_Title pointer of the NewWindow structure). Ok, when it's done, load up

the two source files (you'll actually find them in the Tutorial drawer,

although I suggest you DO have a go at all this). Have a look at them...

Nice, aren't they! AND, they'll assemble correctly.

* ------------------------------------------------------------------------- *

5) Symbol Selection Window

==========================

A major feature of Disect is its ability to replace numbers

present within a program with symbols. Disect can distinguish between two

different types of symbols:

PROGRAM	A program symbol refers to a specific address within the program.

This may be either the destination of a branch, etc, or an access

to a variable, or the address of data.

There are two kinds of program symbol, system and user. System

program symbols, or system address symbols, are used to refer to

absolute addresses in memory, for example, the addresses of chip

registers. These form part of Disect's predefined system symbol

list. User program symbols are defined by the user, and are used

to refer to addresses within the program being disassembled.

CONSTANT	A constant symbol refers to any number used as immediate data, or

as a displacement for the following addressing modes:

ARID	dd(ax)		ARIDI	dd(ax,rx.x)

PCD	dd(pc)		PCDI	dd(pc,rx.x)

Disect's predefined system symbol list includes all of the system

constants of Release 3.0 of the system include files. These are

divided into three classifications:

LVO	a library vector offset used with: jsr dd(ax)

Structure	a system structure offset used with: dd(ax)

Constant	all other system symbols

User constant symbols are defined by the user.

A valid symbol name must begin with either a letter, or the

characters '_' or '.', and may continue with any of these, as well as digits.

Disect supports the use of local program symbols, which must be prefixed

with either a '_' or '.' character, depending upon the current preference

setting. There are a certain number of reserved symbol names which cannot

be used when defining a symbol. The list of these can be found in Section 8.

In order to replace a number within a program, all you have to

do is LMB on the number in the disassembly window. This will open the Symbol

Selection Window, which is used to specify which symbol(s) are to be used to

replace the number. In addition to using real symbols, Disect also allows

'fake' symbols to be created. These are symbols which have no name text.

Instead, the 'name' is the value of the symbol displayed in whatever base you

select. For example, instead of replacing the number 16 with the real symbol

'MP_SIGTASK', you could replace it with the fake symbol '%10000'. Or, the

number 65 may be replaced with a fake symbol: "A". The possible bases for

fake symbols are decimal, hex, ASCII, binary. Such symbols will appear in

the symbol list where appropriate.

Whenever you replace a number, the address where the number

occurs becomes logged in the usual way. In addition, if you are replacing

an address with a (new, program) symbol, then this address is also logged.

Consider the following:

632472		move.l	632804,d0

~	~	~

632804		ori.b	#0,d0

If you LMB on the '632804' in the first line, and replace it with a symbol,

eg: 'variable', then the addresses of both lines will be logged. First,

Disect will allow you to define the address type for the equivalent address

of '632804' (although INCBIN is not supported in this way, at present). In

this example, it might be address type: VARIABLE/LONG/... This will log the

second address, and assign to it a program symbol (named 'variable'). After

this, the Symbol Selection window will close, and Disect will then request

the characteristics of the first address (INSTRUCTION).

The Symbol Selection window contains two list windows. The left

window will display the current list of matched symbols. You may select a

symbol from the list by LMB on it within this window. The right window will

display either a list of system include directories, or a list of the files

within a selected directory. You can LMB on a directory or file name in order

to limit the symbol search to only the selected directory or file. If you RMB

on a system constant symbol (in the other list), then this will select the

system include directory and file which contains that symbol. Note that if

you RMB anywhere else (not in the symbol list window), then the current list

of symbols will be sorted alphabetically.

The currently selected system include path is displayed above the

includes list window; to the right of this text is a 'Parent' gadget to un-

select the current directory and file names.

In the symbol list window, if a symbol has a '*' character disp-

layed after its name, then this denotes that the symbol is not defined in

the official system includes. There are quite a few of these to allow greater

flexibility. They include 'FLAGB_' and 'FLAGF_' definitions of 680x0 process-

or flags (in exec/execbase.i), and a complete list of raw key codes (devices/

keyboard.i), as well as some hardware bits symbols. When disassembly to file,

any such symbols will be output to the support file as EQUs.

Above the symbol list window, are four gadgets. The first two

gadgets allow selection of the type of symbol to be searched for. The first

gadget selects between System or User symbols; the second, between Program or

Constant symbols.

The third gadget allows you to select the type of search to be

performed; there are two possible types of symbol search: Value, and Bit.

The first will search all resident symbols for a direct value match with the

number to be replaced. A Bit search will list all symbols for which a set

bit in the original number corresponds to a set bit in the symbol's value.

This is used when replacing a number which represents bitwise OR of multiple

bit flags. As examples:

move.l	#IDCMP_RAWKEY+IDCMP_CLOSEWINDOW,(a0)

move.w	#DMAF_MASTER+DMAF_COPPER,dmacon(a5)

When searching for symbols in this way, when you have selected the first

symbol to be used, Disect will then only list symbols which represent bits

in the same group of flags (eg: only IDCMP_ flags, or only DMAF_ flags).

The fourth gadget depends on the current symbol type: for program

symbols, a 'Prev' gadget will exist; for constant symbols, an Offset cycle

gadget will exist.

The 'Prev' gadget will automatically select the system/user

program symbol with a value previous to that of the number being replace

(which would be an address). This is useful for finding, for example, a

reference to the low byte of a word variable (assuming you have already

defined a program symbol for the address of the word), or a refernce into

a data table. It becomes especially useful if replacing a system address:

for any direct address reference to a hardware register, pressing this

gadget will result in the symbol 'CUSTOM_BASE' being automatically selected

(ie: address $DFF000). The apprioriate symbol from the hardware directory

(custom.i) can then be selected to account for the required offset to the

actual address. When you attempt to find a previous (user) program symbol,

Disect will only use a local program symbol if this does not result in a

reference to a local program symbol across the bounds of a global program

symbol.

When searching for system/user constant symbols, the Offset cycle

gadget will be displayed. This allows you to select an offset value to be

used when searching for a symbol value match. The possible offsets are:

-1, -2, -3, +1, +8, +16, +24. Such an offset represents the value to be added

to the original search value in order to find the value to which the symbol

search must match symbols. For example:

btst.b	#INTB_AUD3,intreqr(a5)

Now, INTB_AUD3 has the value 10, ie: bit 10 of the word-register. When this

is assembled (some assemblers would allow the above, some would require you

to have '-8' after the 'INTB_AUD3'), the resulting code would actually be:

btst.b	#2,$1E(a5)

So, when you LMB on the '2' in the disassembly window, obviously Disect would

search for symbols of value 2. If you've already replaced the '$1E' with

'intreqr', then you'll be expecting an 'INTB_' symbol. This would return

'INTB_SOFT'. Incorrect. Hence, an offset of '+8' will solve the problem, and

produce the relevant result (by searching for symbols of value 2+8, hence

'INTB_AUD3' would be found!). The actual result would, of course, be:

btst.b	#INTB_AUD3-8,intreqr(a5)

Above the four symbol gadgets is a string gadget which is used

for displaying the current search value, or the current list of selected

symbols

Above the four symbol gadgets is a string gadget. This has a dual

purpose of displaying the currently selected list of symbols which will be

used to replace the number, and to allow a symbol name to be input. When

searching for user program or constant symbols, you may enter any valid

unused symbol name here, and this will be created as a new user symbol which

will be assigned the value of the replace-number.

If you enter the name of an existing system/user symbol, then

this will have the effect of selecting the symbol. In this case, the symbol

search value (ie: originally the number which was being replaced), will be

adjusted to take account of the value of the symbol which you entered. This

is useful since there are times when a number being replaced is formed from,

for example, an accumulation of structure offsets. As an example, consider a

program which allocated it's variables memory dynamically, then accessed them

using ARID mode via a5. Supposing this memory block was 2k in size, and you

have determined that bytes 240 to 273 are used for an MP structure (message

port, exec/ports.i), so you have defined a user constant symbol 'mport' to

have the value 240. Now, you might find the following:

move.b	255(a5),d0

What this represents is in fact:

move.b	mport+MP_SIGBIT(a5),d0

This can be achieved by entering the name 'mport' (which will be selected,

and the search value of 255 adjusted to 15), then selecting the 'MP_SIGBIT'

symbol by LMB in the symbol list window.

To the right of the string gadget are three check gadgets which

are used to select the type of symbols to be searched for when searching for

system constant symbols. These gadgets will automatically define themselves

according to the context where the replace-number was used. Note that this

cannot be 100% accurate for the type of symbols: always make sure you are

searching for what you want!

Whenever a symbol is selected which results in the search value

becoming zero, then a request window will open. The content of this varies.

If you are making a normal symbol selection (due to LMB in the disassembly

window), then four gadgets will exist: 'New' 'Zero' 'Confirm' and 'Quit'.

When making a dummy symbol selection (due to pressing the 'P' key in the main

window), then a 'Comment' gadget will exist instead of 'Confirm'. If you are

replacing a number within a bitfield instruction (ie: either the offset or

width value), then you get 'New' 'Confirm' and 'Quit'.

The 'New' gadget will erase the currently selected symbol(s) and

allow a new seclection to be made. 'Zero' will allow you to select further

symbols (of value 0). 'Confirm' will cause the original number in the disass-

embly window to be replaced with the currently selected symbol(s). 'Quit'

will abort, and close the Symbol Selection window. The 'Comment' gadget has

a rather useful purpose (see key 'P' in Section 12).

* ------------------------------------------------------------------------- *

6) Address Functions

====================

The Address Functions request is accessed via LMB on the opcode

(mnemonic) area in the disassembly window. The address functions are as

follows:

Log Address

-----------

This allows a program address to be logged. You will be requested

the type of the address. This may be instruction, variable, data, copper,

INCBIN, ASCII string (NULL-terminated), or EVEN string (which is the same,

except Disect will ensure that the string ends at an odd address, so that

the next address is therefore even). Note that any extra pad byte used to

word-align the string must be NULL. If a Process exists, then when an address

is logged (by any method), if the PC is at the address, then it will be auto-

matically defined as an instruction address.

INSTRUCTION	This is the default address type for all addresses at which the

opcode can be successfully identified as a valid instruction.

VARIABLE	This allows an address to be defined as a byte, word, or long

variable. The base in which the variable's value should be

displayed can also be defined.

DATA	Similar to a variable address, a data address consists of any

number of bytes, words, or longs. The size of the data area can

be specified by either: pressing Enter (for 1 byte/word/long);

inputting an expression to determine the number of bytes of data;

selecting the NULL gadget (data continues up to and including the

first NULL byte/word/long); or selecting Symbol gadget (data area

continues until next defined program symbol). The last option

will allow a data area to automatically be adjusted if a new

program symbol is defined after the data area start, and before

what was previously the 'next program symbol'. (Note that this

cannot be used if the next logged address from the data area

start does not have a program symbol, or if there is no next

program symbol).

Incidentally, there is another way of defining data areas. See

section 7.

COPPER	A copper address will be disassembled as a macro which, when

assembled, will create the relevant copper instruction. These

macros will be output to the support source file during file

disassembly (if required). Note that for copper move instruction,

if the hardware SysSymbol data is resident then the move destin-

ation address will automatically be replaced with the relevant

symbol for the hardware register it refers to.

INCBIN	This allows data areas to be defined as INCBIN areas. Such areas

will be disassembled as an INCBIN directive. Upon selection of

this, Disect will request the INCBIN data area size, which must

be entered (no 'next-symbol', or until-NULL'). After this, the

ASL is used to select the file to which the data area should

be written. See also the section about the INCBIN request window.

AS PREVIOUS	This will automatically define an address of the same type, etc,

as the previously defined address. Be careful not to use this

after, for example, defining an address via the Symbol Selection

Window, since this would be taken as the previous address type.

Amend Address

-------------

This allows a previously-logged address to be amended.

Erase Address

-------------

This will remove anything that has been attached to the address.

Add/Erase Section Header

------------------------

A section header is a line of '-' characters, which will be added

before the address.

Add/Erase Header Comment

------------------------

These options allow a full-line comment to be added before the

address, or to be removed from the address.

Add/Erase Line Comment

----------------------

These allow an end-of-line comment to be added to the address, or

removed from the address.

Add/Erase Blank Line

--------------------

These will add/remove a blank line after the address.

Erase Program Symbol

--------------------

This allows a program symbol to be removed from the address.

Erase SEA Symbols

-----------------

This option will remove any symbol(s) which have been used to

replace the instruction's source effective address. This includes the offset

and width values for 68020 bit fields.

Erase DEA Symbols

-----------------

This option will remove any symbol(s) which have been used to

replace the instruction's destination effective address. This includes the

offset and width values for 68020 bit fields.

* ------------------------------------------------------------------------- *

7) DataStrings

==============

In order to log a data area which consists of a number of byte/

word/long elements, ie: all bytes, all words, OR all longs, you use the

address functions request. This allows you to specify the total size of the

data area, and Disect will log only the first address. Obviously, this can

save a lot of memory for large data areas, since Disect does not have to

store information for each address. This is the best way of logging uniform

data areas.

For any data area where its elements are not all the same size

(byte/word/long), you can use a DataString. The DataString request, accessed

via key 'X', allows you to define your own DataString, or select a system

DataString, and use it to define a data area.

A DataString consists of two strings: the format string, and the

base string. The first specifies the size of each element of a data area;

'B' represents a byte, 'W' a word, and 'L' a long. The base string defines

the base of a 'piece' of data: 'P' positive denary; 'N' negative denary;

'H' hex; 'B' binary; 'A' ASCII; 'X' means use the currently selected default

base (either hex or denary, as defined via the preferences). When you use

a DataString to define a data area, successive addresses are logged as data,

the size and base of which corresponds to the relevant characters from the

format and base strings.

Disect contains a number of predefined (system) DataStrings. The

names of these correspond to the names of system structures, eg: Gadget, New-

Window, etc. Each DataString represents the byte/word/long elements of the

relevant system structure. You can also define your own DataStrings, which

will be saved/loaded as part of the Disect data file. In the DataString

request window, after pressing Enter in the base string gadget, if your

DataString is valid, then it will be automatically stored. The LMB can be

used to select a DataString from the currently displayed list.

As of V1.6, when you use a system DataString to define a data

area, a preference exists to allow Disect to automatically add a line comment

to each 'DC.X' directive. These comments will be the names of the fields of

the system structure which the DataString represents.

When you later LMB on the operand of such a 'DC.X' directive (in

order to replace it with a symbol) then Disect actually uses the line comment

text in order to define which system include file is to be searched within

the Symbol Select window. If the required directory's SysSymbol data is not

resident, Disect will attempt to load it. As well as automatically selecting

which include file, Disect also defines which 'group' of bit symbols should

be listed, if necessary. For example, if you LMB on the number of:

DC.W	$44040		; nw_IDCMPFlags

then the Symbol Select window will open in 'intuition/intuition.i', and only

'IDCMP_' flag symbols will be listed! If required, once such a number has

been replaced with a symbol, Disect will automatically erase the line comment

text since it is no longer required. This is a preference, and can therefore

be disabled if necessary. In addition, an item exists in the 'Functions'

menu which will erase ALL existing SDS (system DataString) line comments.

When defining an data area using a system DataString, Disect can

detect pointers to other system structures. For example, within a NewWindow

structure, nw_FirstGadget will be detected as pointing to a Gadget structure.

In this case, Disect will move the disassembly window to the address of the

Gadget structure, and a request window will open which allows you to confirm

to Disect that you wish the data area to be defined, or alternatively, you

can cancel the definition.

The window also contains a string gadget, which will contain

either the text of any existing program symbol at the start of the data area,

or a symbol name which Disect will create automatically. Whichever, this is

the symbol name which will be defined for the first address of the data area.

It is not possible to enter any other program symbol name. For example, for a

Gadget structure, Disect may create a program symbol of 'gadget0'. The number

at the end of the symbol name is defined to be unique for each data area

definition. It is recommended that you use these automatic symbol names

(see below).

Whenever a data area is defined in this way (due to a pointer to

it within a previously-defined data area), then the program symbol of it is

used to replace the number/address within the existing data area. Example:

DC.L	gadget0		; nw_FirstGadget

~	   ~

~	   ~

~	   ~

gadget0	DC.L	$13ef26		; gg_NextGadget

Two of the system structures which are supported by DataStrings

exist as an array of structures. These are TagItem and NewMenu structures.

When you define a data area for either of these structures, Disect will also

define any following structures, until the end of the array. For example,

a TagItem array ends when ti_Tag holds a value of TAG_END.

One last point to mention is that when a data area is defined via

a system DataString (directly from the request window, as opposed to via a

pointer to it in a previously-defined data area), if the first address of the

area does not have a program symbol, then Disect will automatically create

one for the address, as described above. The thing to be aware of is that

when disassembling to a source file, Disect has the ability to detect such

program symbols, and if it does so, it will disassemble the data area using

a single macro (instead of a sequence of DC.X directives). For example:

gadget1	DC.L	gadget2

DC.W	326,4,50,14

DC.W	GFLG_GADGHCOMP,GACT_RELVERIFY,GTYP_BOOLGADGET

DC.L	border12,0,intuitxt7,0,0

DC.W	2

DC.L	0

The above would automatically be converted to:

gadget1	GADGET	gadget2,326,4,50,14,GFLG_GADGHCOMP,GACT_RELVERIFY,

&		GTYP_BOOLGADGET,border12,0,intuitxt7,0,0,2,0

This will not occur: if any address in the data area has a line end comment;

if any address in the data area other than the first address has either a

header comment, or a program symbol; or if you have not used Disect's auto-

matic symbol names. The syntax for continuing macro parameters over more than

one line follows that required by Devpac. Any required macro definitions

will be output to the Support source file.

As a result of this feature, any program symbol name which might

be created by Disect is now reserved; this means that you cannot use these

names for any other program symbols you might create. The complete list of

reserved symbol names (and macro names) can be found in Section 9.

The DataString request window contains the following:

Data Start Address

------------------

A string gadget allows the data start address to be specified.

This will be initialised, when the window opens, to be the start address of

the (upper) disassembly window.

DataString Name

---------------

A string gadget is used to enter the name of a user DataString.

If you enter the name of an existing DataString, it will be selected.

Data Format String

------------------

This string gadget allows you to enter up to 512 characters to

specify the format of the data area. Each data element may be specified to

be byte, word, or long, by entering the characters 'B', 'W', or 'L'. Note

that if you enter, for example, 'LLBBBW', then Disect will automatically

insert an extra 'B' before the 'W' to ensure word-alignment. Disect will

permit an odd number of 'B' if it is at the end of the string.

Data Base String

----------------

This string is used to enter an equal number of characters to

specify the base of each data element. Each may be any of: 'P' positive den-

ary; 'N' negative denary; 'H' hex; 'B' binary; or 'A' ASCII.

Associated with each string are two numbers. The first displays

the number of characters for each string. The second displays the number of

successive, same, characters at the end of each string.

System/User Cycle

-----------------

This will toggle between a list of system and user DataStrings.

Delete

------

This will delete the currently-selected user DataString.

Define

------

This will close the window, and cause Disect to attempt to define

the data area using the selected DataString. If an error occurs, the disass-

embly window will be moved to the address which Disect was attempting to log

when the error ocurred. Where possible, Disect will preserve any existing

program symbols and comment texts within the definition data area. Any blank

lines will be removed, but one will be added at the end of the data area.

Quit

----

This will close the window, without defining the data area.

* ------------------------------------------------------------------------- *

8) Key Macros

=============

The 'Macros' menu allows up to ten key macros to be defined and

used. This allows sequences of key presses, LMB or RMB clicks, and gadget

selections to be recorded, and played back automatically by pressing a

single key. As an example of just how useful this can be, supposing you

were dissecting a program which contained a data table of, say, a hundred

entries, each one consisting of the following sort of thing:

DC.B	"some text string",0,0

DC.L	program_function67

Before this has been logged as data, it might be disassembled as, for

example:

$42be8	DC.W	$736f

$42bea	blt.b	$42c51

$42bec	DC.W	$2074

$42bec	bcs.b	$42c68

~	   ~

~	   ~ etc

To define a key macro to format a single entry, first of all you would make

sure that the disassembly window is at the start address of the first entry.

You then begin the macro definition by selecting the menu item 'Macros/Begin

Definition/Macro 1'. First, by LMB on the first 'DC.W', you would define the

address as an 'EVEN String'. This gives:

$42be8	DC.B	"some text string",$0,$0

$42bfa	ori.b	#$0,-(a0)

Next, LMB on the 'ori.b', then select 'Add Blank Line', and define this

address as a single 'Data/Long/Hex' long. Finally, by clicking on the

disassambly window down arrow twice, then pressing 'Enter', the macro will

be defined. By moving the disassembly window down two lines, you will ensure

that each time you use the macro, the relevant addresses will be formatted,

since the macro has been defined to operate upon the first two lines in the

disassembly window. All that is now required is to press the '1' key to use

the macro to define successive entries in the data table, until the end of

table is reached.

But 'ah!', I hear you say: how will I know when I have reached

the end of the table? Well, one possible solution is to begin definition of

macro 2, and to first of all press key '1' (which would cause the next entry

in the data table to be formatted). Then, you could hold down the right ALT

key whilst you LMB click once on the address digits at the left hand side of

the first line in the disassembly window. This will move the memory window

to this address, at which point you would press 'Enter' to complete the macro

definition. All of this would give you a second macro which enables you to

see the content of the next address which would be affected by the macro,

before you actually use it on this address. Note that it is perfectly safe

to redefine macro number 1: macro number 2 does not depend on macro number 1,

even though macro number 1 was played during definition of macro number 2.

During definition or playback of any macro, if any errors occur,

then the macro definition/playback will be aborted. This would result in a

macro NOT being defined. This also occurs if any Process exception occurs

other than a trace or a breakpoint exception.

A defined macro can be replayed either by pressing it's key

(1-0), or using the menu item. Via the menu item, a request window will

open which allows you to specify either to play the macro once, to play the

macro a specific number of times, or to play the macro until an expression

is TRUE. This is probably most useful for something such as the following:

'w1=$662084', which would play the macro until the (upper) disassembly

window reached a certain address.

Macro playback can be aborted by clicking on the main program

window, and holding down the space bar.

* ------------------------------------------------------------------------- *

9) Expressions

==============

Disect allows complex expressions to be input whenever a numeric

value is required. Such expressions may contain program or constant symbols,

which may be system or user symbols, although local program symbols are not

permitted. Numbers may be expressed as denary, hex, binary, or character

constants (eg: 'a', "ab", 'abc', "abcd"). In addition, addresses can be

represented as offsets from hunk/project memory starts. For example, the

text "[0:16]" represents the address of the 16th byte of hunk 0 (the first

hunk). If there is only one project memory area (eg: an executable of only

1 hunk, or a binary file, has been loaded), then a hunk number does not have

to be specified ("[16]" would be accepted). Any numbers within the "[]"

characters may be either hex or decimal. If a base offset address has been

defined, then if you were to input "[16]", then this would be taken as an

offset from the base offset address (instead of as an offset from the start

of the first/only hunk).

It is also possible to prefix a number with a '.', for example:

'.$80000046'. This will cause Disect to treat the number as an FFP value,

which in this case would be evaluated to a value of 32. If the value is too

large/small to be stored as a long integer, then an error will result.

Disect supports the following operators, in decreasing order of

precedence:

HIGH PREC	-		MINUS, eg: -32

~		bitwise NOT, eg: ~$0FF0

<< >>		SHIFT LEFT and RIGHT, eg: %1<<16

& ! ^		bitwise AND, OR, EOR

* / \		MULTIPY, DIVIDE, MODULO

+ -		ADD, SUBTRACT

= < > <> <= >=	comparisons

Precedence may be overridden via parentheses ( and ). Expressions

are evaluated left-to-right. Byte, word, and long indirection is supported

via parentheses:

{4}.l		long content of address 4

The following is also possible:

{{4}.l}.w		word content of address specified by

long content of address 4

The comparison operators return the value 0 if FALSE, and -1 (or

$FFFFFFFF) if TRUE. Expression evalution occurs using 32-bit unsigned integer

arithmetic. The operator | is equivalent to ! (bitwise OR); != may be used

for <> (inequality).

There also exists a number of reserved symbol names. Unlike all

other symbols, they are case-insensitive:

M1-M9	disassembly window markers 1-9

W1	start address of upper/single disassembly window

W2	start address of lower disassembly window

W3	start address of memory window

If register a6 is equal to any of these, then the relevant name will be

displayed in the register window instead of the actual address:

BASEEXEC	Exec library base address

BASEDOS	DOS library base address

BASEGFX	Graphics library base address

BASEINT	Intuition library base address

BASEASL	ASL library base address

BASEGADT	GadTools library base address

BASEMFFP	MathFFP library base address

The following are only valid if a Process exists:

D0-D7	current value of data registers

A0-A7	current value of address registers

PC	current value of program counter

SR	current value of status register

CCR	current value of condition codes register

SP	current value of a7

SEA	source effective address of instruction at PC

DEA	destination effective address of instruction at PC

INITSP	value of a7 when Process initially entered

PROCP	address of Process's Task/Process structure

In addition, Disect will not allow symbol names such as 'A235768', or

'a23ff40', since such names may be automatically created during disassembly

to file, and hence should not exist already.

The following lists all the symbol names reserved for creation

of automatic program symbols for system structure data areas which have been

defined via DataStrings. Each symbol name is the base name to which an ascii

number is added. All possibilities (actually, an infinite number) of each

base name text are reserved.

System Structure	Base Symbol		Macro Name

----------------	-----------		----------

BB		bblock		BOOTBLOCK

BitMap		bitmap		BITMAP

BoolInfo		boolinfo		BOOLINFO

Border		border		BORDER

ColorSpec		colrspec		COLORSPEC

EasyStruct		easystru		EASYSTRUCT

ExtGadget		extgadg		EXTGADGET

ExtNewScreen	extnscrn		EXTNSCREEN

ExtNewWindow	extnwind		EXTNWINDOW

Gadget		gadget		GADGET

Hook		hook		HOOK

Image		image		IMAGE

IntuiText		intuitxt		INTUITEXT

KeyMap		keymap		KEYMAP

LH		listhead		LISTHEAD

LIB		lib		LIBRARY

LN		listnode		LISTNODE

MC		memchunk		MEMCHUNK

ME		mementry		MEMENTRY

MH		memheadr		MEMHEADER

ML		memlist		MEMLIST

MLH		minlisth		MINLISTHEAD

MLN		minlistn		MINLISTNODE

MN		message		MESSAGE

MP		messport		MESSAGEPORT

Menu		menu		MENU

MenuItem		menuitem		MENUITEM

NewGadget		newgadg		NEWGADGET

NewMenu		newmenu		NEWMENU

NewScreen		newscrn		NEWSCREEN

NewWindow		newwind		NEWWINDOW

PropInfo		propinfo		PROPINFO

StringExtend	strextnd		STREXTEND

StringInfo		strinfo		STRINFO

TTextAttr		ttxtattr		TAGTEXTATTR

TagItem		tagitem		TAGITEM

TextAttr		textattr		TEXTATTR

TextFont		textfont		TEXTFONT

TextFontExtension	tfontext		TEXTFONTEXT

* ------------------------------------------------------------------------- *

10) MultiBin Files

==================

The 'Save MultiBin' item in the 'Project' menu allows a single

binary file to be created which consists of more than one block of memory.

The item will open a request window which contains a list of entered address

ranges, string gadgets for entering address ranges, and a couple of gadgets

at the bottom of the window.

The 'SegList' gadget allows a memory-resident SegList to be

used to automatically define the address ranges to be saved. By entering

the start address of the first hunk in memory into the string gadget next

to the 'SegList' gadget, and then clicking on the 'SegList' gadget, Disect

will scan the SegList, and store the start and end addresses of each hunk

in the SegList. Be careful! The address to be entered as the start address

of the first hunk should be the address at which the PC begins when the

SegList is executed (the SegList is immediately before this address in

memory).

There is also a check gadget which will force the file which is

saved to be pure binary. In general, if you have used the 'SegList' gadget,

then this should be unchecked, since extra information is saved in the file

which enables the MultiBin file ('.mbin') to be reloaded later to the same

addresses from which each block of memory was saved, and possibly created as

a Process. This is of most use when dissecting an executable which allocates

memory, into which it decompresses the real program as a complete SegList.

By specifying the initial execution address of the first decompressed hunk,

the complete program can be saved as a MultiBin file. For those who are

interested, a description of the MultBin file format is at the end of this

section.

If the gadget is checked, then only the exact content of the

specified memory ranges is saved, as a binary file ('.bin'). This means that

the saved memory cannot be loaded back later to the exact addresses from

which each block was saved.

The 'Load MultiBin' item in the Project menu allows a previously

saved MultiBin file to be loaded. This presents a request window which has

an 'Allocate Saved Memory' check gadget. If checked, then Disect will attempt

to load the MultiBin file to the exact addresses from which it was saved.

Alternatively, if unchecked, the file will be loaded to any available memory.

A separate executable program called 'AllocMultiBin' allows the

required memory of a MultiBin file to be allocated. It should be used immed-

iately after re-booting your machine. It will open an ASL requester with

which you specify the MultiBin file to be loaded. If an error occurs, the

program will re-open this requester; if the file loads successfully, the

program will merely wait until you click on it's window's close gadget, and

will then free the memory used by the MultiBin file.

Once the MultiBin file has been loaded, you should leave 'AMB'

in memory, and load Disect. Then select 'Load MultiBin' from the 'Project'

menu, select the same MultiBin file as was loaded by 'AMB', and check the

'Allocate Saved Memory' gadget in the relevant request window. From this

window, click on the 'OK' gadget, and 'AMB' will automatically free the used

memory, then terminate, allowing Disect to load the MultiBin file to the

required memory.

Also in the Save Multibin request window is an 'Add' gadget,

which will attempt to add a selected address range to the current project,

if the memory is available.

MultiBin File Format:

---------------------

For each memory block:

LONG	start address of memory

LONG	end address of memory

LONG	memory type (MEMF_ returned by Exec.TypeOfMem()

followed by the memory block itself

File ends with 3 LONGs, all of value zero.

* ------------------------------------------------------------------------- *

11) Screen Icons Reference

=========================

Here is information about the icons at the top of the main

program screen, from left to right across it.

Project Icon

------------

The first icon is used to access the Project Request. This allows

the following operations:

1) Executable will load an executable file. After selecting the

file to be loaded, a request window will allow you to specify

whether or not Disect should extract any program symbols, and

create a Process. The window also contains a string gadget with

which to enter any CLI arguments which are to be passed to the

Process. If program symbols are extracted from an executable

file, then Disect will automatically log their addresses as

instructions, since it is not possible to determine between

code/data/etc. All this means is that some addresses will have

to be amended via the Address Functions request... no major

problem.

2) Memory will allow a memory area to be logged as the project.

This feature also allows a Process to be created, the starting

address of which will be the first address of the memory area.

It is not possible to create a Process for a ROM memory area.

3) Load Binary will allow a non-executable file to be loaded,

with the option of creating it as a Process. In addition, you

may specify a preferred memory address to which the binary file

should be loaded. If you specify an address, but the memory is

already used, then Disect will not load the binary file.

4) Boot Block allows a 1024-byte disk boot block to be loaded; it

may be created as a Process if required. The initial PC address

will be the 12th byte of the boot block. Note that when a boot

block is loaded, the first three longs are formatted appropriatly

(the boot ID, checksum, and whatever the third one is?). If a

Process is created, then Disect will pass the address of the

trackdisk's IOEXTTD structure to the Process in a1.

5) Load Data and Save Data can be used to load/save the data

that Disect creates when you disect a loaded project. This data

is formed when you do anything to an address within the project

memory range (replacing numbers with symbols, creating program

symbols, adding blank lines and comments, etc). Note that the

current disassembly window markers are also saved/loaded.

6) Save Binary allows a memory area to be saved to a file. Note

that you can resave a loaded binary file by specifying m9 and m8

for the start and end addresses (assuming you've not changed them

since loading the binary file).

7) Erase Data will erase any existing Disect Data.

Note that whenever a project is loaded, disassembly window marker 9 is auto-

matically defined at the first address of it. For binary files, marker 8 is

also defined at the last address.

INCBIN Icon

-----------

This icon will access the INCBIN request. This allows you to

define data areas which are to be disassembled using an INCBIN assembler

directive. In order to define an area, you simply enter the start and end

addresses of the area, and, if these are valid, Disect will then ask you to

select the file to save the binary area as. If the file is written ok, then

the INCBIN area will be defined.

You can select an existing INCBIN area by LMB in the address

ranges list window. In order to delete an INCBIN area, you select it, then

enter NULL strings for the start and end addresses (ie: empty the string

gadget, and press 'Enter'). An INCBIN area may be unselected by pressing

the RMB.

The Save gadget is used in order to re-save, or rename, an

existing INCBIN area's binary file. This is only achieved by selecting

an INCBIN area first.

Address Stack Icons

-------------------

The address stack is used in order to follow the flow of a prog-

ram. If the RMB is pressed over an address in the disassembly window, the

current window start address will be stacked, and the disassembly window will

be moved to the new address. The three icons are: retrieve last stacked

address, stack current disassembly window address, and delete last stacked

address. Note that if pressing left alt while RMB, the lower disassembly

window address will be stacked/changed.

Smiley Icon

-----------

Reserved for future use...

WorkBench Icon

--------------

This allows the workbench screen to be opened/closed.

Arrow Icons

-----------

The upper pair of left and right arrow icons can be used to move

the upper disassembly window; the lower pair are for the lower disassembly

window (if this is open). By also pressing the right ALT key, either pair

can be used to move the memory window.

* ------------------------------------------------------------------------- *

12) Keyboard Reference

======================

This section contains general program information, related to

the keys which invoke each feature of Disect. The complete list is below:

Windows	Cursor Keys		Move upper disassembly window

-------

& LEFT ALT		Move lower disassembly window

& RIGHT ALT		Move memory window

& SHIFT		Move window fast

A	{LEFT ALT}	Define disassembly window address

F1-F9	{LEFT ALT}	Go to disassembly window marker

SHIFT F1-F9	{LEFT ALT}	Define disassembly window marker

F10	{LEFT ALT}	Retrieve last address stack entry

ESCAPE		Refresh all windows

+		Swap both disassembly window addresses

\		Address/Offset display mode

TAB		Single/double disassembly window

QUOTE (above TAB)	Change number base (hex/decimal)

Y		Move disassembly window to previous addr

U		Move disassembly window to next address

O		Move disassembly window after data area

Functions	F		Search memory

---------	G		Continue search

W		Fill memory

Q		Copy memory

S		Display symbols

D		Disassemble to file

I		Display information

H		Display processor history

M		Display memory dump

Z		Evaluate expression

X		DataStrings (see section 7)

P		Dummy symbol selection

DEL		Display preferences

HELP		Display resident SysSymbol preferences

ESCAPE		Abort search/disassemble to file

Process	E		Execute instruction (whole of JSR/BSR)

-------	T		Trace instruction (stops inside JSR/BSR)

C		Breakpoint after PC & resume

V		Breakpoint at PC DEA & resume

N		Next instruction (without execute)

K		Kill Process

B		Define breakpoint

L		Kill breakpoints

DISASSEMBLY WINDOW ADDRESS (A)

==============================

This opens a window through which to input the required address.

If the left alt key is pressed with the 'A' key, then the input will be used

to define the lower disassembly window address.

DISASSEMBLY WINDOW MARKER (F1-F9)

=================================

With the shift key, keys F1 to F9 will define disassembly window

markers 1 to 9 at the current disassembly window address. To delete a marker,

both keys can be pressed a second time. Without the shift key, the disass-

embly window will be moved to the address of the selected marker. If the left

alt key is also pressed, then the lower disassembly window will be used.

RETRIEVE STACKED ADDRESS (F10)

==============================

Key F10 will retrieve the last stacked address and move the dis-

assembly window to it. Again, left alt can also be pressed to move the lower

disassembly window.

REFRESH MAIN WINDOWS (ESCAPE)

=============================

As well as refreshing all windows in the main program screen,

the escape key will, if a Process exists, move the (upper) disassembly window

to the current PC address (if this address is not visible in a disassembly

window).

MOVE TO PREVIOUS ADDRESS (Y)

============================

This will move the disassembly window to the previous logged

address from the first logged address in the disassembly window. Left alt

will select the lower disassembly window.

MOVE TO NEXT ADDRESS (U)

========================

This will move the disassembly window to the next logged address

from the first logged address in the disassembly window. Left alt will select

the lower disassembly window.

MOVE AFTER DATA AREA (O)

========================

This will move the disassembly window to the address after the

first logged data address in the disassembly window. This is useful in order

to pass large (uniform) data areas.

SEARCH MEMORY (F,G)

===================

Key 'F' will open the Search Memory request window, with which

you can define the ranges of memory to be searched, the type of search data,

and the data to be searched for.

At the top of the window are two string gadgets to input the

start and end addresses (inclusive) of a range. You should input the start,

and then the end, after which, if the range is ok, the address range will be

stored. Note that you may not enter zero as an address. The current ranges

are displayed in a window below these gadgets. To the right of this window

are gadgets to scroll through the list of address ranges. This list will be

initialised to contain the start and end addresses of any loaded project

memory. If you LMB on any address range, it will be selected, and can then

be changed. If you select a range, then enter no input for the start and end

addresses, the range will be deleted from the list. The RMB can be pressed to

un-select a selected address range. Note that this info applies to all other

windows where address ranges have to be specified.

The gadgets at the bottom of the window are used to select the

type of search. This may be byte, word, long, ASCII, or source. The string

gadget allows input of the search data. For byte/word/long, this must be a

sequence of expressions, with a comma to separate each parameter. For ASCII/

source, the text string entered is the search data. Note that ASCII will

search memory for a text string, while source will actually disassemble the

specified address ranges, and compare the resulting source with the input

text. Obviously, searching for source is slower. The case gadget allows you

to specify whether to ignore case when searching for ASCII/source. If check-

ed, then Disect will use case-sensitive search; if unchecked, then case will

be ignored.

If searching for byte/word/long, you may enter a single '?' char-

acter instead of an expression, in order to specify that a search-element is

unknown. For example, if you entered 32,"A",?,16 for a byte search, then the

following would be successfully matched: 32,65,46,16 and 32,65,12,16... etc

When searching for ASCII/source, the '?' character may be used in the same

way. In order to specify a search for a '?' character, you must enter '??'

instead. When searching for ASCII or source, any number of spaces input as

the search data will be matched to any number of spaces (or tabs) during the

search.

Key 'G' will continue a search which has been halted due to the

search data being found. If searching for byte/word/long or ASCII data, then

Disect will move the memory window (screen lower right) to the address where

the search data has been found. If searching for source, then the (upper)

disassembly window will be moved instead.

Note that no checks are performed on the memory areas: if you

(accidentally?) read from hardware registers, etc, you'll probably regret it...

An alarm sound will begin whenever the search data is found, or

when the search ends (data not found).

In order to specify the end of an address range, it is possible

to enter the following into the 'End' string gadget:

=	specifies an address range of a single byte (ie: the

end address is the same as the start address) - most

useful for Fill Memory (!)

+X	specifies an address range of (X+1) bytes (ie: the

end address is the start address + X; 'X' can be

any valid expression)

,X	specifies an address range of X bytes; 'X' can be

any valid expression

This is possible in all request windows which allow input of an address

range. (Search Memory/Fill Memory/Copy Memory/INCBIN/Load Memory/Save Binary/

Disassemble To File)

FILL MEMORY (W)

===============

This opens the Fill Memory request window, which is similar to

Search Memory (above). The fill data type may be byte, word, long, or ASCII.

Note that no checks are performed on the memory areas: if you (accidentally?)

write to hardware registers, etc, you'll probably regret it...

COPY MEMORY (Q)

===============

A window will open, allowing you to specify the start and end

addresses of the source memory area, and the start address of the destination

memory area. These two areas may overlap without any corruption occurring.

Note that no checks are performed on the memory areas: if you (accidentally?)

write to hardware registers, etc, you'll probably regret it...

DISPLAY SYMBOLS (S)

===================

This will open a window via which a list of all current user

symbols is displayed. At the window top is displayed the total number of

program and constant symbols. The gadgets at the bottom of the window allow

selection of whether to list program, constant, variable, or unused symbols,

and the order in which to list the symbols (either alphabetically, or by

increasing symbol value - NOTE that to resort the symbol list, you must

also press the RMB).

The next two gadgets allow you to delete or rename a selected

symbol. A symbol may be selected by LMB on it. Once a symbol has been

deleted, it cannot be retrieved. To rename a symbol, you should select it,

press the 'Rename' gadget, enter the new name into the string gadget which

is automatically activated, then press Enter.

There is also another gadget, 'VM'. This allows a constant symbol

to be changed to a variable symbol, and vice versa. See 'Process Variables

Memory' for more info.

As of V1.2, if you LMB on a program symbol which has already

been selected, then the symbol window will be closed, and the (upper)

disassembly window will be moved to the address of the symbol.

As of V1.3, if you press a letter key (with or without a shift

key), or the ".", or "_" keys, then, if the symbols have been sorted alpha-

betically, the window will move to the first symbol beginning with that

character.

DISASSEMBLE TO FILE (D)

=======================

Use of this feature is described in the tutorial (above); here is

some specific information.

The disassembly process occurs in two passes. On the first pass,

Disect disassembles the required address ranges internally, creating a list

of all referenced addresses which have not had a program symbol created for

them, a list of all required system symbols, and a list of all required user

symbols. Any immediate long numbers which lie within the disassembly address

range(s) which have not been replaced by a symbol are also checked for.

Also, during the first pass, Disect performs checks for any local

program symbol references across the bounds of a global, For example:

some_sr	bsr	.do_it

rts

another_sr	bsr	.elsewhere

.do_it	jsr	.miles_away

If any such references are found, Disect will move the upper disassembly

window to the reference of the local program symbol, and the lower window

to the address of it. Disassembly to file will be abandoned.

On the second pass, the disassembly occurs again; this time the

program and support source files are created. Now, any address references for

which no program symbol exists will be have automatic symbols written for

them. For example, the following code:

0000675372		lea	683208,a0

~	~	~

~	~	~

0000683208		DC.W	10

would be disassembled as:

lea	A683208,a0

~	~	~

~	~	~

A683208	DC.W	10

After the whole disassembly is complete, there may still be some referenced

addresses which were not disassembled directly. For example:

0000675372		move.w	683208,d0

~	~	~

~	~	~

0000683206	data_table	DC.L	10

0000683210		DC.L	20

Here, the first line would have been disassembled containing 'A683208', but

there would be no program symbol of this name. Instead, any missing address

references will be output to the end of the program file, by offsetting them

from the previous global program symbol. As an example, the above would create:

A683208	EQU	data_table+2

If Disect finds a data area where all bytes are NULL, instead of

creating lots of 'DC.X 0' source, Disect will convert the source to a single

line of 'DCB.X n', where n=the required number of bytes/words/longs. This

will not occur if the first line of the data area (ie:the only logged address

of the data area) has had it's DC.X operand (a zero!) replaced with a symbol,

or if the line has a line comment.

During disassembly to file, Disect will concatenate multiple

lines of DC.B, DC.W, or DC.L statements to form (much) fewer lines of source.

DISPLAY INFORMATION (I)

=======================

The displayed information contains the number of logged program

addresses, and the number of program and constant user symbols. Below this

is displayed the address range and size of each project memory area, with the

type of the memory (C=chip, F=fast, P=public).

DISPLAY PROCESSOR HISTORY (H)

=============================

After any instruction is traced or executed (or skipped via key

'N' - see below), or whenever an active Process reaches a breakpoint, or

whenever an exception occurs, the current processor state is stored. This

information is accessed via the processor history display. The most recent

entry is displayed in the lower window, with previous entries above. The

display of register contents is in hex, although by LMB on the digits of a

number, its value is displayed in denary in the small box in the top left of

the request window. The thing to note is that for any entry, the register

values represent the values after executing the previous instruction, while

the PC, SEA/DEA and instruction disassembly represent the next instruction

to be executed.

DISPLAY MEMORY DUMP (M)

=======================

This opens a large memory dump window, the start address of which

will be initialised to the current address of the first line of the small

memory window in the main program screen. The window may be moved around

memory using the cursor keys (and the shift key to move it faster). A string

gadget in the window top left allows a new start address to be input. The

base of the address digits will be the same as the currently selected base

for the main program screen (denary or hex). Note that the ASCII display will

show the character '' for any memory locations for which the content is out-

side the ASCII range of 32 to 127.

By LMB on a memory location's content, Disect will display the

denary values of the byte, word, and long starting at that address. The cycle

gadget in the window allows you to select whether to display the B/W/L as

signed or unsigned denary.

RMB on a memory location's content will cause the Base Offset

Address to be defined at the address of the selected byte. The preferences

will also be updated to display offsets instead of addresses.

EVALUATE EXPRESSION (Z)

=======================

This opens a window containing a string gadget for input of an

expression. The current string content will be evaluated whenever Enter is

pressed. Expression results are displayed as unsigned denary, signed denary

(if the sign bit is set; ie, either bit 31, bit 15, or bit 7 depending upon

whether the value is in the long, word, or byte value range - try it out with

results of 4294967295, 65535, and 255 - you'll see what I mean!), hex, ASCII,

binary, and any program symbol if one is found of the result value.

DUMMY SYMBOL SELECTION (P)

==========================

This will request a single expression, the result of which will

be passed to the Symbol Selection Window. This allows you to select symbol(s)

in the usual way, for any number, although obviously no information will be

stored about your selection. When a symbol match occurs, the window will

display the 'New', 'Zero', and 'Quit' gadgets as usual, but the 'Confirm'

gadget will be replaced with a 'Comment' gadget. If this is selected, then

the Symbol Selection window will close, and a message will appear in the

main window. By LMB on the mnemonic of an instruction within the disassembly

window, the actual text of the selected symbols can be automatically stored

as a line comment for the instruction. The comment text will automatically

have the chars "*!*" added to the end, allowing such comments to be found

in disassembled a source file, using a text editor. As an example, if a

program routine allocated temporary space for definition of a NewWindow

structure:

link	a5,#-48

Although you cannot replace '-48' with 'nw_SIZE', by making a dummy selection

for a value of '48', the Symbol Selection window will list 'nw_SIZE'. By

selecting this, then the 'Comment' gadget, the symbol text is stored as

a comment (with a marker allowing it to be found and edited, when a source

file has been disassembled).

DISPLAY PREFERENCES (DEL)

=========================

This window allows selection of program preferences. The gadgets

at the bottom of the window allow the last saved preferences to be loaded,

the current settings to be saved, the current settings to be used, and the

previous settings to be restored (ignoring any changes since window opened).

The preferences are as follows:

AutoSymbol	This enables/disables automatic symbol replacement. This means

that as addresses are disassembled via display of the disassembly

window(s), any address which has not been replaced by a program

symbol will automatically be replaced by any program symbol which

has a matching value. In addition, if the 'hardware' directory

SysSymbols are resident, then any references to chip registers

or CIA addresses will also be automatically replaced. Also, any

instructions of the form 'jsr dd(a6)' will have their displace-

ments replaced with _LVO symbols, if a6 contains the base address

of any of the libraries: Exec, DOS, Graphics, Intuition, ASL,

MathFFP, GadTools. This only occurs if a Process exists, and

will take place whenever the instruction at the PC is a jsr.

This preference also controls Process Variables Memory references

symbol replacement (see elsewhere).

Auto 0(Ax)	This enables/disables automatic display of a '0' before '(Ax)'

for the address-register-indirect addressing mode. This is useful

because some assemblers will automatically optimise address-

register-indirection-with-displacement references to ARI if the

displacement is zero. This feature allows a '0' to be displayed

so that it can then be replaced with a symbol (for example:

exec/lists.i 'LH_HEAD' which has the value 0).

Case	This allows selection of upper or lower case for disassembly.

Note that assembler directives will always be disassembled as

upper case.

Base	This allows selection of denary or hex for disassembly.

A7/SP	This allows selection of 'a7' or 'sp' for disassembly.

Local	This allows selection of the character to prefix local program

symbols. This may be either '.' or '_'.

PSymb ':'	This enables/disables automatic display of a ':' character after

program symbols in the program symbol field of a line.

SAD	This enables/disables automatic display of program symbols

instead of addresses in such places as register values, memory

dump addresses, addresses in request windows, etc.

Offset	This allows addresses to be displayed as hunk/project memory

start offsets instead. If selected, any address which lie within

the project memory range will be displayed as, for example,

"0:256", where 0 is the hunk number, and 256 is the offset.

In the disassembly window, this will be enclosed in "[]" chars.

In addition, you may define a base offset address (using an item

from the preferences menu). If you do this, then ALL addresses,

not just addresses in project memory range, will be displayed

as offsets relative to the base address, eg: "-$32", or "[+146]".

A small request window is used to define the base address. This

contains a string gadget for input of the address, and a check

gadget to enable/disable the current base address.

Comment DS	This enables/disables automatic line comments when Disect defines

a data area using a system DataString.

Erase DSC	This enables/disables automatic removal of DataString-defined

line comments after symbolic replacement of a number.

Blank Lines	This preference enables/disables automatic blank lines. If used,

Disect will add blank lines after any of these instructions:

DBcc, Bcc, JMP, JSR, RTD, RTE, RTR, RTS.

Process Mem	This informs Disect whether or not to install a memory monitor

when a Process is created (not if a Process is grabbed). If one

is installed, then when the Process terminates, a request window

will open to list any unfreed memory allocations. Disect will

display the start and end addresses of each memory block, as well

as the size. Also displayed will be either: 'SELF' if the memory

was allocated by a function within the Process; the name of the

library containing the function which allocated the memory (if

this can be determined); 'ROM' if the memory was allocated by a

ROM function which could not be named as a library. If the alloc-

ator cannot be determined, then the return address when AllocMem()

was called will be displayed.

The memory monitor is installed by calling Exec.SetFunction() for

the functions AllocMem() and FreeMem(). Before doing this, Disect

first checks the current destination addresses of their JMP inst-

ructions (the jump table just below ExecBase). If either of these

instructions do not jump to a ROM address, then Disect will not

attempt to install the memory monitor. Note that it is necessary

to install a memory monitor for certain features of the Auto

Trace mode (see elsewhere).

DISPLAY SYSSYMBOL PREFERENCES (HELP)

====================================

As mentioned above, this window allows selection of resident

SysSymbol (system symbol) data. The four gadgets at the bottom of the window

work as for Preferences (above). Response to changing the preferences occurs

when the window is closed. Note that you cannot remove a directory's data

from memory if it is used (ie: if any of it's symbols are used by the project

in memory). The digits after each directory's name represent the size of the

data file. It's best not to load data without using it, since any resident

data is checked in such circumstances as expression evaluation, checking if

a symbol name is used already, etc.

EXECUTE INSTRUCTION (E)

=======================

Only available if a Process exists, this will execute the single

instruction at the PC. Note that the whole of a JSR/BSR will be executed, ie:

the Process will be suspended at the address after the JSR/BSR instruction.

Any permanent breakpoint at the PC will be ignored.

One small point to add is that it is possible to execute PC-

relative JSR/BSR instructions which use 68020 specific address modes, even

with a pre-68020 processor. (The reason: JSR/BSR instructions are buffered

before execution, so therefore PC-relative modes get changed to absolute-

long address mode. In the case of BSR, the instruction is changed to JSR

absolute-long, so therefore any BSR which uses an 020 mode is permitted

on a pre-68020 processor. If you didn't follow that, it does not matter...)

TRACE INSTRUCTION (T)

=====================

This will trace the instruction at the PC, if a Process exists.

This means that if the instruction is a JSR/BSR, then the Process will be

suspended at the first instruction of the subroutine which is called. Any

permanent breakpoint at the PC will be ignored. In addition, if you trace

a JSR or a BSR, then any form of breakpoint at the PC is ignored.

RESUME PROCESS (Process Menu)

=============================

This will activate a suspended Process, hence it and Disect will

be running 'simultaneously' (multi-tasking). Any permanent breakpoint at the

PC will be ignored during the (first) execution of the instruction there.

BREAKPOINT AFTER PC (C)

=======================

This will define a stop breakpoint after the instruction at the

PC, and resume the Process. This is useful if the instruction is a DBcc/Bcc,

since an entire program loop may be executed, and the Process suspended after

the loop has been completed the required number of times. A permanent break-

point at the PC will be ignored during the (first) execution of the instruc-

tion there.

BREAKPOINT AT DEA (V)

=====================

This will define a stop breakpoint at the DEA of the instruction

at the PC, and resume the Process. This, again, is useful for executing

program loops, but ones where the exit point of a loop does not sequentially

lead to the next instruction. Any permament breakpoint at the PC will be

ignored during the (first) execution of the instruction there.

NEXT INSTRUCTION (N)

====================

This will move the PC to the next instruction without executing

the current one. Any breakpoint at the PC will be ignored.

KILL PROCESS (K)

================

This will kill a suspended Process. This is achieved by changing

the Process's stack pointer to the address it was when the Process began,

moving the PC to an RTS instruction, and resuming the Process.

Whenever a Process terminates, if any data exists, you will be

given a chance to save it before it is (automatically) erased.

DEFINE BREAKPOINT (B)

=====================

This allows a breakpoint to be defined. The program requests the

address of the breakpoint, and it's type. This may be permanent, conditional,

stop, or count. Conditional breakpoints also require an expression. This will

be evaluated after input, and if no error occurs, the breakpoint will be

defined. Whenever execution of an active Process reaches a conditional break-

point, then its expression will be evaluated. If the result is not zero, then

the breakpoint is removed, and the Process suspended. If the result is FALSE

(equal to zero), then the Process is resumed. If the expression cannot be

evaluated (for example, due to an unknown symbol name), then the Process will

be suspended, with the breakpoint intact, and an error message displayed.

Count breakpoints have associated with them a long counter. When-

ever the breakpoint is reached, the counter is decreased. If it reaches zero,

then the breakpoint is removed, and the Process suspended, otherwise the

Process will resume.

Stop breakpoints are simple breakpoints which will cause the

Process to be suspended. They are removed whenever encountered. Stop break-

points are in fact count breakpoints with an initial counter value of 1.

Note that an alternative method of defining a breakpoint is to

LMB on the address digits in the disassembly window, at the address where the

breakpoint is required.

When an active Process reaches a breakpoint, if the breakpoint

causes the Process to be suspended, then the PC will be at the address of

the breakpoint, and the instruction there will not yet have been executed.

Whenever an exception occurs, the PC will either be at the address of the

instruction which caused it, or at the address of the next instruction.

KILL BREAKPOINTS (L)

====================

This will remove all currently-defined breakpoints. It works even

when the Process is active.

* ------------------------------------------------------------------------- *

13) Mouse Reference

===================

This section describes the areas of the main program screen which

will respond to LMB or RMB activity.

Disassembly Window

------------------

1) Address Digits LMB at left of window to define breakpoint. #

2) Address Digits RMB at left of window to remove breakpoint.

3) Program Symbol area LMB define program symbol for the address.

4) Program Symbol area RMB on a local moves window to its global.

5) Mnemonic LMB to access address functions.

6) Mnemonic RMB to move PC to instruction.

7) Number LMB in instruction operands to replace with symbol(s).

8) Program Address RMB in instruction operands to stack current

address and move window to new address. *

9) Displacement RMB in instruction operands to stack current

address and move window to EA, eg: 32(a6). *

# Right ALT may also be pressed, which will move the memory window to the

address instead of defining a breakpoint.

* Left ALT may also be pressed to move the lower disassembly window; right

ALT may be pressed to move the memory window (in which case, no address

will be stored on the address stack).

PC/SR/EA Window

---------------

1) PC Value Digits LMB to define new PC value.

2) SR Value Digits/Flags Text LMB to define new SR value.

3) SEA/DEA Text LMB to select register zoom lock.

4) SEA/DEA Value Digits RMB to display previous program symbol.

Register Window

---------------

1) D0-D7/A0-A7 Value Digits LMB to define new register value.

2) D0-D7/A0-A7 Text LMB to select register-zoom lock.

Memory Window

-------------

1) Address Digits LMB to define new memory display address.

The following request windows also have responses to mouse buttons:

Memory Dump

-----------

1) Memory content LMB to display byte/word/long value as denary.

2) Memory content RMB to define base offset address.

Breakpoints Display

-------------------

1) Breakpoint information LMB to move (upper) disassembly window.

Symbol Display

--------------

1) Symbol name LMB (twice) to move (upper) disassembly window.

* ------------------------------------------------------------------------- *

14) Menu Reference

==================

This section details any program functions which are accessible

only via menus (no keyboard commands), and are not documented elsewhere.

SCAN '.GS' FILE (Project Menu)

==============================

This allows a '.gs' file as produced by GenAm to be scanned, and

its constant symbols extracted. When doing this, if Disect finds a symbol

name which is already used (either by a system or a user symbol), then the

values of the two symbols are compared. If they both have the same value,

then the symbol from the '.gs' file is ignored. If the values differ, then

a request window will open, allowing you to either rename the symbol from

the '.gs' file (type in the new name, and press 'Enter'), or to ignore the

symbol from the '.gs' file (click on the 'Ignore' gadget). Note: it is best

to scan a '.gs' file ONLY AFTER you have already loaded any required system

symbols for your current project. This is because when SysSymbol data is

loaded, no checks occur for existing symbol names. When you later save your

'.DSCT' data, the symbols read from the '.gs' file will also be saved, so

you only need to scan a '.gs' file once.

BREAKPOINT AT ACTIVE PROCESS PC (Process Menu)

==============================================

This allows Disect to regain control of an active Process. In

terms of Exec task-switching, the Process will be either Wait()ing on signals

or Ready (suspended only until its next quantum). If the Process is Waiting,

then the breakpoint will be defined, but the Process will remain Waiting

until it receives a valid signal (caused by, eg: LMB on it's window), at

which point it will reach the breakpoint.

If the Process is Ready, then the breakpoint can only be defined

if the Process's PC is within project memory range (ie: not within ROM, or a

system library, etc).

DISPLAY BREAKPOINTS (Process Menu)

==================================

This displays any defined breakpoints: the breakpoint address,

and breakpoint type. Permanent breakpoints are denoted by '*' and conditional

by '?'. Count and stop breakpoints have a number displayed with represents

the remaining counter value (a stop breakpoint is a count breakpoint of value

1). Conditional breakpoints have their expressions displayed after the '?'.

By LMB on a breakpoint text line, the (upper) disassembly window can be moved

to the address of the breakpoint.

* ------------------------------------------------------------------------- *

15) Process Variables Memory

============================

As of V1.7, if a Process exists, it is possible to define an area

of memory as variables memory which is used by the Process. This memory may

be either within the Process's memory (project memory range), or it may have

been allocated by the Process. This is most useful when dissecting a program

which references its variables using either of the addressing modes ARID

(eg: '32(a4)'), or ARIDI (eg: '16(a5,d0.w)'), since it allows references

to known variables to be automatically replaced with user-defined symbols,

as will normally occur with direct address references to logged addresses.

The 'Process' menu contains an item 'Variables Memory'. This

will open a request window which contains: two string gadgets for input of

variables memory start and size; a single string gadget for input of the

address of a 'JSR _LVOAllocMem(a6)' instruction; and two more gadgets, 'Ax',

and 'Previous'.

The first two string gadgets allow the variables memory area to

be specified directly, if this is known. If you enter a start address and a

size which matches the address and size of any block of memory which is known

to have been allocated by the Process, then the address of the 'JSR AllocMem'

which allocated this memory will automatically be stored in the third string

gadget. It is also possible to enter the address of a 'JSR AllocMem', from

which Disect will be able to determine the address and size of the variables

memory. The 'Previous' gadget will automatically define all three string

gadgets according to the last memory allocation made by the Process. Note

that this is only possible if a memory monitor was installed when the Process

was created (see main preferences).	The 'Ax' gadget allows you to select

which address register is to be taken as a variables memory base pointer.

The purpose of this is best illustrated by example. If you were

dissecting a Process which had allocated a block of memory and stored its

address in a4, or had done 'lea [1:0],a4', and you had the following:

move.b	#24,32(a4)

If you LMB on the '32' and replace it with a user constant symbol which you

named 'some_var':

move.b	#24,some_var(a4)

If the DEA is within a known variables memory area, then the

symbol 'some_var' is treated as a 'variable constant symbol'. What this means

is that whenever the PC reaches any instruction which contains either an ARID

or a ARIDI reference such as '32(a4)', then Disect will automatically replace

the '32' with the known variable symbol 'some_var'.

All of the information contained in the Variables Memory request

window is saved as part of the DSCT data file, so that, for example, once

the 'JSR AllocMem' which allocates the variables memory has been identified,

Disect will automatically define the variables memory start and size when

this instruction is executed the next time you load the original Process.

* ------------------------------------------------------------------------- *

16) Grabbing An Existing Process

================================

The 'Project' menu item 'Grab Process' allows an existing memory-

resident Process to be grabbed, and used as if the Process had been created

by Disect in the normal way. A request window will open, which will list

all current Processes which are waiting, either as a result of a call to

Exec Wait(), or Exec WaitPort(). To grab a Process, simply LMB on its name

to select it, then click on the 'Grab' gadget. At this point, if the Process

is no longer Waiting, then Disect will not attempt to grab it. If the Process

is still Waiting, Disect will effectively send it one of the signal(s) it is

waiting for, and the Process will resume, under control of Disect. Lovely!

Also in the 'Project' menu is another item 'Ungrab Process'. This

will allow a grabbed Process to be resumed, no longer under the control of

Disect.

This feature should be used with caution. It is vital that some

Processes remain functional at all times, ie: it is not safe to grab them

since their activities must not be interrupted. The only way to discover if

a Process can be safely grabbed (and ungrabbed) is to try it. Note that some

Processes can be grabbed, and only when you attempt to ungrab them will

problems occur. You have been warned...

Another word of warning: a Process which has been grabbed may

have its SegList memory (the memory in which the Process resides) freed

automatically, as soon as the Process has replied to its WorkBench start-up

message. As a result, it may be unsafe to single-step the last instructions

of a Process, since the OS may free the Process's memory before the very last

instruction has been executed by Disect. As an example:

bsr	close_down	; reply to WB message

moveq.l	#0,d0	; set return code

rts		; Process terminates here !

If you execute (key 'E') the bsr, the Process replies to its WB start-up

message, which may cause the Process's memory to be freed. Disect will

then have a PC at the moveq.l instruction, but any operation which uses

memory (eg: opening a Disect window) may cause the content of the PC

address to become corrupt. If this occurs, then you should select the

'Move PC To Return Code' item from the 'Process' menu. This will move

the PC to an address which is guaranteed to contain the final two

instructions in the example above.

* ------------------------------------------------------------------------- *

17) Auto Trace Mode

===================

Hopefully, this will at times prove to be a rather useful little

feature. What it does is to automatically trace the instructions of your

Process until some certain pre-specified condition is met. It is accessed

via the 'Process' menu, which will open a request window. This contains a

'Resume' and a 'Quit' gadget: the first will enable auto trace mode, whilst

the second will not. Note that once enabled, auto trace mode can be disabled

be pressing the 'Esc' key. Whenever you save the main program preferences,

then the current auto trace mode settings and expressions are also saved. The

remaining content of the auto trace request window is roughly divided into

two.

On the left, you have six check gadgets and two string gadgets.

These define the conditions which will cause auto trace mode to be disabled.

The first two check gadgets are used to define whether or not auto trace mode

is disabled whenever an instruction is reached which will attempt to perform

a word or long read/write using an odd source/destination effective address.

As with all the conditions which may disable auto trace mode, the Process

will 'suspend' before the current instruction has been executed.

The third check gadget is used to disable auto trace mode when-

ever the instruction at the PC is RTS. This can be used to effectively

resume the Process until the RTS of the current function/subroutine. Note

that Disect will count how many JSR/BSR instructions are traced, in order

to know how many RTS instructions must be ignored before the actual RTS

which will disable auto trace mode is reached.

The next check gadget, if checked, will cause auto trace mode

to be disabled whenever the destination effective address of an instruction

is not within 'permitted' memory. Disect considers the following to be

permitted:

1) The memory in which the Process resides.

2) The memory in which the Process's Process STRUCTURE resides.

3) The Process's stack.

4) Memory used by the LIB structure of any resident library.

5) Any memory allocated by the Process.

Any writes to memory allocated by the Process will only be detected as being

permitted if a memory monitor was installed when the Process was created (not

grabbed!). This option can be found in the Preferences request window. For

word or long writes to memory, Disect will also check if the address of the

last byte written is outside permitted memory.

The final two check gadgets are used to enable/disable each

expression entered into the two string gadgets. If enabled, before tracing

an instruction, Disect will evaluate an expression, and will disable auto

trace mode if the result is TRUE (non-zero). Note that this is the equivalent

of placing a conditional breakpoint at every instruction of your Process

(which would be otherwise impossible), including any external libraries or

functions, including ROM (also not otherwise possible). If the expression

contains either of the reserved symbol names 'SEA' and/or 'DEA', then if

an instruction does not have a source/destination effective address, the

expression will be ignored (for that instruction only). As a result, Disect

will not allow a single expression to contain BOTH 'SEA' and 'DEA' (since

this would result in the expression being ignored for instructions which

have only a source OR a destination effective address). This is why there

are two expressions permitted: one can contain 'SEA', and the other, 'DEA'.

The are other conditions which will always cause auto trace mode

to be disabled: if PC is 0 or odd; if the opcode at PC cannot be identified;

if the destination effective address is hardware register INTENA; if the

instruction at PC is privileged; or if a breakpoint is reached (if the

breakpoint would normally cause an active Process to be suspended).

In the right of the auto trace request window are seven check

gadgets used to control various options. The first two tell Disect whether

or not to trace ROM functions, and any external functions (not within the

memory range of the Process, and not in ROM). Whenever a JSR/BSR instruction

is reached, Disect uses the address of the subroutine to decide if it should

be traced. If not, then the JSR/BSR instruction is executed, as occurs via

pressing key 'E' normally, hence the subroutine will execute in real-time.

Note that a JSR/BSR is treated as a call to a ROM function either if the

subroutine address is in ROM, or if the instruction at this address is a JMP

into ROM (eg: exec.library). If a function is not traced, then auto trace

mode will resume after the function has returned.

The third check gadget will enable/disable writes to the history

buffer after each instruction has been traced. Whenever a JSR/BSR is executed

(ie: not traced), then the history buffer will be written to after the JSR/

BSR has completed, the function having returned successfully.

The final four check gadgets will enable/disable the refreshing

of the four main program windows after each instruction has been traced.

The second item in the Process menu is 'Resume Until RTS'. This

will enable auto-trace mode, with all options disabled except 'Suspend At

RTS'. This is useful for quickly getting to the end of the subroutine in

which the PC is currently located. Note that the 'Odd SEA' and 'Odd DEA'

options for auto trace mode will be as defined via the Auto Trace Mode

request window (ie: they are not automatically disabled during Resume Until

RTS).

Also in the 'Process' menu is another item 'Breakpoint After

External BSR/JSR'. If you're not interested in this then read no further...

Right, the reason I did this one was as follows: when tracing

ROM functions, more specifically exec.library Wait(), auto trace mode would

be disabled at the following instruction:

move.w	#$4000,$dff09a	; disable interrupts

Now, we cannot possibly expect to continue auto-trace mode (take my word for

it...), so what this menu item will do is to place a stop breakpoint at the

address after the last traced internal JSR/BSR to an external function.

What this means is that somewhere within the memory range of our Process is

the following:

~	~	~

~	~	~

jsr	_LVOWait(a6)

~	~	~

~	~	~

Since this is the (internal) JSR which called an external function, Disect

will define the breakpoint immediately after this instruction, and resume

the Process. Once the Wait() has completed, the Process will be suspended

after the JSR instruction. NOTE: before using this menu item, in this

example it is necessary to move the PC to the instruction before the

'move.w #,$dff09a' (ie: you MUST NOT use it with the PC at this instruction).

To move the PC to the previous instruction, you merely need to RMB on the

mnemonic of the instruction in the disassembly window. Remember this!

* ------------------------------------------------------------------------- *

18) Hardware Monitor

====================

This feature is essentially unfinished, although it is usable

(with caution), so I decided to include it in the demo version of Disect

(in the hope of some constructive feedback...?)

If enabled, the hardware monitor allows an active Process to

alter the values of certain hardware registers, without crashing the system

when the Process is suspended. This also applies to a Process being active

in the sense of only executing a single instruction. At present, the status

of DMA flags, interrupt flags, and ADK flags are monitored, as well as all

interrupt vectors, and all trap vectors. Support is not yet available for

any Process which alters the VBR. In addition, the TRACE and ILLEGAL vectors

must NOT be altered by the Process.

Disect also allows two user-definable functions to be used, one

to be called whenever the Process is resumed, and one to be called whenever

the Process is suspended. Each is defined via its own request window, which

contains a string gadget for input of the function address, and a check

gadget which is used to enable/disable the function. A 'Load' gadget also

allows an executable file to be loaded to be used as a function.

Also in the request window are 16 check gadgets which define

which registers must be preserved during the function, and 16 string gadgets

which allow register values to be passed to the function. If a register is

preserved, then it will be the same after the function as it was before the

function was called; any registers which are not preserved will contain the

return values from the function.

Any enabled function will be executed in Supervisor mode. The

function must remain in Supervisor mode, and must end with an 'RTS'.

* ------------------------------------------------------------------------- *

19) 68020 Modes/Instructions

============================

The following notes concern 68020-specific addressing modes, and

instructions:

1. In a 68020 effective address, only the base displacement can be replaced

by a symbol; the outer displacement cannot. For PC-relative modes, the value

displayed for the base displacement will be the result of adding it to the

PC value for the instruction. This address will be automatically replaced by

a matching program symbol (if the current preferences permit this). For modes

where the base register is an address register, if the base displacement is

a long, then an automatic program symbol match is checked for in the same

way.

2. The offset and width values for bit field instructions can be replaced by

symbols, although only a single user constant symbol is permitted. When using

the symbol selection window to replace a bit field offset/width value, the

symbol search type is pre-defined, and cannot be altered. Since these values

can be replaced by only one symbol each, the facility to enter an existing

symbol name (to be used as an offset) will be disabled.

If you replace a bit field value, the address of the BF instruction is not

logged in the usual way. This has no other implications, except it may save

you a little memory.

3. When a Process exists, during calculation of a 68020 effective address,

if any memory indirection attempts to read a non-existant address, then the

effective address will be considered invalid, and will not be displayed.

Also, if you're using a pre-68020 processor, any disassembly of 68020 modes

which attempts to perform a memory indirection at an odd address will render

the EA invalid, hence it will not be displayed.

4. The following 68020 instructions are not implemented: CAS, CAS2, CALLM,

RTM, PACK, UNPK.

* ------------------------------------------------------------------------- *

20) Installing To Hard Drive

============================

Ok, on the disk, the file 'DisectV1.7' is in fact a script. This

contains an ASSIGN:

ASSIGN DISECT: DisectV1.7:

The last name of this may be changed to reflect the partition

name where you wish to put Disect. Other than this, NOTHING else should be

altered. Simply copy the FOUR directories and FOUR files from the (floppy)

disk to (where-ever!). Important: DO NOT give the executable ('Disect.exe')

an icon in order to double-click on it to launch the program: ALWAYS double-

click on the script file ('DisectV1.7')

* ------------------------------------------------------------------------- *

21) Contact Address

===================

If you have any problems or suggestions concerning Disect, (or

if you want the full version), here's the address:

Dave Alderson

86, Powell Ave

Marton

Blackpool

Lancashire

England

FY4 3HH

Last, but certainly by no means least, many, many, many thanks

to Bill Westhead for helping spread the (e-mail) word, and also to the

following people for helping to test out the early versions:

Marc Kelly

Mark Manning

* ------------------------------------------------------------------------- *