====================

====== |         | |

|    | |---  --- | |

|  -+| |  | +--| | |

====|= ---- +--- - -

+-

====================

SMALL ARMS EXTERIOR BALLISTICS

for

IBM PC's and COMPATIBLES

-----------------------

Application Programming

and Documentation

by

Benjamin W. Hartley

103 Forest Park

Jaffrey, NH   03452 USA

Voice . . . . . . . . . . .(603) 532-6248

CompuServe. . . . . . . . . . .70033,2612

GEnieMail . . . . . . . . . . .B.Hartley1

Internet. . . . 70033,2612@compuserve.com

================================================================

+---------+

+-----+---+     |              (R)

--|         |O    |------------------

|   +---------+ |  Association of

|   |         |-+  Shareware

+---|    O    |    Professionals

------|    |    |--------------------

+---------+    MEMBER

The author is a member of the Association of Shareware Pro-

fessionals, the ASP.   ASP wants to make sure that the shareware

principle works for you.   If you are unable to resolve a share-

ware-related problem with an ASP member by contacting the member

directly, ASP may be able to help.  ASP's Ombudsman can help you

resolve a dispute or  problem with an  ASP member,  but does not

provide technical support for members' products. Please write to

the ASP Ombudsman at 545 Grover Rd., Muskegon, MI 49442-9427 USA

(FAX 616-788-5131 24 hrs./day), or send a message via CompuServe

mail to: ASP Ombudsman 72050,1433.

================================================================

QBALL  --  source code, executable files, and documentation

are (C)1991-95 by Benjamin W. Hartley. All rights reserved.

QBALL                                                           i

DEDICATION

QBALL was inspired by the many computer programs

published in The American Rifleman, and is

respectfully dedicated to their author:

William C. Davis, Jr.

----

DISCLAIMER and WARRANTY

QBALL is supplied as is!!  Responsibility for determining its

suitability for any use rests  solely  with the user.  The author

will not be held responsible for injury or damages resulting from

the use of QBALL;  all remedies rest solely with the user.  QBALL

is distributed without any warranties whatsoever,  except:  QBALL

copies distributed  on defective  author-supplied  media  will be

replaced;  or if QBALL is inoperative through an author error,  a

functional copy is  available  upon request.  The author reserves

the right to modify or update  QBALL  without incurring any obli-

gation to supply updates to any user,  except as otherwise stated

herein.

LIMITED LICENSE

Individual users may make copies of the  QBALL  program files

for backup and personal use.  Individual users may freely distri-

bute the Shareware Edition,  provided that all files are distrib-

uted in their entirety and without modification.

Computer clubs may make copies of the  Shareware Edition  for

distribution.  A media / copying fee not to exceed  $10.00 may be

charged.

Vendor  members of The Association of Shareware Professionals

and any BBS are hereby granted permission to distribute the Share-

ware Edition;  latest revision verification should,  however,  be

obtained from the author.  All other vendors must contact the au-

thor for permission before distribution may begin.

Distribution of the User's Manual in printed form,  or of any

part of the Registered Edition of QBALL, without the author's ex-

press written consent, is prohibited.

ACKNOWLEDGMENTS

The author gratefully acknowledges the efforts of Jan Archam-

beau, Bob Bender,  Lewis Hansen, Leif Bjarne Kleven, Terry Lyman,

Jim McCausland,  Jeremy Moss, Miguel Duran Perello,  Bob Pickell,

and Dave Smith,  all of whom have suggested  improvements to what

QBALL does,  how it does it,  and have provided valuable feedback

and encouragement.  Victor Parrish,  Manager of Radio Shack Store

#1044, Peterborough, NH, generously allowed  intermediate testing

on his store's computers. Many thanks to you all. � B.W.H.

QBALL                                                         iii

TABLE OF CONTENTS

Dedication, Disclaimer/Warranty, License, Acknowledgments . . . i

Copyright and Miscellaneous Information . . . . . . . . . . . .ii

Table of Contents . . . . . . . . . . . . . . . . . . . . . . iii

List of Appendices. . . . . . . . . . . . . . . . . . . . . . .iv

List of Figures . . . . . . . . . . . . . . . . . . . . . . . . v

List of Tables. . . . . . . . . . . . . . . . . . . . . . . . .vi

About Shareware . . . . . . . . . . . . . . . . . . . . . . . vii

Obtaining the Latest Shareware Edition. . . . . . . . . . . .viii

Introduction. . . . . . . . . . . . . . . . . . . . . . . . . .ix

Chapter                                                      Page

1. Getting Started. . . . . . . . . . . . . . . . . . . . . . . 1

1.1 Files Required . . . . . . . . . . . . . . . . . . . . . 1

1.2 Will It Work?. . . . . . . . . . . . . . . . . . . . . . 1

1.3 Copying the Program Files. . . . . . . . . . . . . . . . 1

1.4 QBALL Installation . . . . . . . . . . . . . . . . . . . 1

1.5 General Instructions . . . . . . . . . . . . . . . . . . 2

1.6 Starting QBALL . . . . . . . . . . . . . . . . . . . . . 3

1.7 Uninstalling or Removing QBALL . . . . . . . . . . . . . 7

2. Computing Ballistics Data

2.1 Initial Data Entry . . . . . . . . . . . . . . . . . . . 7

2.2 Computing and Displaying Range Tables. . . . . . . . . . 8

2.3 Range Tables Too Long to Display . . . . . . . . . . . .10

2.4 Other Range Table Output . . . . . . . . . . . . . . . .10

2.5 Printing Range Table Hardcopy. . . . . . . . . . . . . .11

2.6 Saving a Range Table to a Disk File. . . . . . . . . . .13

2.7 Slant Fire Effects . . . . . . . . . . . . . . . . . . .14

2.8 Computing and Displaying a Slant Fire Table. . . . . . .15

2.9 Printing Slant Fire Table Hardcopy . . . . . . . . . . .17

2.10 Saving a Slant Fire Table to a Disk File. . . . . . . .19

2.11 Displaying a Trajectory Graphic . . . . . . . . . . . .20

2.12 Other Ballistic Procedures. . . . . . . . . . . . . . .22

Point-Blank Range . . . . . . . . . . . . . . . . . . .23

Zero for Range and Height . . . . . . . . . . . . . . .25

Wind Vector Corrections . . . . . . . . . . . . . . . .26

Sectional Density . . . . . . . . . . . . . . . . . . .27

Free Recoil Velocity and Energy . . . . . . . . . . . .28

2.13 New or Changed Input Data . . . . . . . . . . . . . . .29

Weapon Identifier . . . . . . . . . . . . . . . . . . .30

Bullet Identifier . . . . . . . . . . . . . . . . . . .30

Muzzle Velocity . . . . . . . . . . . . . . . . . . . .30

Ballistic Coefficient . . . . . . . . . . . . . . . . .30

Bullet Weight . . . . . . . . . . . . . . . . . . . . .30

Metro (Meterological Conditions). . . . . . . . . . . .31

Sight Height. . . . . . . . . . . . . . . . . . . . . .31

Unit System Swap. . . . . . . . . . . . . . . . . . . .31

2.14 Bullet Lookup . . . . . . . . . . . . . . . . . . . . .32

2.15 Metric Data Limits and Conversion Factors . . . . . . .34

QBALL                                                           v

LIST OF FIGURES

Figure                                                       Page

1. Opening Screen . . . . . . . . . . . . . . . . . . . . . 3

2. Main Menu Display. . . . . . . . . . . . . . . . . . . . 4

3. Metric Units Information Line. . . . . . . . . . . . . . 4

4. A Clean Sweep Prompt . . . . . . . . . . . . . . . . . . 5

5. Required Data Entry. . . . . . . . . . . . . . . . . . . 7

6. Computations Menu. . . . . . . . . . . . . . . . . . . . 8

7. Partial Computations Menu, Valid Zero Set. . . . . . . . 8

8. Range Data Entry Window, All Data Set. . . . . . . . . . 9

9. Typical Range Table. . . . . . . . . . . . . . . . . . . 9

10. Partial Range Table. . . . . . . . . . . . . . . . . . .10

11. Range Table Output Menu. . . . . . . . . . . . . . . . .11

12. Printing Range Table Information Display . . . . . . . .11

13. Typical Printed Range Table. . . . . . . . . . . . . . .12

14. Entering a Filename for a Range Table Disk File. . . . .13

15. Saving a Range Table to a Specified Disk File. . . . . .13

16. Horizontal vs Elevated Fire. . . . . . . . . . . . . . .14

17. Choosing Vertical Displacement Type. . . . . . . . . . .15

18. Entering the Angular Vertical Displacement . . . . . . .15

19. Slant Fire Table, Angular Vertical Displacement. . . . .15

20. Slant Table Output Menu. . . . . . . . . . . . . . . . .16

21. Slant Fire Table, Linear Vertical Displacement . . . . .17

22. Printing Slant Fire Table Information Display. . . . . .17

23. Typical Printed Slant Fire Table . . . . . . . . . . . .18

24. Entering a Filename for a Slant Fire Table Disk File . .19

25. Saving a Slant Fire Table to a Specified Disk File . . .19

26. Typical Trajectory Graphics Display. . . . . . . . . . .20

27. Filename Entry Window. . . . . . . . . . . . . . . . . .21

28. Data Recall Window . . . . . . . . . . . . . . . . . . .21

29. Two Trajectories Displayed . . . . . . . . . . . . . . .22

30. The Other Ballistics Procedures Menu . . . . . . . . . .23

31. Point Blank Range Determination. . . . . . . . . . . . .24

32. Point Blank Zero Range Validated . . . . . . . . . . . .24

33. Data Entry, Point Blank Range Table. . . . . . . . . . .25

34. Range Table Using Point Blank Zero . . . . . . . . . . .25

35. Entering Zero for Range and Height Data. . . . . . . . .25

36. Range Table Using Most Probable Zero . . . . . . . . . .25

37. Probable Zero Warning Message. . . . . . . . . . . . . .26

38. Entering Wind Vector Correction Data . . . . . . . . . .26

39. Range Table Showing Nonstandard Wind Deflection. . . . .27

40. Entering a Bullet Diameter

and Display Sectional Density . . . . . . . . . . . . .27

41. Determining Free Recoil Velocity and Energy. . . . . . .28

42. A Clean Sweep Prompt . . . . . . . . . . . . . . . . . .29

43. New Input Data Menu. . . . . . . . . . . . . . . . . . .29

44. Entering a Weapon Identifier . . . . . . . . . . . . . .30

45. Entering a Bullet Identifier . . . . . . . . . . . . . .30

46. Entering Metro (Meteorological Conditions) . . . . . . .31

47. Entering a Sight Height. . . . . . . . . . . . . . . . .31

48. Entering a Ballistic Coefficient, Lookup Active. . . . .32

49. Bullet Lookup Window, Sorted by Diameter . . . . . . . .32

QBALL                                                         vii

ABOUT SHAREWARE

So you've spent a lot of money -- $200, $300, or more for a

computer program. You've tried it out, have found that it isn't

suitable for you, (or worse, doesn't work) and you want your money

back. Know what? Usually, you're out of luck, that's what.

Enter Shareware.

Shareware gives you, the end user, a chance to try software

before buying it, much as you might test-drive a new car. If you

try a Shareware program, and continue using it, you are expected

to register it, just as you would have to pay for the car.

Individual programs differ on details: you may get anything from

the simple right to continue using the software to an updated

program with printed manual. Shareware programs are neither free,

nor are they in the public domain. Copyright laws apply to both

Shareware and commercial software. The Shareware copyright holder,

generally the author, retains all rights to the program.

Shareware authors are accomplished programmers, just like

commercial authors, and the programs are of comparable quality.

(In both cases, there are good programs and bad ones!)

In sum, Shareware is a distribution method, not a type of

software. You should find software that suits your needs and

pocketbook; the Shareware system eases the search, as you can try

before you buy. And because the overhead is low, prices are low

also. Shareware also has the ultimate money-back guarantee: if you

don't use it, you don't pay for it.

QBALL is a shareware program, and you are encouraged to give

unaltered copies to your friends.

The essence of "user-supported" software is to provide personal

computer users with quality, low-cost software, while encouraging

programmers to continue to develop new products. If you find QBALL

useful, and continue to use QBALL after a 30-day trial period, you

must make a registration payment of $25.00 to the author. The

$25.00 registration fee will license one copy for use on any one

computer at any one time. QBALL may be used by any number of

people and may be freely moved from one computer location to

another, so long as there is no possibility of it being used at

one location while being used at another.

Commercial users of QBALL must register and pay for their

copies of QBALL within 30 days of first use or their license is

withdrawn. Contact the author to make site-license arrangements.

Anyone distributing QBALL for any kind of remuneration must first

contact the author for authorization.

Authorization is granted to distributor members of the Asso-

ciation of Shareware Professionals (ASP) who may begin offering

QBALL immediately. The author should be advised so that the

latest, complete version of QBALL is being distributed.

So there you are: quality software at a reasonable price, with

an unbeatable guarantee.

Wotta deal!

-- adapted from original text by Paul Mayer

QBALL                                                          ix

INTRODUCTION

QBALL is a menu-driven application for the rapid computation of

ballistics data for small arms projectiles. QBALL will quickly

compute point-of-impact data, velocity and energy levels, trajec-

tories relative to line-of-sight, and times of flight. Additional

routines allow the calculation of ballistic coefficients, muzzle

velocities from measured bullet drop or a single downrange

velocity, slant fire corrections, point-blank range, trajectory

data for a given range and height, and gun free recoil. Printing

routines (parallel printers only, LPT1) may be used to produce

hard copy and graphics printouts with properly configured

installations. Starting values and the display to which QBALL will

default may be changed using the built-in config-uration routines.

QBALL does not include routines for load development due to the

lack of available data, and product-liability considerations. One

such program is "Load From a Disk," a commercial product,

available from:

Load From a Disk

9826 Sagedale

Houston, TX 77089

You are also assumed to have some knowledge of exterior

ballistics: QBALL will not teach you the subject. Several of the

references listed in the bibliography contain introductions to the

subject, and their study will amply repay the effort expended.

Warning! All QBALL-generated data should be used as a guide to

ballistics performance, only. You must verify QBALL's data with a

standard reloading manual. Several such manuals are listed in the

bibliography.

If you find QBALL of use, but would like it to do something it

doesn't do now, or have suggestions to improve what is does

already, please send in a copy of the Enhancement Form. If you are

the first to suggest an improvement which is included in a later

QBALL release, you will receive a free copy of that release.

Should you have problems with QBALL, you may call the author at

603-532-6248; no collect calls, please! (Should you get the

answering machine, please leave your name, telephone number, and

the best time to return your call.) You may also contact the

author by CompuServe mail message to Ben Hartley, 70033,2612; on

the Internet 70033,2612@compuserve.com; or by GEnieMail, address

B.Hartley1. If you would prefer to write, please do so. The author

will respond to all requests for help.

The latest Registered Edition of QBALL is available from the

author for a $25.00 registration fee. Registration grants to you

an unlimited use license, and entitles you to one free upgrade, a

printed manual, a custom bullet lookup maintenance utility, and

other goodies which change from time to time.

------

QBALL

USER'S

MANUAL

------

QBALL                    Getting Started                        1

Chapter 1. Getting Started

1.1 Files Required.

QBALL is distributed with the files specified in the packing

list document.

The only absolutely required file is QBALL.EXE. In order to use

bullet lookup, however, AMMO.DAT and at least one of the *.NDX

files must be present. In order to configure your printer, both

PRINTERS.DAT and PRCONFIG.EXE must be present.

1.2 Will It Work ?

QBALL is fully functional on MS/PC-DOS compatible computers

equipped with a CGA or better graphics card. It has been found to

be compatible with MS-DOS 3.3, 4.01, 5.0, and 6.nn, the appropri-

ate version of COMMAND.COM, and with versions 3.02 through 4.02 of

JP Software's 4DOS command processor replacement. QBALL will func-

tion properly in the Windows(R) 3.n "DOS box," but will not func-

tion in OS/2 protected mode. QBALL occupies approximately 363k RAM

while operating with the supplied bullet lookup files.

1.3 Copying the Program Files.

It would be wise to make copies of the original QBALL files, and

keep a set of originals in a safe place. Do all the following

operations using your copies!

1.4 QBALL Installation.

Insert the copy of the disk labelled QBALL_EXECS into the

appropriate drive, log onto that drive, type:

INSTALL (+<enter>)

and follow the prompts to install QBALL.

While you may install QBALL on a high-density floppy disk, it

loads sl-o-o-o-w. QBALL should be installed on a hard disk if at

all possible. QBALL installation requires 738 kBytes of disk space

without any documents, or 1020 kBytes with all documents.

The install utility will ask for the target directory in which

to install the QBALL files - C:\QBALL310 is the default - and will

create that directory if it doesn't exist. No other changes will

be made to the existing files or directory structure on your hard

disk. See section 1.7, pg 6 for removal instructions.

+---------------------------------------------------------------+

|QBALL uses "Universal INSTALL" - a product of The GoodSoft Co.,|

|1630 30th Street, Suite 235, Boulder, CO 80301, USA - which was|

|provided through the good offices of Matt Brown of GoodSoft.   |

+---------------------------------------------------------------+

QBALL                    Getting Started                        3

1.6 Starting QBALL.

Log into the directory or onto the disk where you have

installed QBALL.EXE, and type:

QBALL (+<enter>)

+--------------------------------------------------------+

| Hint: If you have a monochrome monitor, or if you just |

| don't like colors, start QBALL with:                   |

|                     QBALL /b (+<enter>)                |

| and save the black/white configuration. See Chapter 5. |

+--------------------------------------------------------+

QBALL will display some startup information, registration

information, and this opening screen:

=================================================================

|                ==========  +               +  +               |

|                |        |  |               |  |               |

|                |        |  +----+   ----+  |  |               |

|                |        |  |    |  +----+  |  |               |

|                |    =+  |  |    |  |    |  |  |               |

|                ======|===  =====+  +=====  =  =               |

|                      +=       Shareware Edition               |

|        [ Exterior Ballistics for PC's and Compatibles ]       |

|        [ v3.10   --   (C) 1991-5, Benjamin W. Hartley ]       |

|        [ Shareware - 30-day Limited Trial Use License ]       |

-----------------------------------------------------------------

When you need fast, accurate smallarms ballistics data.

===============================================================

|NOTICE!! Users are encouraged to verify QBALL-derived data...|

|standard reloading manual.  The author is NOT responsible... |

|misuse or faulty application of QBALL-derived data.          |

---------------------------------------------------------------

EGA

===>> Press any key to continue <<===       Detected

Figure 1. Opening Screen.

QBALL reports what kind of graphics it has found; here, an EGA

card is installed. The alternative is CGA; VGA falls through to

EGA standard. If your default setup enables the printer (the as-

shipped QBALL default), and your printer is off or off line, a

"Printer Fault" message will also appear.

The QBALL version number is shown on the opening screen. In

this example, it's "v3.10." If you need to contact the author with

comments, suggestions, or requests for help, it would be helpful

if you include the version number with your communication.

Press any key to continue. (QBALL will proceed automatically in

60 seconds.)

QBALL                    Getting Started                        5

One frequent use for QBALL is the production of ballistics

tables for a specific cartridge/weapon/load combination. With the

Main Menu displayed, press <C> to access the ballistic computation

routines. This option is discussed in much greater detail in

Chapter 2.

Should you wish to infer a ballistic coefficient or a muzzle

velocity, then press <I>. See Chapter 3 for more information.

You may change the screen display colors; toggle the printer,

graphics grid, and units system; shell-to-DOS; and save new or

changed default startup values. Press <U> to access the Utilities

Menu; see Chapter 5 for more information.

You may also access the "Read a Range Table" (RRTABLE.EXE) from

the Utilities Menu; see chapter 5, section 5.6.

If you have saved graphics trajectory data to a file, you may

recall it for use by pressing <R>. As distributed, QBALL includes

one such sample file. If the file was not copied to the QBALL

directory, this option will not appear. See Chapter 6 for more

information.

To exit QBALL and return to DOS, press <Q>. QBALL will ask you

to confirm that you indeed wish to quit. Press <Y> or <enter> to

return to DOS, or <N> to continue. If you have saved trajectory

data, you will also be asked whether or not to retain it on the

disk. <Y> or <enter> keeps the data, <N> deletes it. You should

retain the data - press <Y> or <enter> - until you've worked with

QBALL for a while.

Let's put QBALL to work... Press <C> to compute ballistic data,

and to do the tutorial which follows. Go on to Chapter 2, page 7.

----------

If this appears when you press <C>, (it shouldn't) then press

<N> to clear any already-entered data and do the tutorial.

=======================================

|      *** M A I N   M E N U ***      |

|  ---------------------------------  |

|  <C>ompute Ballistic or Gun Data    |

===| Clean Sweep... �============================

|                                               |

|       Retain Displayed Data? ([Y] or N)       |

|                                               |

-------------------------------------------------

---------------------------------------

Figure 4. A Clean Sweep Prompt.

See also New or Changed Input Data on page 30.

QBALL               Computing Ballistics Data                   7

Chapter 2. Computing Ballistics Data

2.1 Initial Data Entry.

QBALL requires some initial data to work with. You will be

prompted for the required items; enter the following data:

a. Ballistic Coefficient. . . . . . . . .    .300

------------------------------------------------

| Enter the ballistic coefficient and the bul- |

| let weight manually, and try QBALL's bullet  |

| lookup feature later. (See section 2.14).    |

------------------------------------------------

b. Bullet Weight in grains . . . . . . . . . .180

------------------------------------------------

| This tutorial is based on English units; the |

| same principals apply to Metric units.  (See |

| section 2.15).                               |

------------------------------------------------

c. Use Displayed Weapon?. . . . . .<Y> or <enter>

d. Show Blank Bullet Designator?. .<Y> or <enter>

e. Use Displayed Metro? . . . . . .<Y> or <enter>

------------------------------------------------

| To keep matters simple, use the defaults by  |

| responding Y to these. Remember that <enter> |

| is the same as "Yes" when asked "Y or N."    |

------------------------------------------------

f. The Muzzle Velocity in FPS . . . . . . . .2700

As you can see, the screen display and the info line stay in

step with your entries, as shown in Figure 5.

> Ballistic Computations, Typical Firearm <

---| Enter Required Initial Data... |------------

|                                               |

|   Ballistic coefficient               .300    |

|   Bullet weight            (grains)    180    |

|   Weapon                   Typical Firearm    |

|   Bullet                           �������    |

|   Metro                       As displayed    |

|   Muzzle Velocity:            (FPS)   2700    |

|                                               |

-------------------------------------------------

MV:2700 |C1:.300 |G:180 |Metro:59d 0' 29.53" 78% |H:1.00 |E

Figure 5. Required Data Entry.

QBALL                Computing Ballistic Data                    9

As you stepped through range data entry, the entry window

stayed in step with you, and looks like this when you've entered

all the required data:

=======================================

|    ***   Computations Menu   ***    |

---| Enter Range Data in Yards... |--------------

|                                               |

|   Zero Range     :                     250    |

|   Maximum Range  :                     300    |

|   Range Increment:                      50    |

|                                               |

-------------------------------------------------

---------------------------------------

Figure 8. Range Data Entry Window, All Data Set

After a relatively short time for computations - how short de-

pends partially upon the maximum range and range increment, and

partially upon your computer's speed - QBALL displays the

following table:

-----------------------------------------------------------------

|              Range Table, Zeroed at 250 Yards                 |

|---------------------------------------------------------------|

| Range   R e m a i n i n g  Drop  Path/Sight Line 10 MPH Time  |

| Yards   V: FPS  E: Ft-Lbs Inches M.O.A.   Inches Inches Flght |

| -----   ----------------- ------ --------------- ------ ----- |

|     0     2700      2913    0.00   --      -1.00   0.00 0.000 |

|:  13.60   2659      2826    0.04  0.00      0.00   0.00 0.015:|

|    50     2551      2601    0.62  4.25      2.22   0.28 0.057 |

|   100     2407      2315    2.56  3.94      4.12   1.11 0.117 |

|+ 136.78   2304      2122    4.95  3.19      4.56   2.18 0.164+|

|   150     2268      2055    6.02  2.87      4.51   2.66 0.182 |

|   200     2133      1819   11.19  1.52      3.18   4.93 0.250 |

|~~250.00~~~2003~~~~~~1603~~~18.21~~0.00~~~~~~0.00~~~7.86~0.322~|

|- 294.14   1891      1430   26.16 -1.48     -4.56  11.12 0.390-|

|> 300      1877      1408   27.36 -1.69     -5.31  11.61 0.399<|

-----------------------------------------------------------------

Figure 9. Typical Range Table

The zero range, 250 yards, is highlighted on your screen; here,

it is shown thus: ~~~~~~. The symbols : :, + +, - -, and > < in-

dicate the following "special" ranges:

: :  Range for first crossing of the line of sight;

+ +  Range at which the maximum ordinate (height) occurs;

- -  Range at which the bullet is the same distance below

the line of sight;

> <  The maximum range you entered.

The zero range is always displayed, and the "special" ranges

are always computed and displayed. They are shown to two decimal

places to set them apart from the other intermediate ranges.

QBALL               Computing Ballistics Data                  11

> Ballistic Computations, Typical Firearm <

===================================

|   *** Range Table Output ***    |

|  ----------------------------   |

|  <R>edisplay Range Table        |

|  <P>rint Range Table            |

|  <S>ave Range Table to File     |

|  Slant <A>ngle of Fire Table    |

|  Display Trajectory <G>raphic   |

|  ----------------------------   |

|  <ESC> to Computations Menu     |

-----------------------------------

Figure 11. Range Table Output Menu.

At this point, you may <ESC>ape to the Computations Menu if,

for example, you wanted to change the range or other input data.

You may also redisplay the range table - press <R> - which will

put you back in the range table at the same place you were when

you <ESC>aped from it.

Slant Angle of Fire and Display Trajectory Graphic are discuss-

ed later on, in section 2.7 and section 2.8, respectively.

2.5 Printing Range Table Hardcopy.

Warning! Ensure that your printer is on, on-line, has suffi-

cient paper, and is ready to go before going any further!

Press <P> to print the range table on your printer. QBALL dis-

plays the following, recomputes all the table's data, and sends it

to the printer.

===================================

|   *** Range Table Output ***    |

|  ----------------------------   |

|  <===========================   |

|  <|                         |   |

|  <|  Printing Range Table   |   |

|  S|                         |   |

|  D---------------------------   |

|  ----------------------------   |

|  <ESC> to Computations Menu     |

-----------------------------------

Figure 12. Printing Range Table Information Display.

QBALL               Computing Ballistics Data                  13

2.6 Saving a Range Table to a Disk File.

You may save the range table to a file in/on the current

directory/disk. QBALL allows you to save up to 99 such files -

assuming sufficent disk space - numbered filename.R01 through

filename.R99. The default filename for range table files is always

RG_TABLE. Press <S>; an input box appears as shown in Figure 14.

===================================

|   *** Range Table Output ***    |

|  ----------------------------   |

===| Enter filename... |===========

|                                 |

|   Save as:      RG_TABLE.R01    |

|                                 |

-----------------------------------

|  ----------------------------   |

|  <ESC> to Computations Menu     |

-----------------------------------

Figure 14. Entering a Filename for a Range Table Disk File.

You may press <enter> to accept the displayed default, or over-

type an up-to-eight-character filename of your choice. You may not

use spaces or periods in your chosen filename. Edit your entry

using the standard editing keys; when it is exactly as you want

it, press <enter> to write the range table to the specified file.

Type FILENAME in the editing box, and press <enter>. (If filename

exists, QBALL will ask if a duplicate filename is acceptable.)

===� Enter filename... |===========

|                                 |

|   Saving as:    FILENAME.R01    |

|                                 |

-----------------------------------

Figure 15. Saving a Range Table to a Specified Disk File.

QBALL displays the selected filename as shown in Figure 15,

recomputes all the table data, writes the file, and returns to the

Range Table Output menu.

The format for all range tables saved to disk is identical to

the printed output shown in Figure 13. Range tables are saved in

plain-vanilla ASCII format, and may be manipulated by any editing

program or word processor which can handle pure ASCII. Addition-

ally, the supplied utility, RRTABLE.EXE, may be used to view or

print the saved table. See Chapter 5, section 5.6 for more inform-

ation about RRTABLE.EXE.

QBALL               Computing Ballistics Data                  15

2.8 Computing and Displaying a Slant Fire Table.

With the Range Table Output Menu displayed, press <A> to

develop a slant fire table using the data currently in effect. You

may use a vertical displacement expressed as either an elevation

or depression of the LOS, or as a linear distance above or below

the firing point.

---| Slant Fire Effects... |-------------------

|                                             |

|  Use <H>eight above/below firing point or   |

|  Vertical <A>ngle above/below horizontal:   |

|                                             |

-----------------------------------------------

Figure 17. Choosing Vertical Displacement Type.

Press <A> to use a vertical angle, and enter the elevation or

depression angle as shown. You may use an elevation or depression

angle of 0 to 60 degrees; for this example, use 30 degrees.

---| Slant Fire Effects... |-------------------

---| Enter Vertical Angle... |-------------------

|                                               |

|   Vertical angle      (degrees)          30   |

|                                               |

-------------------------------------------------

Figure 18. Entering the Angular Vertical Displacement.

QBALL displays the Slant Fire Table shown in Figure 19.

----------------------------------------------------------------

|            Slant Fire Table, LOS Elevated +/-30d             |

|-------------------------+------------------------------------|

|   For Horizontal Fire   |      For Slant Fire         Hits   |

|    Range    in +/-LOS   | Slant Range   in +/-LOS     High   |

|   -------   ---------   | ------------  ---------    ------  |

|       0        -1.00    |       0          -1.00      0.00   |

|:    13.60       0.00  : |     13.60         0.01      0.01   |

|      50         2.22    |      50           2.31      0.08   |

|     100         4.12    |     100           4.47      0.34   |

|+   136.78       4.56  + |    136.78         5.23      0.66   |

|     150         4.51    |     150           5.31      0.81   |

|     200         3.18    |     200           4.68      1.50   |

|    250.00       0.00    |    250.00         2.44      2.44   |

|-   294.14      -4.56  - |    294.14        -1.05      3.51   |

|>    300        -5.31  < |     300          -1.64      3.67   |

|                         |                                    |

--------------------------+-------------------------------------

Figure 19. Slant Fire Table, Angular Vertical Displacement.

QBALL               Computing Ballistics Data                  17

Press <V> to change the vertical displacement. When prompted

for the vertical displacement type, press <H>, then press <enter>

to accept the displayed value, 433 feet. The slant fire table

shown in Figure 21 is displayed.

----------------------------------------------------------------

|    Slant Fire Table, Impact 433 feet Above or Below Gun      |

|-------------------------+------------------------------------|

|   For Horizontal Fire   |      For Slant Fire         Hits   |

|    Range    in +/-LOS   | Slant Range   in +/-LOS     High   |

|   -------   ---------   | ------------  ---------    ------  |

|       0        -1.00    |       0          -1.00      0.00   |

|:    13.60       0.00  : |     13.60         0.01      0.01   |

|      50         2.22    |      50           2.31      0.08   |

|     100         4.12    |     100           4.47      0.34   |

|+   136.78       4.56  + |    136.78         5.23      0.66   |

|     150         4.51    |     150           5.31      0.81   |

|     200         3.18    |     200           4.68      1.50   |

|    250.00       0.00    |    250.00         2.44      2.44   |

|-   294.14      -4.56  - |    294.14        -1.05      3.51   |

|>    300        -5.31  < |     300          -1.64      3.67   |

|                         |                                    |

--------------------------+-------------------------------------

Figure 21. Slant Fire Table, Linear Vertical Displacement.

Press <ESC>ape to return to the Slant Fire Output menu.

2.9 Printing Slant Fire Table Hardcopy.

Warning! Ensure that your printer is on, on-line, has suffi-

cient paper, and is ready to go before going any further!

Press <P> to print the range table on your printer. QBALL dis-

plays the following, recomputes all the table's data, and sends it

to the printer.

=======================================

|  ***    Slant Table Output    ***   |

|  --------------------------------   |

|  =================================  |

|  |                               |  |

|  |   Printing Slant Fire Table   |  |

|  |                               |  |

|  --------------------------------|  |

|  <ESC> to Range Table Output Menu   |

---------------------------------------

Figure 22. Printing the Slant Fire Table.

Depending in part upon how long the slant fire table is, how

fast your printer can print, and the size of the printer or other

buffer, the slant fire table is printed, and you are returned to

the Slant Table Output menu. If you want another hardcopy, simply

press <P> to repeat the process.

QBALL               Computing Ballistics Data                  19

2.10 Saving a Slant Fire Table to a Disk File.

You may save the slant fire table to a file in/on the current

directory/disk. QBALL allows you to save up to 99 such files -

assuming sufficent disk space - numbered filename.R01 through

filename.R99. The default filename for slant fire table files is

always SF_TABLE. Press <S>; an input box appears as shown in

Figure 24.

===| Enter filename... |===========

|                                 |

|   Save as:      SF_TABLE.R02    |

|                                 |

-----------------------------------

Figure 24. Entering a Filename for a Slant Fire Table Disk File.

You may press <enter> to accept the displayed default, or over-

type an up-to-eight-character filename of your choice. You may not

use spaces or periods in your chosen filename. Edit your entry

using the standard editing keys; when it is exactly as you want

it, press <enter> to write the slant fire table to the specified

file. Type FILENAME in the editing box, and press <enter>. (If

filename exists, QBALL will ask if a duplicate filename is OK.)

===| Enter filename... |===========

|                                 |

|   Saving as:    FILENAME.R02    |

|                                 |

-----------------------------------

Figure 25. Saving a Slant Fire Table to a Specified Disk File.

QBALL displays the selected filename as shown in Figure 25,

recomputes all the table data, writes the file, and returns to the

Slant Table Output menu.

The format for all slant fire tables saved to disk is identical

to the printed output shown in Figure 23. Slant fire tables are

saved in plain-vanilla ASCII format, and may be manipulated by any

editing program or word processor which can handle pure ASCII. The

supplied utility, RRTABLE.EXE, may be used to view or print the

saved table. See Chapter 5, section 5.6 for more information about

RRTABLE.EXE.

| Special note applicable to Slant Fire Tables:          |

| Range tables and slant fire tables share the same file |

| series for saved-to-disk tables: filename.Rnn.  If you |

| wish to save both a range table and a slant fire table |

| for specific data, you are limited to 49 file pairs.   |

Press <ESC>ape to return to the Range Table Output Menu.

QBALL               Computing Ballistics Data                  21

========================    Press <S> to save the current trajec-

| Save as:  FILENAME   | tory data to a disk file. As with range

------------------------ and slant fire tables, you may specify

Figure 27. Filename    the filename. You may save up to 15 such

Entry Window.       files, filename.TJ1 through filename.TJF

-- that's 1 to 15 in hexadecimal nota-

tion. Type an up-to-eight-character filename, editing it with the

standard editing keys. When it is exactly as you want it, press

<enter> to write the file to disk. QBALL tells you that the data

is being saved, and returns to the graphics display screen.

=================    Note that the Graphics Menu now tells you

| <1> SAMPLE    | that two saved trajectory files are available.

| <2> FILENAME  | Press <R>, then <1> (one) to display the saved

----------------- SAMPLE.TJ1 trajectory file. QBALL reads the

Figure 28. Data  saved data, recomputes the trajectory, and dis-

Recall Window.  plays the graphic shown below in Figure 29.

If more than eight trajectories have been

saved, an arrow appears at the bottom right corner of the recall

window. Press that arrow key to access the remaining saved files.

//////////////////// (Top of screen omitted) ////////////////////

| |                                                             |

|N| +4              x                 x                     +4  |

| |                                        x                    |

| | +2    x   y                 y                           +2  |

|C|                                                             |

| |--0----|---------|-----------|----------y----------x------0--|

|x,y                                                            |

|H| -2      Use a pencil, draw smooth                       -2  |

| |         curves connecting the "x's"                         |

| | -4      and the "y's".                            y     -4  |

|E|                                                             x

| | -6                                                      -6  |

| |                                                             |

|S| -8                        YARDS                         -8  |

| |      50        100         150        200        250        |

--+-------------------------------------------------------------y

| Vel   2551      2407        2268       2133       2003        |

| Eng   2601      2315        2055       1819       1603        |

-----------------------------------------------------------------

Current         SAMPLE.TJ1   =========================

-------         ----------   | *** Graphics Menu *** |

Max Ht: 4.56 @ 137 Yds   2.58 @ 112 Yds | --------------------- |

Min Ht:-5.31 @ 300 Yds  -10.08 @ 300 Yds| Toggle <G>rid (Now on)|

C1/Wt : [ As Shown ]     .300/180 grains| <S>ave Current Data   |

M/TVel: 2700/1877 FPS    2700/1877 FPS  | <R>ecall <1>          |

M/TEng: 2913/1408 Ft#    2813/1408 Ft#  | <P>rint Graphic Screen|

ZeroRg: 250 Yds          200 Yds        | --------------------- |

Metro : [ As Shown ]     59F 0'29.53"78%| <ESC> to Rg Tble Out  |

-------------------------

MV: 2700| C1: .300 |G: 180 | Metro: 59d 0' 29.53" 78% | H: 1.00|E

Figure 29. Two Trajectories Displayed.

QBALL               Computing Ballistics Data                  23

**** Point-Blank Range.

Point-blank range is defined as that range for which the

bullet's path is within a specified distance above or below the

line of sight (LOS). For example, in the range table development

section, we found that the bullet was 4.56 inches above the LOS at

approximately 137 yards, and 4.56 below the LOS at approximately

294 yards. The point-blank range for a deviation of 4.56 inches

is, therefore, 294 yards. Generally, however, you'll want to

specify the deviation, and have QBALL find the ranges for you.

With the Other Procedures Menu displayed, press <P> to access

the point-blank routines.

Enter the maximum deviation which you wish to use; this example

uses 6 inches. (You a deviation from 0.01 inches up to 40 inches.)

> Ballistic Computations, Typical Firearm <

---| Finding Point Blank Range... |--------------

|                                               |

|   Maximum Deviation:      (inches)       6    |

|                                               |

|   Point Blank Zero Range:  (Yards)  278.42    |

|                                               |

|           QBALL will use 278 Yards.           |

|                                               |

|   Point Blank Max Range:   (Yards)  327.50    |

|                                               |

|           QBALL will use 327 Yards.           |

|                                               |

----------> press any key to continue <----------

Figure 31. Point Blank Range Determination.

QBALL computes the zero range necessary to achieve a trajectory

height of 6 inches above and the range at which the trajectory is

6 inches below the LOS, and displays the results as shown in

Figure 31.

Press any key to return to the Computations Menu. The calcu-

lated zero range is displayed.

=======================================

|    ***   Computations Menu   ***    |

|  ---------------------------------  |

|  Range <T>able (0 = 278 Yards)      |

|  Set or Reset Current <Z>ero Range  |

|  <O>ther Ballistics Procedures      |

|  <N>ew or Changed Input Data        |

|  ---------------------------------  |

|  <ESC> to  M A I N   M E N U        |

---------------------------------------

Figure 32. Point Blank Zero Range Validated.

QBALL               Computing Ballistics Data                  25

**** Zero for Range and Height

Suppose that you have measured a trajectory height at a known

range. Given this information, the ballistic characteristics, and

the muzzle velocity, QBALL can compute a most-probable zero range.

With the Other Procedures Menu displayed, press <Z> to find the

most probable zero for a given range and trajectory height.

Enter the range at which the height was measured and the height

in inches or centimeters, as appropriate. The example shown uses

100 yards and 4.12 inches.

---| Zero for Given Range and Height... |--------

|                                               |

|   Range:         (20 to 800 Yds)       100    |

|   Trajectory Height: (inches)         4.12    |

|                                               |

|   Most Probable Zero Rg:   (Yards)  249.94    |

|                                               |

----------� press any key to continue �----------

Figure 35. Entering Zero for Range and Height Data.

Press any key to return to the Computations Menu; you'll find

the zero range - in this case, 249.94 yards - is displayed. Press

<T> to develop a table. QBALL has calculated an appropriate maxi-

mum range; in this case use a range increment of 50 yards to

develop the following range table. (You may use any range incre-

ment; the given range you entered will be displayed in the range

table to 2 decimal places, e.g., 100.00.)

--------------------------------------------------------------

|           Range Table, Zeroed at 249.94 Yards              |

|------------------------------------------------------------|

|Range R e m a i n i n g  Drop  Path/Sight Line 10 MPH Time  |

|Yards V: FPS  E: Ft-Lbs Inches M.O.A.   Inches Inches Flght |

|----- ----------------- ------ --------------- ------ ----- |

|    0   2700      2913    0.00   --      -1.00   0.00 0.000 |

|: 13.58 2659      2826    0.04 -0.01      0.00   0.00 0.015:|

|   50   2551      2601    0.62  4.24      2.22   0.28 0.057 |

|~100.00~2407~~~~~~2315~~~~2.56~~3.94~~~~~~4.12~~~1.11~0.117~|

////////////////// data lines omitted ////////////////////////

| 249.94 2003      1603   18.20  0.00      0.00   7.86 0.322 |

////////////////// data lines omitted ////////////////////////

--------------------------------------------------------------

Figure 36. Range Table Using Most Probable Zero.

Here, the range you entered, 100 yards, is shown ~~~~~~; it is

not otherwise emphasized on your screen display.

QBALL               Computing Ballistics Data                  27

In this case, press <Y> or <enter>; you are returned to the

Computations Menu. Now press <T>, <enter>, <enter> to display the

range table shown in part in Figure 39.

--------------------------------------------------------------

|           Range Table, Zeroed at 249.94 Yards              |

|------------------------------------------------------------|

|Range R e m a i n i n g  Drop  Path/Sight Line 16 MPH Time  |

|Yards V: FPS  E: Ft-Lbs Inches M.O.A.   Inches Inches Flght |

|----- ----------------- ------ --------------- ------ ----- |

|    0   2700      2913    0.00   --      -1.00   0.00 0.000 |

//////////////////////////////////////////////////////////////

Several data lines omitted

//////////////////////////////////////////////////////////////

|>300.00 1877      1408   27.36 -1.69     -5.31  18.58 0.399<|

-------------------------------------------------------------X

Figure 39. Range Table Showing Nonstandard Wind Deflection.

As you can see, the wind deflection column header now displays

the new crosswind, and the deflection values have increased to

reflect the wind speed change.

You may enter wind speeds of 0 MPH to 235 MPH, the world's

record wind speed, set on Mt. Washington, New Hampshire, U.S.A.,

in 1935. Wind directions may be entered in whole clockwise degrees

from 0 (headwind), through 90 (right to left), 180 (tailwind), 270

(left to right), up to 360. QBALL will convert values greater than

360 degrees to a value less than a full circle.

**** Sectional Density.

The sectional densities of two bullets may be used to compare

their relative penetrating power. Sectional density is defined as

the bullet weight in pounds divided by the square of its diameter

in inches.

At the Other Procedures Menu, press <S>, and enter the bullet

diameter in inches or millimeters. You may use the weapon bore

diameter if you do not have a specific bullet diameter; QBALL

assumes that any entered value greater than 1.000 is in milli-

meters and makes the conversion for you. (If you have used bullet

lookup, the diameter is displayed. Simply press <enter> to display

the sectional density.) Enter .308 as the bullet diameter.

---| Finding Sectional Density... |--------------

|                                               |

|   Bullet diameter:        (inches)    .308    |

|                                               |

|           Sectional Density = .271            |

|                                               |

----------> press any key to continue <----------

Figure 40. Entering a Bullet Diameter, Display Sectional Density.

QBALL               Computing Ballistics Data                  29

Comparing the data in Table 1 with that for our example weapon,

it is seen that the example's recoil energy falls between that of

a "classic deer rifle" and of a heavy military rifle. All things

equal, this gives some idea of how hard the example weapon kicks.

To assess perceived recoil, however, one must also take into

account the individual shooter's physical characteristics and

clothing worn, and the weapon's stock shape, among others. Since

none of these is quantifiable, any evaluation of recoil based on

these or similar calculations must be treated as a rough approx-

imation, only. The figures are valid for comparison purposes, but

nothing more!

2.13 New or Changed Input Data.

From time to time, you may wish to change some or all of the

data which you've given QBALL. If you wish to change everything,

i.e., start with completely new data, just as if you had newly-

started QBALL, then <ESC>ape all the way back to the Main Menu,

and press <C>. The following prompt will appear. Press <N> - you

do <n>ot want to keep the displayed data. (If you've changed your

mind, then press <Y> or <enter> to retain all of it, and change

individual items.)

===| Clean Sweep... |============================

|                                               |

|       Retain Displayed Data? ([Y] or N)       |

|                                               |

-------------------------------------------------

Figure 42. A Clean Sweep Prompt.

You may also change individual inputs. From the Computations

Menu, press <N> to access the New Input Data Menu.

=======================================

|     *** New Input Data Menu ***     |

|  ---------------------------------  |

|  <W>eapon Identifier (30 chars)     |

|  <B>ullet Identifier (15 chars)     |

|  Muzzle <V>elocity                  |

|  Ballistic <C>oefficient            |

|  Bullet wei<G>ht                    |

|  <M>etro (Temp GunAlt Press Humid)  |

|  Sight <H>eight                     |

|  Unit System <S>wap (Now English)   |

|  ---------------------------------  |

|  <ESC> to Computations Menu         |

---------------------------------------

Figure 43. New Input Data Menu.

QBALL               Computing Ballistics Data                  31

**** Metro. (Meteorological Conditions).

Press <M>. You must either enter or confirm air temperature,

gun altitude, barometric pressure, and relative humidity. Press

<enter> to accept the displayed value, or overtype and press

<enter> to change it. Standard values are shown in [brackets] on

the prompt line. You may save the new metro data for use as the

startup default; see Chapter 5.

Allowable metro data values are as follows, all inclusive:

Temperature. . . . . . . .-50 degrees to +125 degrees Fahrenheit

Gun Altitude . . . . . . . . . . . . . -1312 feet to 29,028 feet

Barometric Pressure. . . . . . 25 inches to 35 inches of mercury

Relative Humidity. . . . . . . . . . . . . . . . . . .0% to 100%

---| Enter New/Changed Metro... |----------------

|                                               |

|   Temperature               [59dF]      59    |

|   Gun Altitude            [0 feet]       0    |

|   Barometric pressure  [29.53" Hg]   29.53    |

|   Relative Humidity          [78%]:     78    |

|                                               |

-------------------------------------------------

Figure 46. Entering Metro (Meteorological Conditions).

**** Sight Height.

Sight height, the vertical distance between the axis of the

weapon's bore and the imaginary line connecting the front and rear

sights (or the 'scope sight axis), should be fine-tuned to a spec-

ific weapon. Press <H> and enter a new sight height from -2 to +10

inches, inclusive. QBALL rounds sight height to two decimal

places, e.g., 1.375 inches is rounded to 1.38 inches. Sight height

may be saved for use in the QBALL startup data; see Chapter 5.

---| Enter New/Changed Sight Height... |---------

|                                               |

|   Sight height:        (inches)          1    |

|                                               |

-------------------------------------------------

Figure 47. Entering a Sight Height.

**** Unit System Swap.

As supplied, QBALL defaults to English measure: yards, feet,

inches, pounds, grains, foot-pounds, and Fahrenheit temperatures.

If you wish to use Metric measure: meters, centimeters, grams,

kilograms, meter-kilograms, and Celsius temperatures, press <S> to

swap the unit system. To switch to English measure, press <S>

again. You may save "metric" as the QBALL default; see Chapter 5.

See also section 2.15, valid data limits and conversion factors.

QBALL               Computing Ballistics Data                  33

With the highlight bar positioned as shown ~~~~thus~~~~,

press <enter>. QBALL reads the data for bullet #1 of 505 bullets:

the Mak(er), Dia(meter), Weight, C1 (Ballistic Coefficient), and

Bullet Type. The screen header and information line are changed,

thus:

> Ballistic Computations, Typical Firearm, .172 HDY HP <

------------------------------------------------

MV: 2700 | C1: .190 | G: 25| Metro: 59d 0' 29.53" 78%| H: 1.00 |E

Figure 50. Changed Screen Header and Information Lines.

QBALL also reads the bullet's sectional density, which will now

appear in printed and saved table headers. In the supplied list,

"remarks" indicate the maker's stock number. This information, as

well as everything else, may be changed using the QBALL Bullet

Lookup Maintenance utility, supplied to all registered users.

If you use the bullet lookup facility, both the ballistic

coefficient and the bullet weight are always changed. If you wish

to change one, but not the other, enter the appropriate value

manually.

Bullet makers (manufacturers) are abbreviated as follows:

HDY = Hornady        LYM = Lyman          NOS = Nosler

SRA = Sierra         SPR = Speer

Bullet type abbreviations preceded by a lower-case "c" are cast

bullets. The true bullet weight will depend upon the alloy from

which that bullet is cast; the given weight is, however, a work-

able first approximation.

Bullet type abbreviations followed by the lower-case characters

"gc" are equipped with a gas check.

Section 2.15 follows on page 34.

QBALL                Inferred Ballistics Data                   35

Chapter 3. Inferred Ballistics Data

From time to time, you may need to determine the ballistic

coefficient for a given bullet for which that particular inform-

ation is unavailable. (Many manufacturers are singularly reluctant

to divulge it!) Similarly, you may wish to determine a muzzle vel-

ocity for a particular load/weapon combination. QBALL can deter-

mine that data for you.

At the Main Menu, press <I> to access the Infer BC or MV menu.

> Ballistic Computations, Typical Firearm <

===================================

|  ***    Infer BC or MV    ***   |

|  ----------------------------   |

|  Ballistic <C>oefficient        |

|  Muzzle <V>elocity              |

|  ----------------------------   |

|  <ESC>  to  M A I N   M E N U   |

-----------------------------------

MV:     |C1:     |G:     |Metro:  59d 0' 29.53" 78%|H: 1.00|E

Figure 51. Infer BC or MV Menu Display.

3.1 Inferring a Ballistic Coefficient.

Press <C> to infer a ballistic coefficient.

QBALL will ask for two ranges and two velocities. The schematic

layout of a firing range as viewed from above, shown in Figure 52,

will help you visualize the setup. (Two chronographs are shown;

see the note on page 38 if only one chronograph is available.)

It is possible for  R1             Near            Far

to equal  zero,  in which            C h r o n o g r a p h

case, V@1 equals the muz-               _               _

zle velocity. This is the      Gun >>>>|_|>>>>>>>>>>>>>|_|>>>>>>>

only time that  QBALL al-              R1              R2

lows you to enter a range              V@1             V@2

of zero;  "zero"  is  the

default value for R1.            Figure 52. Range Setup Schematic

QBALL                Inferred Ballistics Data                   37

The following is displayed:

---------------------

-| Ballistic Coefficient, 2 Velocities... |-| #    V1      V2   |

|                                          |+-------------------+

|                                          || 1  2700.0  2407.0 |

|        Using 1 round, QBALL finds:       || 2                 |

|                                          || 3                 |

| Between ranges Muzzle and 100 yards,     || 4                 |

|                                          || 5                 |

| Avg Higher Velocity        2700 FPS      || 6                 |

| Avg Lower Velocity         2407 FPS      || 7                 |

| Ballistic Coefficient      .300          || 8                 |

|                                          || 9                 |

| Muzzle Velocity taken as 2700 FPS        ||10                 |

|                                          |+-------------------+

|                                          ||Av  2700.00 2407.00|

|                                          ||sV  ......  ...... |

|                                          ||C1, Last Pair: .300|

-------> press any key to continue <-----------------------------

Figure 54. Velocity Data, MV and Calculated C1 Displayed.

Press any key, or wait 60 seconds for QBALL to continue auto-

matically. If all the required initial data is now available,

you'll be returned to the Computations Menu, otherwise, QBALL asks

for the data it needs.

**** Ballistic Coefficient, R1 = 25.

Use this procedure if you have determined two downrange veloc-

ities from live firing. You can, of course, use two tabulated

velocities if you so desire.

Press <ESC>ape several times to return to the Main Menu, then

select <I>nfer... and Ballistic <C>oefficient.

Enter the following data:

Shorter Range (R1). . . . . . . . .  25

Longer Range (R2) . . . . . . . . . 100

Velocity at R1 (V1) . . . . . . .  2625

Velocity at R2 (V2) . . . . . . .  2407

For illustration purposes, let's reuse the same data; press

<enter> six times, then press <N>.

QBALL displays the screen represented in Figure 55 (next page).

QBALL                Inferred Ballistics Data                   39

3.2 Inferring a Muzzle Velocity.

Assume that you've obtained a supply of ammunition, and that

you know the ballistic coefficient (C1) for the bullet with which

it is loaded. QBALL provides two methods of obtaining a muzzle

velocity from live firing: the first from a measured bullet drop

at a known range; the second from measurement of a single down-

range velocity.

At the Infer BC or MV menu, press <V> to display the Infer MV

using menu.

> Ballistic Computations, Typical Firearm <

===================================

|  ***    Infer MV using    ***   |

|  ----------------------------   |

|  Bullet <D>rop Method           |

|  One Downrange <V>elocity       |

|  ----------------------------   |

|  <ESC> to Infer BC or MV Menu   |

-----------------------------------

MV:     |C1:     |G:     |Metro:  59d 0' 29.53" 78%|H: 1.00|E

Figure 56. Inferring Muzzle Velocity Using Menu Display.

Use the bullet drop method if you do not have a chronograph.

If, however, you have, or have access to, a chronograph, use the

downrange velocity method.

Caution! The bullet drop method is extremely sensitive to

errors in measurement. If you have a choice, use the downrange

velocity (chronograph) method.

**** Muzzle Velocity Using the Bullet Drop Method.

Figure 57 shows in exaggerated form     |    x

what you'll be measuring. The x's in-   x---------x-------------

dicate the bullet path  for a weapon    |         |        |

zeroed at  50 yards;  draw a  smooth    |         |        |  D

curve through the x's. At 100 yrads,    |         |        |

then,  the bullet will strike a dis-    |         |        x ---

tance D below the line of sight.  If    0         50      100

D can be found, and C1 is known, the

required MV can be calculated.           Figure 57. Bullet Drop

Method Schematic

QBALL                Inferred Ballistics Data                   41

Measuring vertical displacements is relatively simple if you

have a scale graduated in fiftieths or hundredths of an inch.

(Such a scale is, however, somewhat difficult to read.) If you

don't have such a scale, the following fraction table may be

helpful.

-------------------------------------------------------

|  1/64=.01563 17/64=.26563 33/64=.51563 49/64=.76563 |

|  1/32=.03125  9/32=.28125 17/32=.53125 25/32=.78125 |

|  3/64=.04688 19/64=.29688 35/64=.54688 51/64=.79688 |

|  1/16=.0625   5/16=.3125   9/16=.5625  13/16=.8125  |

|  5/64=.07813 21/64=.32813 37/64=.57813 53/64=.82813 |

|  3/32=.09375 11/32=.34375 19/32=.59375 27/32=.84375 |

|  7/64=.10938 23/64=.35938 39/64=.60938 55/64=.85938 |

|  1/8 =.125    3/8 =.375    5/8 =.625    7/8 =.875   |

|  9/64=.14063 25/64=.39063 41/64=.64063 57/64=.89063 |

|  5/32=.15625 13/32=.40625 21/32=.65625 29/32=.90625 |

| 11/64=.17188 27/64=.42188 43/64=.67188 59/64=.92188 |

|  3/16=.1875   7/16=.4375  11/16=.6875  15/16=.9375  |

| 13/64=.20313 29/64=.45313 45/64=.70313 61/64=.95313 |

|  7/32=.21875 15/32=.46875 23/32=.71875 31/32=.96875 |

| 15/64=.23438 31/64=.48438 47/64=.73438 63/64=.98438 |

|  1/4 =.25     1/2 =.5      3/4 =.75                 |

-------------------------------------------------------

Table 3. Decimal Equivalents, 1/64 Inch Measure.

Here's one way to develop a sufficiently-accurate value for the

average bullet drop for a three-round string.

Assume you've zeroed your  weap-                 |

on at 50 yards,  and have  fired  3                 |

rounds at 100 yards,  producing the        ---------0------------

group shown in Figure 59. Carefully                 |    |  |  |

measure  each drop,  D1 through D3,                 |    D1 |  |

to the nearest 1/64 inch.                           |    |  D2 |

|  O -- |  |

Suppose the measurements are as                  |       |  D3

follows:                                            |    O --- |

|          |

D1 =    1/8  = 0.12500"                         |          |

D2 = 1  7/32 = 1.21875"                         | O ---------

D3 = 1 11/32 = 1.34375"

Figure 59. Measurements

Add the three  drop measurements        to Find Group Center.

together, obtaining 2.86750; divide

this by the number of rounds, in this case, three. The drop to the

center of the group is, then, 0.89563(+). Round this to 0.90 (or

0.896) for use as the vertical difference in this procedure. QBALL

will accept vertical differences to the nearest 0.001".

Hint: use a scoring plug and measure from the horizontal line

through the point of aim to the top of the plug, and add half the

bullet diameter to obtain the vertical distance.

QBALL        Printed Output and Printer Configuration           43

Chapter 4. Printed Output and Printer Configuration

Many of the QBALL computations can be printed on a properly-

configured printer. In general, text output � range tables, for

example � may be printed without the need for specifically con-

figuring your printer. Printing graphics screens, however, does

require making use of the PRCONFIG.EXE utility.

4.1 Printer Warnings.

QBALL is internally configured to print to LPT1, only. If your

printer is connected to a different parallel port, you will need

to "swap" the ports for the duration of the QBALL session, or

perhaps even reconfigure your hardware setup.

QBALL will detect if your printer is or is not on/online; it

will not detect "paper out."

IMPORTANT: Ensure that your printer is properly set up, turned

on, is on line, and has paper loaded before you try to print any

QBALL output!!

Some "generic" printer buffers did not respond properly to

earlier versions of QBALL. If you are using a printer buffer, and

keep getting an error message, then try bypassing the buffer. The

buffers built into printers do not seem to cause any problem.

QBALL internally disables the graphics print routines until

such time as an appropriate printer driver is chosen using

PRCONFIG.EXE.

4.2 Screen Dumps (Shift-PrintScr).

It is possible to print out one screen's worth of tabular

information using the Shift-PrintScr key combination available in

dos. This is not recommended, however, as you may very well lose

information which has already scrolled past on the screen. (You

may use this screen dump method for graphics screens; see your DOS

documentation, probably under "Graphics" for details. Note that

this method works best with tractor-feed paper, and QBALL config-

ured with a black-and-white screen.)

4.3 Printer Setup.

Do not set up your printer to skip the perforations in fan-fold

paper; QBALL keeps track of the number of lines printed and issues

a formfeed when appropriate, including at the end of each table.

If you have a choice, set the left margin to "0," as QBALL indents

the left side of all printed output an appropriate number of

spaces.

You may also toggle the printer completely off by using the ap-

propriate QBALL configuration routines, as explained in Chapter 5.

QBALL        Printed Output and Printer Configuration           45

The following list of Epson printers is displayed.

> QBALL Graphics Printer Selection <

===| EPSON printers... |=====================================

|     Printer Model                B&W/Col  Resolution      |

|     -------------                -------  ----------      |

|  LQ, SQ, or Action Printer Models   B&W     60 x  60      |

|  LQ, SQ, or Action Printer Models   B&W    120 x  60      |

|  LQ, SQ, or Action Printer Models   B&W    180 x 180      |

|~~LQ,~SQ,~or~Action~Printer~Models~~~B&W~~~~360 x 360~~~~~~|

|  LQ, SQ, or Action Printer Models  Color   180 x 180      |

|  LQ, SQ, or Action Printer Models  Color   360 x 360      |

|  EPL-6000/7000/7500                 B&W     75 x  75      |

|  EPL-6000/7000/7500                 B&W    100 x 100      |

|  EPL-6000/7000/7500                 B&W    150 x 150      |

|  EPL-6000/7000/7500                 B&W    300 x 300      |

|  MX, FX, RX, JX, LX, and DFX        B&W     60 x  72      |

|  MX, FX, RX, JX, LX, and DFX        B&W    120 x  72      |

|  FX, RX, JX, LX, and DFX            B&W    120 x 216      |

|  FX, RX, JX, LX, and DFX            B&W    240 x 216      |

-------------------------------------------------------------

PgUp PgDn <up><dn> Home End [ESC]

Figure 62. Choosing a Printer Model.

Again, use <PgUp>/<PgDn>, arrow, or the <home>/<end> keys to

position the highlight bar over the desired printer model, and

press <enter> to choose that model. You may exit without making

any choices by pressing <ESC>ape.

Here, the highlighted model is any one of several different

Epson printers which feature black and white (B&W) output and a

printing resolution of 360 dpi by 360 dpi. Press <enter> to choose

this printer model. The following prompt box is displayed:

=============================================================

|                                                           |

|     QBALL is currently configured to use P_HOLDER.PRD     |

|                                                           |

|              You have selected: EPSON2VH.PRD              |

|                                                           |

|     Reconfigure QBALL to use EPSON2VH.PRD ? ([Y] or N)    |

|                                                           |

-------------------------------------------------------------

Figure 63. Confirming Reconfiguration.

Press <Y> or <enter> to write the new or replacement printer

driver to the QBALL default configuration data; press N to leave

that data unchanged. In this case, you'd most likely press <Y>.

P_HOLDER.PRD is the as-distributed driver name, and must be re-

placed in order to make the graphics print feature usable. All the

available drivers are listed in the text file PRINTERS.DAT.

QBALL        Printed Output and Printer Configuration           47

Note that all these alternatives specify "lowest available

resolution." If you want a higher-resolution printout, then simply

choose the next higher resolution and try it.

****  Something Prints. This is probably the most maddening demon

to track down and exorcize. Here are some symptoms and

suggestions.

The printout begins fine, then breaks down and prints garbage.

This is often caused by a hardware problem which prevents the

printer from reporting a buffer full condition. Replace the

printer cable with a new one less than eight feet long, bypassing

any external print buffers and printer switches.

The printout has extra line feeds. This usually occurs on older

printers or on IBM graphics printers. The printer has probably

missed or misinterpreted the line spacing command. Make sure the

printer is on-line before attempting to print. If you're using an

IBM printer, choose an IBM driver.

The printout is jagged. The printer itself cannot print at the

called-for resolution. Choose a lower-resolution driver.

A black-and-white printout omits colors (usually yellow), or

some areas of the printout have a shaded background. Configure the

QBALL screen colors to black-and-white. Either start QBALL with

the command QBALL /b or use the screen color routines in the

built-in utilities as described in Chapter 5.

The printout is on multiple pages. This may caused by the same

problem that causes the printer to start out fine, then begin

printing garbage. Replace the printer cable with a new one less

than eight feet long, bypassing any external print buffers and

printer switches.

A laser printer may not have sufficient built-in memory to

print at the called-for resolution. Choose a lower-resolution

driver, or add memory to your printer.

-----------------------------------------------------------------

| The material on pages 46 and 47 is adapted from documentation |

| provided by Genus Microprogramming, Inc., 1155 Dairy Ashford, |

| Suite 200, Houston, TX 77079-3012.  Acknowedgment is made for |

| this and other Genus Microprogramming material.               |

-----------------------------------------------------------------

QBALL   Configuring Display and Defaults, Using Utilities      49

Chapter 5. Configuring Display and Defaults, Using Utilities

5.1 The Utilities and Configuration Menu.

Should you wish to change some or all of the QBALL startup data

or its current configuration, invoke a DOS shell, or read a saved

range or slant fire table, press <U> at the Main Menu to display

this Utilities Menu.

=================================

|  ***   Utilities Menu   ***   |

|  --------------------------   |

|  Toggle <S>witches            |

|  Change Display <C>olors      |

|  <W>rite QBALL .INI Data      |

|  Invoke a <D>OS Shell         |

|  <R>ead a Range Table File    |

|  --------------------------   |

|  <ESC> to  M A I N   M E N U  |

---------------------------------

Figure 64. Utilities Menu.

Press <S> to change several of the QBALL operating settings.

Press <C> to choose the display colors. Press <W> to save the

current configuration. Press <D> to drop to the operating system

without exiting QBALL. If you have saved any range tables or slant

fire tables to a disk file, and if the RRTABLE.EXE utility is in

the current directory, press <R> to "shell out" to the utility.

5.2 Toggle Switches.

Press <S> to access the Switch Toggle Menu.

=======================================

|  ***    Switch Toggle Menu    ***   |

|  --------------------------------   |

|  Unit System <S>wap (Now English)   |

|  Graphics <G>rid    (Now ON)        |

|  <P>rinter on/off   (Now ON)        |

|  <N>oises on/off    (Now ON)        |

|  --------------------------------   |

|  <ESC> to Utilities Menu            |

---------------------------------------

Figure 65. Switch Toggle Menu, Supplied Defaults Shown.

Press <S> to swap between English and Metric measure. The as-

supplied system is English measure: yards, inches, grains, etc.

Press <G> to toggle the initial graphics grid on or off; you

may still toggle the grid when displaying graphics.

Press <P> to toggle the printer. As distributed, QBALL looks

for a printer. If you do not have a printer, or you do not want

hardcopy, toggle the printer "off."

QBALL uses "alert" sounds. If you like peace and quiet, toggle

<N>oises "off."

QBALL   Configuring Display and Defaults, Using Utilities      51

If you're sure of the configuration data, simply press <W>

immediately. Remember, however, that the QBALL initialization data

includes a bit more information than the toggle status and screen

colors. It also controls the

a. Weapon identifier

(but not the bullet make/type);

b. Metro conditions; and

c. the Sight height.

This allows you to "tailor" your copy of QBALL to match your

own weapon, and the actual metro encountered at your firing range.

For example, if your range is at an altitude of 650 feet above sea

level, and you're using, say, a Ruger M77 rifle in .270 caliber

with a sight height of 1.25 inches, that data can be saved for

QBALL to use, saving you the trouble of repeatedly entering it.

5.5 Invoking a DOS Shell.

At the Utilities Menu, press <D> to invoke a DOS shell, i.e.,

access the operating system without exiting QBALL.

When started, QBALL writes a special batch file, QBSHELIT.BAT,

in the current directory. When the <D> option is chosen, QBALL

calls this batch file, which sets the DOS prompt like this:

------------------------------------------------

|   [ Exit >> QBALL ] [drive]\[directory] >_   |

------------------------------------------------

Figure 67. DOS Prompt in Shell.

QBALL then loads a second command processor, COMMAND.COM.

If you try to start QBALL from within its own shell, the

following message is displayed:

----------------------------

| QBALL is already loaded! |

| ------------------------ |

| If not in a QBALL shell, |

| [DEL]ete: "QBSHELIT.BAT" |

| from current directory.  |

| ...press any key to end. |

----------------------------

Figure 68. Already Loaded Error Message.

QBSHELIT.BAT is erased upon normal exit from QBALL. If, for

some reason, the already loaded message appears when you're trying

to start QBALL in the first place, simply delete QBSHELIT.BAT and

restart "QBALL."

When you have finished using the shell, type "exit" (+ <enter>)

to return to the QBALL session.

QBALL   Configuring Display and Defaults, Using Utilities      53

At this point, however, press <1> (+ <enter>); RRTABLE displays

the following:

========================================

Ballistic Computations = Typical Firearm

========================================

Muzzle Velocity    =2700 FPS        Ballistic Coef'nt=.300

Bullet Weight      =180 grains      Sectional Density=n/a

Temperature        =59 degF.        Altitude at gun  =0 feet

Barometric Pressure=29.53 inches Hg Relative Humidity=78 %

========================================

Range Table, Zeroed at 250 Yards

--------------------------------------------------------------

Range  R e m a i n i n g  Drop   Path/Sight Line 10 MPH Time

Yards  V: FPS  E: Ft-Lbs Inches  M.O.A.   Inches Inches Flght

-----  ----------------- ------  --------------- ------ -----

0   2700      2913     0.00    --      -1.00   0.00  0.000

////////////// Several lines omitted ////////////////////////////

> 300   1877      1408    27.36     -1.69  -5.31  11.61  0.399 <

[#01]                  <P>rint  [ESC] exits

Figure 71. Typical RRTABLE Display.

You may make hardcopy of the table by pressing <P>; the re-

sulting output will be identical to the sample shown in Figure 13,

page 12.

IMPORTANT: Ensure that your printer is properly set up, turned

on, is on line, and has paper loaded before you try to print any

QBALL output!!

Press <ESC>ape to exit; RRTABLE will ask you if you want to

read another table. Press <Y> or <enter> to return to the listing

display shown in Figure 70, or <N> to return to QBALL.

If the table you've chosen to read is too long to fit on one

screen, the menu line at the bottom will look like this:

---------------------------------------------------------------

| [#nn]         PgUp PgDn Home End | |  <P>rint  [ESC] exits  |

---------------------------------------------------------------

Figure 72. "Long Table" Menu Line.

Press the appropriate key to navigate through the table; <P>

and <ESC>ape function as previously described.

If you run RRTABLE.EXE from the command line, and there are no

saved tables, RRTABLE will tell you that it has found no tables,

and automatically exits after a short pause. The choice keys

otherwise operate the same, except that exiting will be to DOS,

rather than to QBALL.

QBALL             Recalling Saved Trajectory Data               55

Chapter 6. Recalling Saved Trajectory Data

If one or more saved trajectory files - [FILENAME].TJn - exist

on/in the current disk/directory, you may access them directly

from the Main Menu. For this example, it is assumed that you have

two such files: the supplied SAMPLE.TJ1 and FILENAME.TJ2, which

you saved earlier. At the Main Menu, press <R> to recall the saved

trajectory data. The following is displayed:

-| Recalled Trajectory: SAMPLE.TJ1... |-------------------------

|                     * Typical Firearm *                      |

|Ballistic Coefficient: .300        Zeroed Range   :  200 Yards|

|Bullet Weight        :  180 grains Maximum Range  :  300 Yards|

|Muzzle Velocity      :  2700 FPS   Range Increment:   50 Yards|

|Sight Height         :  1.00"      Point-blank    :   .....   |

|                 Metro: 59dF. 0' 29.53" 78%                   |

----------------------------------------------------------------

=====================================

|  *** Recall Trajectory Data ***   |

|  ------------------------------   |

|  <R>ecall <1> thru <2>            |

|  Display Trajectory <G>raphic     |

|  Use Displayed as <D>efault       |

|  ------------------------------   |

|  <ESC> to  M A I N   M E N U      |

-------------------------------------

Figure 73. Recall Trajectory Data Display and Menu.

The saved [filename].TJ1 is always displayed first. The inform-

ation shown above the menu provides enough information to identify

the file. You may access the others by pressing <R>; a choice box

is displayed thus:

=====================================

|  *** Recall Trajectory Data ***   |

|  ------------------------------   |

|  <R>ecall <1> thru <2>            |

===| Recall... |==  Display Trajectory <G>raphic     |

|  <1> SAMPLE    |  Use Displayed as <D>efault       |

|  <2> FILENAME  |  ------------------------------   |

------------------  <ESC> to  M A I N   M E N U      |

-------------------------------------

Figure 74. Recall Trajectory Data Menu and Choice Box.

In this example, only two data files are available. You may

save as many as fifteen such files, numbered 1 through 9 and A

through F. Press the highlighted number or letter to select the

file. For this example, press <2>. The information box is updated

to reflect the data read from FILENAME.TJ2 - the one you saved.

Erase the choice box without selecting another file by pressing

<ESC>ape.

If you decide not to use any of the saved data, simply <ESC>ape

to the Main Menu and proceed from there to enter and compute data.

----------

APPENDICES

----------

QBALL                  Appendix A - Glossary                    57

absolute zero - Properly, that temperature at which all molecular

motion ceases. For practical purposes, -459.67 deg. Fahrenheit

(0 deg. Rankine), or -273 deg. Celsius, (0 deg. Kelvin).

air density - The mass (or loosely, the weight) of a unit volume

of air.

air resistance - The force exerted upon a projectile by the at-

mosphere tending to retard that projectile's forward flight.

angle of elevation - The vertical angle measured between the line

of sight and the line of departure.

altitude - The vertical distance above or below sea level. Alti-

tudes above sea level are taken as positive; those below, as

negative.

ballistics - The study of moving projectiles. Commonly, what is

meant is exterior ballistics: that portion of the bullet's

flight between the muzzle and the point of impact. (Internal

ballistics studies events prior to the bullet's exit from the

muzzle; terminal ballistics, the interaction between the bullet

and the target.)

ballistic coefficient - The ratio of the sectional density of a

projectile to its coefficient of form. A measure of how well

a bullet retains its velocity. Commonly supplied by reload-

ing manufacturers, or it may be derived from their ballistic

tables. In QBALL, the ballistic coefficient is always taken

at standard metro. The program corrects for nonstandard con-

ditions, but does not display the corrected ballistic coeff-

icient(s). See also: standard metro.

ballistic efficiency - The relative ability of a bullet in flight

to overcome air resistance.

barrel - That portion of a firearm which guides the bullet immed-

iately after propellant ignition.

bore - The inside of a firearm's barrel. In rifled firearms, the

bore diameter is the original dimension of the barrel before

the rifling grooves are cut or swaged in it. Bore diameter may

also be taken as the transverse dimension between the lands of

the rifling. See also: grooves, lands, rifling.

brass - 1. Alloy of copper and zinc commonly used in the fabrica-

tion of cartridge cases. 2. One or more cartridge cases con-

taining neither powder, unfired primer, nor bullet.

bullet - The actual projectile in small arms ammunition. Commonly

corrupted to refer to the entire cartridge.

QBALL                  Appendix A - Glossary                    59

elevation - 1: Vertical sight adjustment. 2: The (usually small)

angle between the axis of a firearm's bore and the horizontal.

energy - Here, the kinetic energy of a moving bullet, expressed in

FtLbs (English measure) or MetKg (Metric measure). Calculated

by multiplying the bullet mass by the square of the velocity,

and dividing by two.

FPS (also F.P.S. and Ft/Sec) - Feet per second. A unit of velo-

city.

FtLbs (also Ft.-Lbs. and Ft#) - Foot pounds. The energy required

to lift one pound a vertical distance of one foot in a standard

gravitational field.

form factor - A multiplier which relates a bullet's shape to that

of the standard projectile used to prepare a particular ballis-

tic table.

G1 functions - Table of ballistics functions based upon the Gavre

Commission tests in France, 1898.

grain - 1: A weight unit equal to 1/7000 of a pound, 0.002286

ounces, or (approximately) 0.0649 grams. 2: An individual part-

icle, or kernel, of powder. In load specifications, always re-

fers to the weight of the powder charge, never to individual

particles.

gram - In Metric measure, 1/1000 kilogram, approximately 15.4

grains (weight).

grooves - Swaged impressions or cuts spiraled through a bore to

impart spin to projectiles. See also: lands, rifling.

group - The pattern made at the target by a number of shots fired

at a single point of aim and (usually) a common sight setting.

Usually measured from center-to-center of the two most widely

spaced points of impact.

gun - In popular usage, refers to firearms, generally. Properly, a

type of artillery or naval ordnance.

handgun - A firearm designed to be held and fired with one hand.

hold off - The distance a shooter must shift his point of aim to

the left or right to compensate for wind deflection.

hold over - The distance a shooter must raise his point of aim to

be on target when his firearm is zeroed at a lesser range. Thus

by extension, hold under is the distance a shooter must lower

his point of aim to compensate for ranges less than the zeroed

range, or to compensate for slant fire effects. See also: slant

fire.

QBALL                  Appendix A - Glossary                    61

MetKg (also Met-Kg) - Meter kilogram. The energy required to lift

a mass of one kilogram a vertical distance of one meter in a

standard gravitational field.

metric - Having reference to: 1. Length measurements based upon

the meter, its subdivisions, or its multiples; 2. Weight mea-

surements based upon the kilogram, its subdivisions or its

multiples.

metro - An abbreviation for "Meteorological Conditions." The air

temperature, barometric pressure, gun altitude, and relative

humidity existing at the place and time of firing. See also:

standard met.

midrange - Half the distance from the muzzle to the point of aim.

midrange trajectory - The height of the trajectory at a point

halfway between the firearm's muzzle and the point of aim.

minute - 1: Angular measure equal to 1/60 angular degree.

2: 1/1440 solar day.

MOA (also M.O.A.) - Minute of angle or minutes of arc. Equal to

1/60 of an angular degree. At 100 yards, 1 MOA subtends approx-

imately 1.047 inches.

MpS (also Met/Sec) - Meters per second. A unit of velocity.

muzzle - The foremost point of a firearm's barrel; that part of a

firearm closest to the target.

muzzle energy - The energy developed by a moving projectile at the

firearm's muzzle, generally expressed in FtLbs (English mea-

sure) or MetKg (Metric measure).

muzzle velocity - The speed of a projectile at a firearm's muzzle,

generally expressed in FPS (English measure), or MpS (Metric

measure).

N.R.A. (also NRA) - National Rifle Association. If you shoot, you

should belong to this organization.

ogive - 1. The curve of a bullet's forward portion. 2. The radius

of this curve, usually expressed in calibers.

pistol - 1. Single-shot handgun. 2. Multi-shot handgun which does

not rely on a rotating mechanism to achieve multiple shots.

3. In popular usage, any handgun.

point-blank range - That range for which the bullet's path varies

from the line of sight by no more than a specified amount from

the muzzle to that range.

QBALL                  Appendix A - Glossary                    63

shotgun - Firearm of relatively large bore, having no rifling,

designed to be fired by a single individual, using both hands,

from the shoulder. A shotgun commonly discharges a number of

projectiles simultaneously, although it may also use a slug.

See also: slug.

sight height - The measured vertical distance between the hori-

zontal centerline of the sights and the axis of the bore.

sight radius - The distance between the rear and the front sight,

on those weapons equipped with iron (non-optical) sights.

sighting in - The process of firing to determine the point of im-

pact at a given range and adjusting the sights so that the

point of impact has the desired relationship to the point of

aim. See also: zero.

slant fire - That fire occurring if the point of aim is at a sig-

nificantly greater or lesser altitude than the firing point.

Generally, slant fire effects may be ignored if the vertical

angle between the imaginary line to the desired point of aim

and the horizontal is 5 degrees of arc or less, or if the

difference in altitude is less than approximately 1/20 the

range to the point of impact.

slant range - In slant fire, that distance measured from the fire-

arm's muzzle to the point of aim. Slant range is always greater

than horizontal range, thus producing the "hits high" phenome-

non experienced if the weapon's bore elevation differs signif-

icantly from the horizontal.

slug - Properly, a single projectile of near-bore diameter intend-

ed for use in a shotgun. Commonly corrupted to refer to bul-

lets, generally.

small arms (also smallarms) - Rifles, shotguns, pistols, or revol-

vers, generally; those weapons intended to be used by a single

individual, as opposed to crew-served weapons, e.g., machine

guns. QBALL deals only with weapons which use a solid projec-

tile.

sonic velocity - The speed of sound through air.

standard met - Atmospheric conditions as follows: 1. (English

measure) Temperature of 59 deg Fahrenheit, barometric pressure

of 29.53 inches of mercury, gun altitude of zero (sea level),

and relative humidity of 78 percent; 2. (Metric measure)

Temperature of 15 deg Celsius, barometric pressure of 750 mm

of mercury, gun altitude of zero (sea level), relative humid-

ity of 78 percent. Firing tables are usually corrected to

standard met conditions.

stp - Standard Temperature and Pressure. See also: standard met.

QBALL           Appendix B - Miscellaneous Information         65

Inferred Ballistic Data -- Ballistic Coefficient.

Tables B.1 and B.2 summarize the determination of a ballistic

coefficient using tabular values. The values for Table B.1 are

taken from data published in the 1990 Edition of The Gun Digest,

page 229; the values for Table B.2 were calculated using QBALL.

-------------------------------------------------------

| From Gun Digest Table for 30-06 Spfd, 180 gr bullet:|

|                          Range in Yards             |

|               ------------------------------------  |

|               Muz     100      200     300     400  |

|               ------------------------------------  |

| Velocity      2700   2469     2250    2042    1846  |

| Energy        2913   2436     2023    1666    1362  |

| Trajectory    -1.5    2.5      0.0    -9.3   -27.0  |

-------------------------------------------------------

Table B.1. Data from Gun Digest Ballistic Table.

QBALL's Infer Ballistic Coefficient routine, using 100 and 200

yards for the required ranges and the published velocities, re-

ports a ballistic coefficient (C1) of 0.384. Using this C1 and the

appropriate values for muzzle velocity, bullet weight, and zero

range, QBALL produces the data shown in Table B.2.

-------------------------------------------------------

| QBALL Data Table for 30-06 Spfd, 180 gr bullet:     |

|                          Range in Yards             |

|               ------------------------------------  |

|               Muz     100      200     300     400  |

|               ------------------------------------  |

| Velocity      2700   2470     2251    2043    1846  |

| Energy        2913   2437     2024    1668    1362  |

| Trajectory    -1.5    2.1      0.0    -9.0   -26.2  |

-------------------------------------------------------

Table B.2. Data Using Inferred C1.

The maximum variations are: Velocity, 1 FPS; Energy, 2 FtLbs;

and Trajectory, 0.8 inches. It may be seen, therefore, that QBALL

can produce results in substantial agreement with published data.

Inferred Ballistic Data -- Muzzle Velocity.

This data is from the Speer Reloading Manual #11.

-------------------------------------------------------

| From Speer Reloading Manual. 180 gr bullet, C1=.30  |

|                          Range in Yards             |

|            ---------------------------------------  |

|            Muz      50    100    150    200    250  |

|            ---------------------------------------  |

| Velocity   2400   2259   2122   1990   1863   1742  |

| Energy     2302   2039   1800   1582   1388   1213  |

| Trajectory -1.5    0.0   -0.2   -2.3   -6.7  -13.5  |

-------------------------------------------------------

Table B.3. Trajectory Data, Speer Manual #11.

QBALL           Appendix B - Miscellaneous Information         67

An examination of the data in Table B.6 indicates that remain-

ing velocities found using QBALL will be, on average, 0.47% high

relative to the Speer data, and 0.42% low relative to to the

Hornady data. The Hornady data, however, must be considered sus-

pect, as the actual ballistic coefficients available in the Hor-

nady table do not exactly match those specified. Even so, it would

appear that the QBALL velocity results are certainly within one or

two percent of published data. In practical terms, there is prob-

ably that much variation in velocity between successive rounds in

a string. Since energy is velocity-dependent, it is obvious that

the energies calculated using QBALL will be in error by similar

proportions, and in the same direction.

Error Analysis -- Trajectory.

----------------------------------------------------------------

|        Trajectory Comparisons  -  Conditions as Shown        |

|        ==============================================        |

|                   [ All at Standard Met ]                    |

+--------------------------------------------------------------+

|Range = 250 yards, zeroed at 50 yards; C1 =.12, MuzVel = 2100 |

|        Inches above/below line of sight at...(yards)         |

|        ---------------------------------------------         |

|        Muzzle     50       100       150       200       250 |

|        ------    ---      ----      ----      ----      ---- |

|QBALL    -1.5     0.0      -1.3      -6.4     -16.9     -34.3 |

|SPEER    -1.5     0.0      -1.3      -6.4     -17.0     -34.7 |

|HORNADY  -1.5     0.0      -1.2      -6.3     -16.7     -33.2 |

+--------------------------------------------------------------+

|Range = 400 yards, zeroed at 200 yards; C1 =.26, MuzVel = 2500|

|          Inches above/below line of sight at...(yards)       |

|          ---------------------------------------------       |

|          Muzzle       100        200        300        400   |

|          ------      ----       ----       ----       ----   |

|QBALL      -1.5        2.9        0.0      -13.0      -39.4   |

|SPEER      -1.5        3.0        0.0      -13.0      -39.6   |

|HORNADY    -1.5        3.0        0.0      -13.1      -39.7   |

+--------------------------------------------------------------+

|Range = 400 yards, zeroed at 200 yards; C1 =.50, MuzVel = 3300|

|          Inches above/below line of sight at...(yards)       |

|          ---------------------------------------------       |

|          Muzzle       100        200        300        400   |

|          ------      ----       ----       ----       ----   |

|QBALL      -1.5        1.1        0.0       -5.2      -15.1   |

|SPEER      -1.5        1.1        0.0       -5.2      -15.2   |

|HORNADY    -1.5        1.1        0.0       -5.3      -15.7   |

----------------------------------------------------------------

Table B.7. Trajectory Comparisons. QBALL Data rounded to 0.1 inch.

An examination of the data in Table B.7 shows that there is

substantial agreement between the QBALL results and published

data. In general, you may expect QBALL to give you excellent

practical accuracy.

QBALL           Appendix B - Miscellaneous Information         69

QBALL and Other Ballistics Programs

QBALL user Jim McCausland provided several sets of data devel-

oped with other ballistics programs. The following tables contain

the input data which Jim used, and the differences obtained run-

ing QBALL with the same input data.

----------------------------------------------------------

| Program: Sierra Ballistics.                            |

| Input Data:                                            |

|     Ballistic Coefficient (C1)....... .299             |

|     Bullet Weight.................... 130 grains       |

|     Zero Range....................... 200 yards        |

|     Maximum Range.................... 450 yards        |

|     Range Increment..................  50 yards        |

|     Sight Height..................... 0.9 inches       |

|     Crosswind........................ - no wind -      |

|     Metro............................ Standard         |

+--------------------------------------------------------+

| QBALL vs Sierra Ballistics Results                     |

|     Remaining Velocity............... 0.9 FPS          |

|     Remaining Energy................. 1 FtLb           |

|     Total Drop....................... 0.51 inches      |

|     Path/Sight Line.................. 0.17 inches      |

|     Time of Flight................... 0.000424 seconds |

----------------------------------------------------------

Table B.10. QBALL and Sierra Ballistics.

----------------------------------------------------------

| Program: BALCALC.                                      |

| Input Data:                                            |

|     Ballistic Coefficient (C1)....... .295             |

|     Bullet Weight.................... 130 grains       |

|     Zero Range....................... 200 yards        |

|     Maximum Range.................... 500 yards        |

|     Range Increment..................  50 yards        |

|     Sight Height..................... 0.9 inches       |

|     Crosswind........................ - no wind -      |

|     Metro............................ Standard         |

+--------------------------------------------------------+

| QBALL vs BALCALC Results                               |

|     Remaining Velocity............... 1 FPS            |

|     Remaining Energy................. 1 FtLb           |

|     Total Drop....................... 0.29 inches      |

|     Path/Sight Line.................. - - - - -        |

|     Time of Flight................... 0.00105 seconds  |

----------------------------------------------------------

Table B.11. QBALL and BALCALC. Available in BALCALC,

unavailable in QBALL: Bullet Momentum, Lead in feet.

QBALL           Appendix C - Selected Bibliography             71

Davis, William C., Jr. Ballistics on the Home Computer, The

American Rifleman, CXXXI, 6 (June, 1983), p. 36.

--. Ballistics on A Pocket Calculator, The American Rifleman,

CXXXV, 6 (June, 1987), p. 42.

--. Calculate Ballistic Coefficients, The American Rifleman,

CXXXVII, 3 (March, 1989), p. 44.

--. What Is The Ballistic Coefficient?, The American Rifleman,

CXXXVII, 3 (March, 1989), p. 12.

Dobson, Paul (ed.). Air Gunner '94. Wokingham [Berks, UK]: Romsey

Publishing Co. Ltd., 1993.

Federal 1992 Ammunition [Catalog and Guide]. Anoka: Federal

Cartridge Co., 1991.

Hatcher, Major General Julian S. Hatcher's Notebook. 3d ed.

Harrisburg: The Stackpole Co., 1962.

Hodgdon Basic Data Manual, Shawnee Mission: Hodgdon Powder

Company, Inc., 1987.

Pejsa, Arthur J. Modern Practical Ballistics. 2d ed. Minneapolis:

Kenwood Publishing, 1991.

Ramage, C. Kenneth (ed.). Lyman Reloading Handbook. 46th ed.

Middlefield: Lyman Products Corp., 1982.

Reiber, Ron L., and Heers, Edward A. (eds.). Hornady Handbook of

Cartridge Reloading, Rifle-Pistol. 3d ed. Grand Island: Hornady

Manufacturing Co., 1980.

Reloaders' Guide for Hercules Smokeless Powders. Wilmington:

Hercules Inc., 1988.

Remington Firearms, Ammunition, Clothing and Accessories Catalog.

Wilmington: Remington Arms Co., Inc., 1992.

Speer Reloading Manual #11, Lewiston: Omark Industries, Inc.,

1987.

Warner, Ken (ed.). Gun Digest, 1990. 44th ed. Northbrook: DBI

Books, 1989.

--. Gun Digest, 1991. 45th ed. Northbrook: DBI Books, 1990.

--. Gun Digest, 1992. 46th ed. Northbrook: DBI Books, 1991.

--. Gun Digest, 1993. 47th ed. Northbrook: DBI Books, 1992.

--. Gun Digest, 1994. 48th ed. Northbrook: DBI Books, 1993.

--. Gun Digest, 1995. 49th ed. Northbrook: DBI Books, 1994.

Winchester Ammunition Product Guide. East Alton: Winchester/Olin

Corp., 1993.

QBALL         Appendix D - Low Velocity Applications           73

Special Notes for Air Rifles, Air Pistols

and Other Low-Velocity Applications

In response to a number of requests, QBALL version 3.00 allows

muzzle velocities down to 200 FPS (61 MpS), and will compute re-

maining velocities down to ca. 100 FPS (30.48 MpS). If you are

using QBALL to compute data for an air rifle, an air pistol, or

any other low-velocity application, then you must be aware of some

very real limitations.

QBALL uses the G1 drag functions, a development of the Gavre

Commission test firings in France in 1898. These test firings were

conducted using a sharply-pointed bullet at firearm velocities,

neither of which conditions is applicable in low-velocity cases.

As a consequence, data calculated for low velocities are question-

able, at best. Applying the adage that "...some poor data is

better than no data at all," however, QBALL will give you some

idea of trajectories and energy levels for your application.

Just how close - or how far off - is "some idea..."?

If another method of developing trajectory data is available,

and if it allows velocities as low as those under consideration,

then data computed using that method and using QBALL may be com-

pared. Fortunately, such a method does exist: the Ingalls Tables,

a complete set of which is found in Hatcher's Notebook.*

While the Ingalls Tables permit a minimum velocity of 100 FPS,

Hatcher cautions that "...there is necessarily a great uncertainty

as to just exactly what the retardation is at low velocities.".**

For example, consider the QBALL input data shown in Table D.1.

-----------------------------------------------------------------

| Weapon Data                     Range Data                    |

|   Muzzle Velocity:   579 FPS      Zero Range:      30 yards   |

|   Sight Height:      0.0 inches   Maximum Range:  108 yards   |

| Pellet Data                       Range Increment: 20 yards   |

|   Ballistic Coefficient: 0.010  Metro Data                    |

|   Weight:          18.5 grains    60�F 0' 30.0" 67% (Ingalls) |

-----------------------------------------------------------------

D.1. Input Data for a Typical Air Rifle.

_____________________________

* Hatcher, Major General Julian S. Hatchers' Notebook, 3d ed.

Harrisburg: The Stackpole Co., 1962, p. 584ff.

** op.cit., p. 568.

-----

INDEX

-----

QBALL                                                           75

INDEX

A                          C continued

ABOUT SHAREWARE . . . . . .  vii  Chronograph

ACKNOWLEDGMENTS . . . . . . .  i     inferring ballistic coeff.35

Air rifles, pistols . . . . . 73     inferring muzzle velocity.42

Already Loaded! message . . . 51     minimum safe range . . . .42

AMMO.DAT. . . . . . . . .  1, 32     using single . . . . . . .38

ASP Ombudsman . . . . . .  cover  Clean Sweep . . . . . . . 5, 29

Association of Shareware          Computations menu

Professionals (ASP). . i, vii     Range table. . . . . . . . 8

Set, reset zero. . . . . . 8

Other ballistics proc. . . 8

B                    New/Changed Inputs . . . . 8

Ballistic Coefficient             Configuration

changing . . . . . . . . . 30     saving . . . . . . . . . .50

entering . . . . . . . . . .7     screen colors. . . . . . .50

example from tabular data. 65     switch toggles . . . . . .49

inferring. . . . . . . . . 35     tailoring. . . . . . . . .51

using [L]ookup . . . . . . 31     temporary. . . . . . . . .50

valid data limits. . . . . 30  Contacting author . . cover, ix

Bibliography. . . . . . . . . 71  COPYRIGHT INFORMATION . . . .ii

Bullet Diameter

in sectional density . . . 27

Bullet weight                                   D

changing . . . . . . . . . 30  D.NDX . . . . . . . . . . . .32

entering . . . . . . . . . .7  Data Entry

in sectional density . . . 27     ballistic coefficient. 7, 30

using [L]ookup . . . . . . 31     bullet [L]ookup. . . . 32-33

valid data limits. . . . . 30     bullet diameter. . . . . .27

Bullet [L]ookup                      bullet weight. . . . . 7, 30

AMMO.DAT, *.NDX needed . . .1     gun weight, dec. pounds. .27

ballistic coefficient,            gun weight, lbs/ounces . .27

weight both changed . . 33     limits see specific item

choosing input data. . . . 32           see also Metric limits

diameter displayed . . . . 32     maximum deviation. . . . .23

entering ballistic                metro. . . . . . . . . 7, 31

coefficient . . . . . . 32     muzzle velocity. . . . 7, 30

index (*.NDX) file req'd . 32     powder (propellant) wt. . 28

manufacturers. . . . . . . 33     range increment. . . . . . 8

range, maximum . . . . . . 8

range, zero. . . . . . . . 8

C                    sight height . . . . . . .31

Changing input data                  target above/below gun15, 16

ballistic coefficient. . . 30     weapon identifier. . . 7, 30

bullet identifier. . . . . 30     wind direction . . . . . .26

bullet weight. . . . . . . 30     wind velocity. . . . . . .26

clean sweep. . . . . . . . 29  DEDICATION. . . . . . . . . . i

metro. . . . . . . . . . . 31

muzzle velocity. . . . . . 30            D continues

sight height . . . . . . . 31

unit system swap . . . . . 31

weapon identifier. . . . . 30

QBALL                                                           77

M continued                             P

Menus                             P_HOLDER.PRD. . . . . . . . .45

Computations . . . . . .8, 24  Point blank range

Graphics . . . . . . . 30, 31     determining. . . . . . . .23

Infer MV using . . . . . . 39     graphics display of. . . .25

Main . . . . . . . . . . . .4     valid data limits. . .24, 34

New Input Data . . . . . . 29  Powder (propellant) weight

Other Procedures . . . . . 23     in free recoil/velocity. .27

Range Table Output . . . . 11     valid data limits. . .28, 34

Recall Trajectory Data . . 55  PRCONFIG.EXE. . . . . . . . .44

Screen color select. . . . 50  Printed output

Slant Table Output . . . . 16     common problems. . . . . .46

Switch Toggle. . . . . . . 49     graphics . . . . . . . . .22

Utilities. . . . . . . . . 49     PRCONFIG.EXE/PRINTERS.DAT 44

Metric conversion factors . . 34     printer setup, general . .43

Metric data limits. . . . . . 34     printer setup, graphics. .44

Metro                                printer warnings . . . . .43

changing . . . . . . . . . 30     range table. . . . . . . .11

standard/default . . . . . .4     samples. . . . . . . .12, 18

valid data limits. . . 30, 34     shift-printScr dumps . . .43

Most Probable Zero. . . . . . 25     slant fire table . . . . .17

Muzzle velocity                      toggle printer on/off. . .49

changing . . . . . . . . . 30     using RRTABLE.EXE. . . . .53

entering . . . . . . . .7, 30  PRINTERS.DAT. . . . . . . . .44

inferring (calculating). 39ff  Probable Zero . . . . . . . .25

valid data limits. . . 30, 34

Q

N                 QBALL and other programs. . .69

New or Changed Inputs             QBSHELIT.BAT

clean sweep. . . . . . .5, 29     Already loaded! message. .51

single inputs. . . . . . 29ff     manual delete. . . . . . .51

Nonhorizontal fire                   normal exit. . . . . . . .51

see Slant fire

Nonstandard metro . . . . . . 30                  R

Range, generally

O                    entering . . . . . . . . . 8

Obtaining latest shareware. viii     valid data limits. . . 8, 34

Opening screen. . . . . . . . .3  Range increment

Other ballistics procedures          entering. . . . . . . . . .8

free recoil vel/energy . . 28     in graphics display . . . 20

point blank range. . . . . 23     valid data limits . . . . .8

sectional density. . . . . 27  Range, zero

wind vector corrections. . 26     entering. . . . . . . . . .8

zero for range & height. . 25     finding probable. . . . . 25

Other ballistics programs            valid data limits . . . . .8

BALCALC. . . . . . . . . . 69

BALTEC . . . . . . . . . . 69            R continues

Barnes Ballistics. . . . . 70

Sierra Ballistics. . . . . 69