Jun. 1995

DE-220 SERIES ETHERNET CARD

1. PACKET DRIVER PROGRAM FOR 3Com 3+ Share

2. PACKET DRIVER PROGRAM FOR Sun MicroSystems PC-NFS

3. PACKET DRIVER PROGRAM FOR FTP PC/TCP

===============================================

*******************************************

* PACKET DRIVER PROGRAM FOR 3Com 3+ Share *

*******************************************

The D-Link 3Com 3+ Share Driver Program lets you run 3Com 3+ Share

on your PC with the D-Link DE-220 series Ethernet card.

The following files will be used in this installation:

Files          Contents

-----------   -------------------------------------------------------

DE220PD.SYS     Packet driver program for DE-220 series card

ETH.SYS         3Com 3+ Share driver program

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for

the D-Link DE-220 series 16-Bit Ethernet card to the boot

disk/diskette.

This program will replace the program of the same file name

(ETH.SYS) when you perform the installation procedure  in

step 1  above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure

you have the following two statements.  The order of these

two statement must be as shown below.

DEVICE = DE220PD.SYS <-d> <packet_int_no> <-w>

DEVICE = ETH.SYS

You may now boot and start your 3COM workstation.

*****************************************************

* PACKET DRIVER PROGRAM FOR Sun MicroSystems PC-NFS *

*****************************************************

The D-Link PC-NFS Driver Program lets you run Sun MicroSystems PC-NFS

on your PC with the D-Link DE-220 series 16-Bit Ethernet Card.

The following files will be used in this installation:

Files           Contents

-----------     ----------------------------------------------------------

PDNFS.SYS       PC-NFS Driver Program

DE220PD.SYS     Packet driver program for DE-220 series card

DE220PD.COM     Packet driver program for DE-220 series card

Two methods can be used to install:

METHOD 1:

=========

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  On the screen of the PC-NFS install program, select 3C501

(the 3Com EtherLink card) as your network interface card. When the screens

prompt you, select 3 for the interrupt level (IRQ), and select 300h for

the base I/O address.

2. Copy the following files to the \NFS directory of your hard disk:

PDNFS.SYS

DE220PD.SYS

DE220PD.COM

3. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\DE220PD.SYS <-d> <packet_int_no> <-w>

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

4. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.

METHOD 2:

=========

1. Install the PC-NFS program according to the procedure described in the

PC-NFS manual.  On the screen of the PC-NFS install program, select 3C501

(the 3Com EtherLink card) as your network interface card. When the screens

prompt you, select 3 for the interrupt level (IRQ), and select 300h for

the base I/O address.

2. Copy the following files to the \NFS directory of your hard disk:

PDNFS.SYS

DE220PD.SYS

DE220PD.COM

3. Modify the CONFIG.SYS file as follows:

DEVICE=C:\ANSI.SYS

DEVICE=C:\NFS\PCNFS.SYS /m

DEVICE=C:\NFS\SOCKDRV.SYS

DEVICE=C:\NFS\PDNFS.SYS

LASTDRIVE=V

AUTOEXEC.BAT file:

SET PATH = C:\; C:\NFS

SET TZ=EST5EDT

DE220PD <-d> <packet_int_no> <-w>

NET YPDOMAIN ypserver

NET START RDR

NET YPSET homer

NET NAME john

NET USER F: \\user\john

We use the name "john" here as an example.

4. Reboot the PC to start PC-NFS.

You may now execute the PC-NFS commands to communicate

other computers through the TCP/IP protocol.

****************************************

* PACKET DRIVER PROGRAM FOR FTP PC/TCP *

****************************************

The D-Link packet driver programs above let you run FTP's PC/TCP

with the D-Link DE-220 series Ethernet Card.  Two methods can be used:

METHOD 1:

=========

If you use the DE220PD.SYS program, the following statement must

be included in your CONFIG.SYS file:

DEVICE = DE220PD.SYS <-d> <packet_int_no> <-w>

This statement may be placed anywhere in the CONFIG.SYS file.

METHOD 2:

=========

If you use the DE220PD.COM program, follow the procedure below:

1. Execute the DE220PD.COM as follows:

>DE220PD <-d> <packet_int_no> <-w>

2. Execute the ETHDRV.EXE device driver program (provided by FTP).

You are now ready to use FTP's PC/TCP.

Notes:

<packet_int_no> is the software interrupt vector number used by the

packet driver program, which can be from 96 to 128.

<-d>            Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

<-w>            the -w parameter is for use with the Windows environment only.