Jun. 1995

DE-220 Family Series Ethernet Card

Windows for Workgroups V3.11 Driver programs

============================================

**********************************************************

*  README for Windows for Workgroups V3.11 Installation  *

**********************************************************

The DE220 Family Ethernet LAN drivers of DE-220/DE-220P/DE-220E cards for Windows for

Workgroups 3.11 contained in this diskette are as follows:

\WFW311\DE22X.386     D-Link NDIS 3.0 WFW driver for Windows for Workgroups

\WFW311\DE22X.DOS     D-Link NDIS 2.0 DOS driver for Windows for Workgroups

\WFW311\DE22X.COM     D-Link NetWare 3.X ODI driver for Windows for Workgroups

\WFW311\NET.CFG       NET.CFG file for Windows for Workgroups

\WFW311\OEMSETUP.INF  OEMSETUP file for Windows for Workgroups

\DISK1

\DISK2

Installation Procedure:

1. When a LAN driver is installed, Windows for Workgroups will present a list

of all available drivers.  Since there will be no D-Link DE-220 Family

drivers (DE22X.386, DE22X.DOS and DE22X.COM driver programs ) initially,

select UNLIST for 3rd party adapters not in the driver list.

2. Windows for Workgroups will ask for the drive/path containing the DE-220

Windows for Workgroups drivers.  Insert the D-Link driver diskette in

drive A: and respond with the following

A:\WFW311

3. Windows for Workgroups will attempt to locate an OEMSETUP.INF file in the

specified path. If you have entered the path name correctly, Windows

for Workgroups should copy the appropriate drivers to the Windows for

Workgroups system.

4. At this point, the driver would have been properly copied and the

installation would proceed to the next step.  Please refer to your

Windows for Workgroups installation guide for correct procedures.

5. After the initial installation, the DE-220 drivers will be permanently

copied to the Windows for Workgroups system.

Subsequent network driver lists will already contain D-Link DE-220

drivers.