Jun. 1995

DE-220 SERIES ETHERNET CARD

Windows '95 DRIVER PROGRAM

===================================================

*******************************************************************************

* NOTE 1: This driver kit will only work on Beta 2 or later.                  *

*                                                                             *

* NOTE 2: When install network card driver, Win95 CD ROM or floppies should   *

*         exists.                                                             *

*                                                                             *

* NOTE 3: When D-Link PnP card or D-Link PCMCIA card is first inserted, users *

*         should shutdown the Win95 and reenter Win95 to access the network   *

*         environment. For PnP card and PCMCIA card only the following 1st    *

*         item should be cared.                                               *

*******************************************************************************

1. Copy NETDLINK.INF from A:\WIN95 to C:\WINDOWS\INF if the Windows path is

in C:\WINDOWS.

2. From the Control Panel, select Network. The Network windows should appear.

3. From the Network window, click on "Add". You will then be asked to select

a component type, double click on "Adapter".

4. A list of network card manufacturers and model numbers should now be shown.

Choose D-Link manufactur and D-Link card, then click on "OK". The resource

Requirement is asked and you should choose the appropriate IoBase and IRQ.

Then Click on "OK".  The chosen network information will be shown. Then

Click on "OK".

5. Insert D-Link driver diskette to A: and locate D-Link driver in A:\WIN95.

While copying D-Link drivers, Win95 also need Win95 CD ROM to copy other

files in  D:\Retail if your CD ROM drive is D:. So you need to change

the corresponding path(A:\WIN95 or D:\RETAIL).

6. When all the setting are complete, click on "OK" on the properties page

to return to the network window. If everything looks correct here, click

on "OK" to close the network window.