Release Notes

S3ID Chip Identify Utility for DOS/Win9X, 1.00.04 (09/10/99)

Recipient

S3

Purpose

identify’s Savage Xtreme as part 398

Savage/MX/IX rev FB support to show correct mem types.

Description

The S3ID utility is a DOS utility designed to let end-users discover the type of S3 display adapter installed in their systems in order to facilitate downloading the correct driver for their adapters.  The user should run S3ID from a DOS mode or a DOS prompt in Windows 9X, and the utility reports the detected S3 adapter.

S3ID also sets its return code depending on the adapter found, so that users can use batch files and ERRORLEVEL statements to perform different actions for different adapters, if need be.

Software Requirements

DOS or Windows 9X

Language Support

Only English support is provided.

©Copyright 1999 S3 Incorporated. All rights reserved. If you have received this document from S3 Incorporated in electronic form, you are permitted to make the following copies for business use related to products of S3 Incorporated: one copy onto your computer for the purpose of on-line viewing, and one printed copy. With respect to all documents, whether received in hard copy or electronic form, other use, copying or storage, in whole or in part, by any means electronic, mechanical, photocopying or otherwise, is not permitted without the prior written consent of S3 Incorporated, P.O. Box 58058., Santa Clara CA 95052-8058. S3, S3 ON BOARD, S3d (design and word), Trio and ViRGE are registered trademarks of S3 Incorporated. The S3 Corporate Logo, Sight. Sound. Speed., Trio64V2, Savage, Savage3D, Savage4, DuoView, and Streams Processor are trademarks of S3 Incorporated. Other trademarks referenced in this document are owned by their respective companies. The material in this document is for information only and is subject to change without notice. S3 Incorporated reserves the right to make changes in the product design without reservation and without notice to its users.

Products Supported

The following are the chipsets supported/detected by S3ID, with the accompanying return code.

| Code | Chip Description |
| --- | --- |
| 0 | 911 |
| 1 | 924 |
| 2 | 801 |
| 3 | 805 |
| 4 | 805i |
| 5 | 928 |
| 6 | 928 PCI |
| 7 | Vision 864 |
| 8 | Vision 964 |
| 9 | Vision 868 |
| 10 | Vision 968 |
| 11 | Trio 32 (732) |
| 12 | Trio 64 (764) |
| 13 | Trio 64 (764) PNP |
| 14 | Trio 64 (764) PNP |
| 15 | Trio 32 (732) |
| 16 | Trio64V+(765) LPB |
| 17 | Trio64V+(765) LPB |
| 18 | Trio64V+(765) LPB |
| 19 | Trio64V+(765) CM |
| 20 | Trio64V+(765) CM |
| 21 | Trio64V+(765) CM |
| 22 | Trio 64 UV+ (767) |
| 23 | ViRGE (325) |
| 24 | ViRGE/VX (988) |
| 25 | Aurora 64V+ (M65) |
| 26 | Trio 64V2 DX (775) |
| 27 | Trio 64V2 GX (785) |
| 28 | ViRGE DX (375) |
| 29 | ViRGE GX (385) |
| 30 | Plato/PX (551) |
| 31 | ViRGE/GX2 (357) |
| 32 | ViRGE/GX2+MV (359) |
| 33 | ViRGE/GX2+ (359) |
| 34 | ViRGE/MX (260) |
| 35 | ViRGE/MXC (240) |
| 36 | ViRGE/MX+MV (280) |
| 37 | Trio3D (365) |
| 38 | Trio3D (366) |
| 39 | Savage3d (391) |
| 40 | Savage3d w/MV (390) |
| 41 | Trio3D/2X (368) |
| 42 | Savage4 LT (394) |
| 43 | Savage4 GT/Pro (395-397) |
| 44 | Savage/MX (270) |
| 45 | Savage/MX w/MV (290) |
| 46 | Savage/IX (274) |
| 47 | Savage/IX w/MV (294) |
| 255 | Chip not identified |

Additions and Changes

1.00.04

CR29173 	not identifying Savage Xtreme as part 398.

CRxxxxx 	incorrectly identifies memory size and type on Savage4s with 32-bit memory.

CR 	fixed Savage/MX/IX rev FB support to show correct mem types.

1.00.03

Changed description for 8C02 from ViRGE/MX+ (240) from ViRGE/MXC (240).

S3ID.C

As of 1.00.02

###### Resolved as Fix

Fixed swapped Savage3d part numbers (390 vs 391).

Fixed a case where program would not output anything useful after displaying register contents (on Nvidia TNT).

Changed description for 8C02 from ViRGE/MX+ (280) from ViRGE/MX+ (240).

Added 362 to Trio3D/2X part number.

Added Savage/IX (274 and 294) to supported chips.

As of 1.00.01

Added detection of Trio3D/2X, Savage3d (391), Savage4, and Savage/MX.

- Previous Changes

- None

## Application/Utility Components

- S3ID consists of only the DOS executable file S3ID.EXE.

## Registry Settings

- None.

- Known Limitations or Issues

- S3ID must be run from DOS or DOS mode in Windows 9X.  The utility will not function correctly in OS/2 or Windows NT, although it should display a warning if run in such an environment.  In OS/2 or NT, if S3ID is run directly from the GUI (i.e. not from a command prompt), you may see a command prompt appear and disappear quickly; this is normal behavior for any DOS program.

- S3ID will not detect installed adapters correctly if multiple adapters are installed in the same system.  Most likely, it will detect the primary adapter, but not the secondary one.

- S3ID cannot distinguish between most of the Savage4 chips (GT, Pro, Pro+, and Xtreme), so Savage4 chips are listed as either LT (394), or GT/Pro (395-398).

- For some adapters, there is no way to tell whether the board is using SDRAM or SGRAM, so S3ID will simply list "SDRAM/SGRAM," which means the memory can be either type.

- Resolved Issues

CR29173 	not identifying Savage Xtreme as part 398.

CRxxxxx	incorrectly identifies memory size and type on Savage4s with 32-bit memory.

CRxxxxx	fixed Savage/MX/IX rev FB support to show correct mem types.

##### Special Build Notes

Developed and compiled on Microsoft Visual C++ 1.5.  To build this project, run the BUILD.BAT file in the project directory.

Unresolved Issues

- None.

Installation Instructions

No installation procedure is required.

SQA Engineering Release Comments: Status as of Date: 09/02/99

- This version of this software product has been in test for less than 48 hours.

- Regression Status: Regression of all open issues HAS been completed.

- Acceptance Status: This utility HAS passed a limited suite of acceptance testing for the following controllers:

Savage4 - 8M SDRAM

Savage4 - 16M SDRAM

Savage4 - 32M SDRAM-PCI

Savaeg4 - 8M

Savage4 - 16M

Savage4 - 16M-32bit Memory

Savage4 - 32M

Savage4 - 16M SGRAM

Savage3D 390 - 8M SDRAM

Savage3D 391 - 8M SGRAM

SavageMX - 290 8M SGRAM (Macrovision)

Trio3D - 365 4M SGRAM

Trio3D/2X - 8M

Trio64 - 1M

Virge  - 2M and 4M

Issues Fix Verified:

CR29173 	not identifying Savage Xtreme as part 398.

CRxxxxx 	incorrectly identifies memory size and type on Savage4s with 32-bit memory.

CRxxxxx 	fixed Savage/MX/IX rev FB support to show correct mem types.

S3ID also sets its return code depending on the adapter found, so that users can use batch files and ERRORLEVEL statements to perform different actions for different adapters, if need be.

Disparities with Developer's Release Note:

None

Issues not Fixed:

None

New Issues Found:

None

Known Limitations or Issues:

S3ID must be run from DOS or DOS mode in Windows 9X.

The utility will not function correctly in OS/2 or Windows NT, although it should display

a warning if run in such an environment.

S3ID will not detect installed adapters correctly if multiple adapters are installed in the same system.