INSTALLING HS/LINK EXTERNAL PROTOCOL

IN Virtual BBS 5.4

Please refer to the RemoteAccess manual for detailed information on

installation of external protocols.  This information was tested

with HS/Link version 1.00 and Virtual BBS version 5.40.

Virtual BBS version 5.4 comes pre-installed for HS/Link.  Follow

these instructions if you wish to verify HS/Link installation, or

to make changes.  It also adds a single direction "upload" mode for

HS/Link.

Place a copy of HSLINK.EXE into your main C:\VBBS directory, or

some other directory in your PATH list.

Run VCONFIG.EXE and press 6 for DOWNLOAD PROTOCOLS.

The download definition of the HS/Link should look like this:

[DOWNLOAD PROTOCOLS]

Key Name                     Command Line (download protocols) (*=batch)

--- ------------------------ ------------------------------------------------

*H  HSLINK (Bidirectional)   HSLINK -p%1 -e%2 -UC:\VBBS\SYSOP %3

In place of C:\VBBS\SYSOP substitute your own upload directory, if

you use a different name for it.

The bi-directional transfers only work when the Download command is

used because you cannot request files to be received.  You may want

to explain to your users that they will need to describe any files

they upload while downloading AFTER the transfer is completed.

Return to the main menu of VCONFIG and select 7 for UPLOAD

PROTOCOLS.

[UPLOAD PROTOCOLS]

Key Name                     Command Line (upload protocols) (*=batch)

--- ------------------------ ------------------------------------------------

*H  HSLINK                   HSLINK -p%1 -e%2 -u%3

Note for systems running locked COM ports:

We have found that if you use a locked DTE of 9600 or above, you

will need to have a NS16550AN UART installed.  Also, for rock solid

performance you should add the -HS (Handshake Slow) switch to your

upload and download command lines.

This should get you set up in under 10 minutes.

You can edit the download and upload command lines to include

options on the HSLINK command line, but most folks won't need to

change anything.  If your system can't keep up with the COM port

while accessing the disk drive, you may want to add -HS to both

lines.  If you want to experiment, there are a number of optional

parameters you can read about in HSLINK.DOC and HSLINK.NEW.

I hope these instructions help, but remember, I can't take

responsibility for any damages resulting from these instructions.

If you get stuck, it always helps to read the DOC files that come

with RemoteAccess and with HS/Link.  If all else fails, call up the

Tool Shop and ask for help on the Support Conference.  Either I

will help you, or somebody else will know the answer and give you a

hand.

Good luck!

Samuel H. Smith

Sysop of The Tool Shop

Phone number         Modem type                Access

--------------    ---------------------   -------------------

(818) 891-3772    US Robotics HST 9600    Contributors only

(818) 891-1344    Hayes-V series 9600     Contributors only

(818) 891-6780    US Robotics 2400        Free to all