Marstek Documentation

ZSoft Corporation's MARSTEK.SYS is designed to work with the

Marstek 105, 128, 800 and M-105 hand scanners. This scanner device

driver has only two options: scanner type and DMA channel.

Scanner type:

/1     Marstek 105

/2     Marstek 128

/3     M-105, Mars 105 plus or Marstek 800 LoRes (100-400 dpi)

/4     Marstek 800 HiRes (500-800 dpi)

Note: 	Marstek 800 requires that the DIP switch underneath

the handscanner be set to '0' with the /3 CONFIG.SYS

option and to '1' for the /4 option.

DMA channel:	/C1		DMA channel 1

/C3		DMA channel 3

Example CONFIG.SYS entry:

DEVICE=MARSTEK.SYS /3 /C1

The driver is setup for the M-105 scanner and DMA channel 1.

These Marstek handscanners do not support interrupts, therefore

to obtain the best performance, MARSTEK.SYS requires EMS.

Paint Brush may save buffers to the hard disk during the scan.

When the disk is accessed, scanlines can be lost, resulting in gaps

in the image. The entire scan may be scanned to Expanded Memory and

then read back as required, greatly reducing the chance of losing

scanlines.