STANDARD MICROSYSTEMS CORPORATION

Novell NetWare 286 2.1x and 2.2 Server driver

For EtherCard PLUS Family LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

OVERVIEW

--------

To successfully complete the installation of the adapter driver

you must perform the following steps:

1.0 Hardware:

1.1    Determine the LAN adapter hardware type.

1.2    Determine the LAN adapter configuration.

1.3    Install LAN adapter hardware.

2.0 Software:

2.1    Generate Software.

2.2    Select Software Configuration.

* NetWare Generation Utility.

* Jumpers Configuration Utility.

2.3    Install LAN software.

Specific details are provided in the following sections.

INSTALLATION PROCEDURE

----------------------

1.0 HARDWARE

1.1   DETERMINING THE LAN ADAPTER HARDWARE TYPE

1.1.1 Jumper Configured Adapters

WD8003E        8K RAM

WD8003EBT     32K RAM

WD8003S        8K RAM

WD8003SH       8K RAM

WD8003WT       8K RAM

WD8013EBT     16K RAM

Note: Refer to JUMPERS.DOC in the root directory

for more information on jumpers.

1.1.2 Micro Channel Adapters

WD8003ET/A     16K RAM

SMC8003E/A     16K RAM

WD8003ST/A     16K RAM

SMC8003W/A     16K RAM

SMC8013EP/A    16K RAM

SMC8013WP/A    16k RAM

1.1.3 Software Configured Adapters

WD8003EB        8K RAM

SMC8003W        8K RAM

SMC8003WC       8K RAM

SMC8003EP       8K RAM

SMC8013EP      16K RAM

SMC8013W       16K RAM

SMC8013EW      16K RAM

SMC80013EPC    16K RAM

SMC80013WC     16K RAM

SMC80013EWC    16K RAM

1.2   DETERMINING THE LAN ADAPTER CONFIGURATION

For information on RAM, Base I/O and IRQ, see the

SYSMAP.DOC file furnished.

Also refer to User Installation guide available with the

retail packaged adapter.

1.3   INSTALLING LAN ADAPTER HARDWARE

Refer to User Installation guide available with the retail

packaged adapter.

2.0 SOFTWARE

2.1   Generate Software

2.1.1 From Floppy

When generating NetWare 286 Software from a floppy drive,

the generation software will prompt the user to insert

the driver diskette at the appropriate time.

Note: NetWare installation procedures require the driver

files to be located in the root directory of a

floppy disk whose volume label is LAN_DRV_200.

If you copy these files to another floppy disk,

be sure that you set the volume label of the new

disk to LAN_DRV_200.

2.1.2 From Hard Disk

After copying the Netware 286 floppies to the hard disk as

described in the Netware 286 manual, a directory with the

name 'LAN_DRV_.200' must be created at the same level as

the 'LAN_DRV_.001' directory. Copy all the .OBJ files to

this directory.

- For Netware 2.1x -

Copy the SMCPLUSW.LAN file to the directory 'SHGEN-1'.

Copy the SMCPLUS.LAN file to the directories 'AUXGEN'

and 'BRGEN'.  Then run either 'SHGEN' for a workstation,

'NETGEN' for a server or 'BRGEN' for a bridge.

- For Netware 2.2 -

Copy the SMCPLUSW.LAN file to the directory 'WSGEN'.

Copy the SMCPLUS.LAN file to the directories 'SYSTEM-1'

and 'ROUTEGEN'.  Then run either 'WSGEN' for a workstation,

'INSTALL' for a server or 'ROUTEGEN' for a router.

For more information on using this software, refer

to NetWare 286 Installation Manual.

2.1.3 Running Appletalk On Your Server

First determine whether the network is using AppleTalk

Phase I or Phase II.  If you are not sure, read the

APPLE.DOC file located in this directory.

AppleTalk Phase I software is incorporated in the regular

286 server driver.

AppleTalk Phase II requires a special server driver called

SMCPLS2?.OBJ(where ? is A, B, C or D) which is located in

the NETW_286 directory. These driver files must be copied

into the root of the SuperDisk or the LAN_DRV_200 directory.

Either the standard driver or AppleTalk Phase II driver, but

not both, may be in the root directory of LAN_DRV_200 when

the INSTALL or NETGEN programs are run.  If you are using

the SuperDisk, the DRIVERS.BAT file can be used to perform

the file copying.

DRIVERS.BAT copies the following AppleTalk phase 2 files

into the root directory on the SuperDisk:

SMCPLS2A.OBJ

SMCPLS2B.OBJ

SMCPLS2C.OBJ

SMCPLS2D.OBJ

SMCPLS2.LAN

The following AppleTalk phase 1 files are copied to

NETW_286 directory for safe keeping.

SMCPLUSA.OBJ

SMCPLUSB.OBJ

SMCPLUSC.OBJ

SMCPLUSD.OBJ

SMCPLUS.LAN

2.2   Select Software Configuration

Once the proper LAN driver has been selected, the desired

configuration parameters must be chosen from a menu.

This EtherCard PLUS Family LAN Adapter driver has been

designed to limit the need to reconfigure or regenerate the

IPX.COM file, even though a variety of EtherCard PLUS

Family LAN Adapters and adapter configurations may be

present in your workstations this is described in section

2.2.2.

2.2.1 Software Configured Adapters

- Workstation Software -

When one software configured LAN adapter card is

present in a workstation, choose the default

configuration 'First Software Configured Adapter'.

The shell driver will read the specific configuration

information directly from the  adapter.

If the workstation contains more than one software

configured LAN adapter, See - Server Software -

description below to determine which configuration

to use.

- Server Software -

Select 'First Software Configured Adapter' for the

first software configured LAN adapter.  Select

'Second Software Configured Adapter' for the second

software configured LAN adapter, etc.

First, Second, etc., are determined by the Base I/O

address of the adapter, i.e., if there are two

adapters one with Base I/O address 240 and the

other has an address of 280, then 240 will be the

'First Software Configured Adapter'.

2.2.2 Jumper Configured Adapters

When configuring a server or workstation employing a jumper

configured adapter, a configuration entries must be used,

which specifies a base I/O, IRQ, and RAM size which matches

those selected on the adapter. If none of the choices in the

table match the hardware configuration desired, then select

the first/default configuration entry and run Novell's

JUMPERS.EXE version v2.00 configuration program. This utility

allows the user to configure the LAN software to use any

supported hardware configuration parameters. For more

information on JUMPERS.EXE, refer to the documentation

furnished with the software. The JUMPERS utility is available

from Novell or CompuServ.

You can create an IPX.COM which will run on software

configured, Micro Channel and jumper configured adapters.

To do this choose a configuration which matches the

configuration of the jumper configured adapter. The

workstation driver always automatically detects software

configured adapters and reads the configuration information

directly from the adapter, even if the configuration

entry selected specified a different I/O base, IRQ

and RAM base.

The RAM location for jumper configured adapters is set by

the driver to the location specified in the configuration

selected.

- Selecting RAM Size -

There are configuration selections designating RAM sizes

of 8, 16 and 32 Kbytes of RAM.

Adapters with 8 Kbytes of RAM can be used with configura-

tion selections which specify 8, 16 or 32 Kbytes of RAM.

Adapters with 32 Kbytes of RAM must be used with configura-

tion selections which specify 32 Kbytes of RAM.

The EtherCard PLUS Family AT bus 16-bit adapters can function

in two modes,  16-bit mode and 8-bit mode.

In 16-bit mode, the adapter performs 16-bit data transfers

and uses the full 16 Kbytes of RAM. In 8-bit mode, the

adapter performs 8-bit data transfers and uses only 8

Kbytes of RAM.

To operate the 16-bit adapter in 16-bit mode, place the

adapter in a 16-bit bus slot and configure the driver

to a configuration selection which specifies 16 or 32

Kbytes of RAM.

To operate the 16-bit adapter in 8-bit mode, configure

the driver to a configuration selection which specifies

8 Kbytes of RAM, or place the adapter in an 8-bit bus

slot and specify any driver configuration.

NOTE:  The Default StarLAN packet type is IEEE 802.3 as of

release V3.07 of the 286 Server and Workstation

driver.  If the Novell StarLAN (AT&T StarLAN

compatible) packet type is required, it can be

changed via the JUMPERS Configuration Utility.

2.2.3 Micro Channel Adapters

- Workstation Software -

When one Micro Channel LAN adapter card is

present in a workstation, choose the default

configuration 'First Software Configured Adapter'.

The shell driver will read the specific configuration

information directly from the adapter.

If the workstation contains more than one EtherCard PLUS

Micro Channel LAN adapter, choose 'First, Second, Third

or Forth Software Configured Adapter' to select the Micro

Channel adapter that IPX.COM is to use. First, Second, etc.,

are determined by the slot number of the adapter, i.e., if

two adapters are in the machine in slots 2 and 4, then the

adapter in slot 2 is the 'First'.

- Server Software -

Select 'First Micro Channel Adapter' for the first

LAN adapter.

Select 'Second Micro Channel Adapter' for the second

LAN adapter, etc.

First, Second, etc., are determined by the slot number

of the adapter, i.e., if two adapters are in the machine

in lots 2 and 4, then the adapter in slot 2 is the

'First Micro Channel Adapter'.

NOTE:  When there is more than one adapter in a machine,

each adapter needs to be configured so that

interrupts are not shared.  Do NOT use the automatic

configuration option when initializing the adapter

from the Reference Diskette.  Automatic

configuration assumes that interrupts are shared.

Instead, initialize the adapter by selecting the Set

Configuration option from the main menu.  Set the

interrupt vector to an interrupt not already in use

by another adapter.

2.3   Install LAN software

If installing an AppleTalk driver in a NetWare 286 server,

see NetWare for Macintosh Installation Supplement.

NOTE:  When installing a NetWare External Bridge to be used

with AppleTalk phase 2, the minimum requirements for

system memory are 5 Mbytes.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.