Contents

Introduction

Features & Specifications

System Requirements

Installing the Device

Verifying the Installation

Uninstalling the Device

Disclaimer

Introduction

PC Link Cable is your smart and convenient accessory for connecting cellular phone to your USB-equipped Windows host computer. It provides a bridge connection with a 16-pin male connector on one end and a standard Type-A USB plug connector on the other end. You simply attach the 16-pin male connector onto the serial port of the cellular phone and plug the USB connector into your PC USB port. It allows a simple and easy way of adding serial connections to your PC without having to go through inserting a serial card and traditional port configuration.

PC Link Cable is ideal for connecting cellular phones to your computer. It provides serial connections up to 1Mbps of data transfer rate. And since USB does not require any IRQ resource, more devices can be attached to the system without the previous hassles of device and resource conflicts.

Finally, the PC Link Cable is a USB Specification compliant device. The PC Link Cable is designed to work on all Windows operating systems.

Features & Specifications

_ Supports cellular phones

_ Easy Plug and Play Installation

_ Full Compliance with the Universal Serial Bus Specification v1.1

_ Supports automatic handshake mode

_ Over 1Mbps data transfer rate

System Requirements

_ Windows 98, Windows ME, Windows 2000, or Windows XP operating system

_ AMD or Intel Pentium 133MHz or better based computer

_ USB 1.1 or higher port

Installing the Device

This section will guide you on how to install the driver for PC Link Cable under Windows XP, Windows 2000, Windows ME, and Windows 98 operating systems.

Note: Before you plug in the PC Link Cable for the first time, make sure that you do not attach any serial device first to the adapter. You must only plug in the

PC Link Cable itself.

The following steps will show how to install the device under Windows XP. Basically, the procedures are also somewhat the same for other Windows operating systems.

1. Power on your computer and boot to Windows.

2. Locate the USB port of your computer and plug in the USB to Serial adapter.

3. Windows will detect the new device and will initiate the Found New Hardware Wizard prompting you to install the software for the detected USB Device.

Select to install from a list or specific location. Click Next to continue.

4. When prompted for the driver location of the device, click Browse and refer to the USB to Serial driver disk or folder drive. Click Next to continue.

5. Windows should detect the driver as NEC Portable Phone. Before Windows installs this, it may prompt you that this device driver has not yet passed Windows XP Logo compatibility. Click Continue Anyway.

6. Windows will then start to install the driver for the NEC Portable Phone.

Once installation is complete, Windows will prompt you that it has finished installing the software. Click Finish to continue.

Verifying the Installation

This section will guide you on how to verify if the PC Link Cable driver was properly installed. You will also need to determine the COM port assignment made by Windows for the PC Link Cable.

Note: Before you plug in the PC Link Cable for the first time, make sure that you do not attach any serial device first to the adapter. You must only plug in the

PC Link Cable itself.

To verify if the device is properly installed and determine the COM port assignment of the device:

1. Click Start _ Control Panel _ System. Inside System Properties, click on Device Manager. Double-click on Ports (COM & LPT). You should see the PC Link Cable Comm Port (COM3) device listing. This also means that Windows has assigned the device to COM3 port.

Note: The COM port assignment of the PC Link Cable varies on your computer hardware configuration. Windows will check what COM port are unused or free in your computer and assigns that port number to the device.

2. If you want to assign the COM Port of the device to another port number, you can double-click on the device PC Link Cable Com Port (COM3) to view properties.

3. Inside the PC Link Cable Com Port (COM3) Properties, click on the Port Settings tab.

Click on the Advanced button.

4. Inside the Advanced Settings, click on the COM Port Number and check what other port numbers are unused. If it so happens that Windows assigned a COM3 to the device, you may try to reassign it to a lower unused port number. Click OK when finished. Try running HyperTerminal to test if the new port number is OK.

Note: Some software programs may only support up to COM4 and may not work if the port is assigned to COM5 or higher.

5. Once you have verified the device to be properly installed, you may now proceed to using the PC Link Cable in connecting to NEC portable Phone. Attached the serial device to the serial port of the adapter and then plug in the NEC portable Phone to the USB port of your computer.

Uninstalling the Device Driver

It is easy to uninstall the USB to Serial device driver:

1. Run the DRemover98.exe Uninstall program found on your driver utility CD or diskette.

2. The uninstall program will then prompt you if you want to remove the utility program. Click OK to continue.

3. After uninstall is complete, the program will prompt you to restart Windows.

Click Yes to continue. Make sure you remove the diskette from floppy drive.

4. Windows will also show a dialog box that the driver was removed successfully.

Wait for Windows to restart your computer.

Disclaimer

Information in this document is subject to change without notice. The manufacturer does not make any representations or warranties (implied or otherwise) regarding the accuracy and completeness of this document and shall in no event be liable for any loss of profit or any other commercial damage, including but not limited to special, incidental, consequential, or other damages.

No part of this document may be reproduced or transmitted in any form by any means without the express written permission of the manufacturer.

All brand names and product names used in this document are trademarks or registered trademarks of their respective holders.