Introduction

Section 1 explains how to install a MicroMASTER in a system

which already has an Intel Above Board in it.  If you are

installing them simultaneously please see section 2.  If you

have a MicroMASTER installed and are installing an Above Board

please see section 3.

OVERVIEW

In the early days of the PS/2 there was no easy way to install

non-IBM memory adapters in a way that the system 'saw' the

memory, so IBM issued an interim method of installing an

initializer on the first track of the hard disk (track 0).

Later IBM changed the set configuration program on the reference

disk (SC.EXE) to allow for installation of these initializers.

However, not everyone has these new set configuration programs.

Because the Adapter Description File (ADF) for the memory

adapters would cause an error if it had the new keyword in it

and was run on an old version of set configuration, some vendors

are not using the newer method, but staying with the older

interim method.  The new set configuration program also removes

initializers which it does not know about, so if two vendors use

different methods, the older method must be used after the newer

method.  This, however, causes the initializer installed later

to execute later.  Because the MicroMASTER must be initialized

after all other memory boards, the MicroMASTER initializer must

be installed last and that will not be the case if the

MicroMASTER uses the new method and the Above Board uses the

old.  Therefore Aox provides a program which will install our

initializer after all others.  Intel also provides a program,

SOFTSET, which, among other things, will install the initializer

for the Above Board.

You will note in all the sections, that the Above Board is to be

set up in extended memory mode only (non-LIM or non-EMS or

non-expanded).  LIM or EMS (expanded) memory needs a range of

addresses in the Option ROM area, i.e. address 0c0000h thru

0dffffh (just above the video memory and just below the BIOS

ROM).  The MicroMASTER uses 32k of this 128k area (1/4 of the

area).  You will find that other adapters also use these areas,

such as the ESDI, and SCSI disk adapters, as well as LAN

adapters.  Very quickly you find there is no room left in the

Option ROM area.  The Option ROM area used by the MicroMASTER is

only used by the original CPU and is not 'seen' by the

MicroMASTER, so if a LIM emulator is loaded on the MicroMASTER

side, that area of the Option ROM may be used by the LIM

emulator without adversely affecting operations.  Two LIM

emulators we at Aox have found to work well are QEMM from

Quarterdeck and 386MAX from Qualitas.

SECTION 1

Install MicroMASTER with Above Board already installed.

Before attempting to install the MicroMASTER, run SOFTSET (from

Intel) and set the amount of expanded memory to 0.  The

MicroMASTER will only work if the Above Board is in extended

mode and has no expanded memory (see the Intel manual for the

difference and the overview above if you must have expanded

memory).  This includes expanded memory for ram disks and print

spoolers.  Also, remove any calls of expanded memory drivers in

your config.sys.  These calls typically look like "device =

emm.sys".

Install the MicroMASTER per the manual, except that instead of

running Auto Configuration from your reference disk, you will

run Set Configuration and Change configuration.  (You can reach

the Set Configuration menu by pressing the "N" key when you are

prompted ("Automatically configure the system (y/n)?")

Using the navigation keys listed at the bottom of the screen,

get to the second changeable line of the MicroMASTER section in

the Change Configuration menu.  You should set "Use track0

init." to Yes (the default is No).  Save and exit out of the

configuration menu by the navigation keys on the bottom.  (Some

of the navigation keys vary from one version of the

configuration program to another.)

After your machine has rebooted, switch to the A drive and run

the Aox supplied program SETINIT from the Aox MicroMASTER option

diskette.  This will look for a file named I706F.ADF (which

needs to be on the same disk) and link it into the initializers

on track 0 of your hard disk.  If an error message saying "Could

not find the Intel Above Board 2 Plus initialization file" is

displayed, the set configuration program probably removed it.

To reinstall it, rerun SOFTSET (setting all extended, no

expanded memory).  Then rerun SETINIT.  Next time you reboot,

the MicroMASTER will start up.

If you change the amount of memory on the MicroMASTER after the

above setup, you will not have to rerun any of it.  If you must

rerun SOFTSET (because you changed the amount of memory on the

Above board) please read section 4.

SECTION 2

Install MicroMASTER and Above Board together.

Unless you have had a fair amount of experience in installing

adapters in PS/2s, it is recommended that you first install the

Above Board and then follow the procedure in section 1.  To

install the Above Board, follow the instructions in the

installation guide for the Above Board, but set the expanded

memory size to 0 (see section 1 and the overview above).  The

following is for the people with the patience of Job who have

endured many adapter installations on a PS/2.

Installation procedures for both boards will say to copy the

option diskette information onto a backup of your reference

disk.  You can do this by booting from the backup of the

reference disk and specifying copy option diskette.  Or you can

us the DOS COPY command with the file specification as "*.ADF"

(see your DOS manual if you don't know how to use the COPY

command).  With the machine off, install both boards per the

installation manuals.  When you turn on the PS/2, you should get

a 165 error.  If you have the backup reference diskette in the

drive, you will boot up from it.  If not, put it in and press

the F1 key.  Once the diskette boots, you will be prompted for

Auto configure.  Answer no, and move on to the set configuration

and then on to the change configuration.  Using the navigation

keys listed at the bottom of the screen, get to the second

changeable line of the MicroMASTER section in the change

configuration menu.  You should set "Use track0 init." to Yes

(the default is No).  Save and exit out of the configuration

menu by the navigation keys on the bottom.  (Some of the

navigation keys vary from one version of the configuration

program to another.)

After your machine has rebooted, run the Intel supplied program

SOFTSET and set the amount of expanded memory to 0.  The

MicroMASTER will only work if the Above Board is in extended

mode and has no expanded memory (see the Intel manual for a good

explanation of the difference and see the Overview of this

document if you must have expanded memory.)  This includes

expanded memory for ram disks and print spoolers.  Then run the

Aox supplied program SETINIT from the Aox MicroMASTER option

diskette.  This will look for a file named I706F.ADF (which

needs to be on the same disk) and link it into the initializers

on track 0 of your hard disk.  If an error message saying "Could

not find the Intel Above Board 2 Plus initialization file" is

displayed, SOFTSET didn't put in the Above Board initialization

so try rerunning SOFTSET (setting all extended, no expanded).

Then rerun SETINIT.  Next time you reboot, the MicroMASTER will

start up.

If you change the amount of memory on the MicroMASTER after the

above setup, you will not have to rerun any of the above.  If

you must rerun SOFTSET (because you changed the amount of memory

on the Above board) please read section 4.

SECTION 3

Install Above Board with MicroMASTER already installed.

Put your backup reference diskette in the drive and reboot your

PS/2.  During the Aox logo and sign on, press the [O] key.  This

will cause the MicroMASTER to not take over, and let the

diskette boot on the original CPU.  Move to the set

configuration menu and then on to the change configuration menu.

Using the navigation keys as listed at the bottom of the

screen, get to the second changeable line of the MicroMASTER

section in the change configuration menu.  You should set "Use

track0 init." to Yes (the default is No).  Save and exit out of

the configuration menu by the navigation keys on the bottom.

(Some of the navigation keys vary from one version of the

configuration program to another.)

Install the Above Board per the manual, choosing the auto

configuration.  When you run the Intel supplied program SOFTSET,

set the amount of expanded memory to 0.  The MicroMASTER will

only work if the Above Board is in extended mode and has no

expanded memory (see the Intel manual for an explanation the

difference and the Overview of this document if you must have

expanded memory.)  This includes expanded memory for ram disks

and print spoolers.

After installing the Above Board, switch to the A drive and run

the Aox supplied program SETINIT from the MicroMASTER option

diskette.  This will look for a file named I706F.ADF (which

needs to be on the same disk) and link it into the initializers

on track 0 of your hard disk.  If an error message saying "Could

not find the Intel Above Board 2 Plus initialization file" is

displayed, SOFTSET didn't put in the Above Board initialization

so try rerunning SOFTSET (setting all extended, no expanded

memory).  Then rerun SETINIT.  Next time you reboot, the

MicroMASTER will start up.

If you change the amount of memory on the MicroMASTER after the

above setup, you will not have to rerun any of it.  If you must

rerun SOFTSET (because you changed the amount of memory on the

Above board) please read section 4.

SECTION 4

Rerunning SOFTSET after MicroMASTER installation

Rerunning SOFTSET after the MicroMASTER has been installed

poses a problem because SOFTSET will try to put memory it

controls after other memory in the system.  As was stated

previously, the MicroMASTER must be the last in the memory map.

So the MicroMASTER must be turned off, but just pressing the o

key (return to original cpu) at power-on logo time, will not

work because the end of memory pointer has already been changed.

The way to turn off the MicroMASTER, is to run SETINIT with the

-r switch (ie SETINIT -r).  This will disable the MicroMASTER

initializer on track 0 of the hard disk.  After you reboot you

can then rerun SOFTSET (remember to set no expanded, all

extended memory).  After SOFTSET you can then run SETINIT again

(without the switch) to reenable the MicroMASTER initializer and

then reboot for the changes to take place.

Legal Notices:

Aox and MicroMASTER are trademarks of Aox, Incorporated. Intel

and Above are trademarks of Intel Corporation. IBM and PS/2 are

trademarks of International Business Machines  Corporation.

Other brand and product names are trademarks of their respective

owners.