ChgColor

--------

ChgColor is, what I think, is a useful utility for changing colors

on a PC with an EGA board.  I am placing it in the public domain.  I

am not asking for any donations or contributions.  I would like,

however, to hear any comments anyone might have about it.  I would

also like to hear about any enhancements or changes made to ChgColor.

I am registers at the following two BBSs:

The Software Society (201) 729-7410

The Pegasus Project  (201) 797-8957

ChgColor allows you to change the EGA's color palette registers.

Although the EGA has 256 colors available you may only use 16 of them

at any one time.  ChgColor allows you to change the default colors for

for each of the 16 registers.  You could change what is normally

displayed as WHITE to any other color you wish.  These color will stay

in effect in any of the programs you run including DOS.

ChgColor leaves a part of itself resident.  This had to be done

because some programs and BIOS calls change the palette registers back

to the factory default.  ChgColor intercepts the INT 10h (BIOS video)

interrupt and checks to see if a palette change or a video reset was

requested.  If it was, the registers are then reset to the user

selected colors.

The usage of ChgColor is as follows:

chgcolor reg:color reg:color reg:color ...

where reg is a valid register number from 0 to 15

0                    Black

1                    Blue

2                    Green

3                    Cyan

4                    Red

5                    Magenta

6                    Brown

7                    Light Gray

8                    Dark  Gray

9                    Light Blue

10                   Light Green

11                   Light Cyan

12                   Light Red

13                   Light Magenta

14                   Yellow

15                   White

where color is a valid EGA color from 0 to 255.

chgcolor with no parameters will reset the palette registers

to their factory defaults.

The semi-colon between reg and color is used for clarity, a

space will work just as well.

The assembler source code has been included for anyone who

happens to be interested. Enjoy.

John Toledo

312 E26th St.

Paterson, NJ 07514

(201) 523-6048