Tandy Sound Programming Information

compiled by

John A. Ball    (john.ball@two-t.com)

2398 Victoria Street

Prince George, B.C.

V2L 2M2 Canada

August 15, 1994

The Tandy 1000s and 2500s contain a complex sound generator which has 4

channels of sound, 3 that produce square waves and 1 that produces white

noise. The original Tandy 1000s contained a Texas Instruments SN76496 chip

but the 2500s contain a chip with additional sound capabilities (digital

sound playback and recording). The volume of each channel can be adjusted

and all 4 channels can be used at the same time. Each channel will continue

to play until the pitch is changed or the sound volume is turned off.

In the 1000s the sound chip can not be heard until the sound output is

selected by setting bits 6 & 5 high of i/o port address H61, i.e. use the

BASIC statement OUT &H61, &H6C . The commands and data for the sound chip

are then sent to i/o address HC0.

In the 2500s the sound chip output does not have to be enabled and the

commands and data are sent to i/o address H1E0. Note that addresses H1E1,

H1E2, and H1E3 will also program the sound chip.

SN76496 Registers

Reg  Used for                   Data/Command Bits

0   Tone 1 Frequency            1|000|XXXX Tone lower 4 bits

Tone 1 Frequency            0 0XX XXXX Tone upper 6 bits

1   Tone 1 Attenuation (vol)    1|001|XXXX Attenuation 0-15 15 is no sound

2   Tone 2 Frequency            1|010|XXXX Tone lower 4 bits

Tone 2 Frequency            0 0XX XXXX Tone upper 6 bits

3   Tone 2 Attenuation (vol)    1|011|XXXX Attenuation 0-15 15 is no sound

4   Tone 3 Frequency            1|100|XXXX Tone lower 4 bits

Tone 3 Frequency            0 0XX XXXX Tone upper 6 bits

5   Tone 3 Attenuation (vol)    1|101|XXXX Attenuation 0-15 15 is no sound

FB Shift  Periodic Noise with frequency

6   Noise Control               1|110|00|00     N/512

1|110|00|01     N/1024

1|110|00|10     N/2048

1|110|00|11     Tone 3

FB Shift  White Noise with frequency

1|110|01|00     N/512

1|110|01|01     N/1024

1|110|01|10     N/2048

1|110|01|11     Tone 3

7   Noise Attenuation (vol)     1|111|XXXX      Attenuation 0-15

where N= 3,579,545 and the frequency of the sound channel is

f = 3,579,545 / (32*COUNT) where COUNT is a 10 bit number (0-1023)

In addition to the complex sound generator chip the 2500 series and

the 1000SL, have the additional feature of being able to

record sound and play them back with a 8 bit digital to analog

convertor (DAC). This feature is programmed using i/o addresses

HC4 to HC7 in the 1000sl and in the 2500 series H1E4 to H1E7. The

DAC also can be used as a tone generator with a programmable

waveshape.

Port H1E4 Write

01E4    Bit 1   Bit 0   DAC Function Selected

0       0       Joystick

0       1       Sound Channel

1       0       Successive Approximation

1       1       Direct Write to DAC

Bit 2   Enables DMA for SA/DA bus direct writes

Bit 3   Clears DMA interrupts

Bit 4   Enables DMA EOP Interrupt

Bit 5   Sound Divider Sync Enable

Bit 6   Sound Chip Extra Divide Enable

Bit 7   Reserved

Port H1E4 Read

Bit 3   DMA Interrupt Flag set to 1 when DMA done

Bit 7   Successive Approximation Done

Port H1E5 Write Waveshape Model Select

Bit 2 Bit 1 Bit 0       Duty Cycle

0     0     0           6.25%

0     0     1           12.5%

0     1     0           18.75%

0     1     1           25.0%

1     0     0           31.25%

1     0     1           37.5%

1     1     0           43.75%

1     1     1           50.0%

Bit 7 Bit 6     Waveshape Selected

0     0         Pulse

0     1         Ramp

1     0         Triangle

1     1         Reserved

Port H1E5 Read DAC Registers

Direct Read of DAC (DF1,0 = 1X bin)

Direct Read of Control Register (DF!,0 = 01 bin)

Port H1E6 Read/Write Frequency LSB for DAC sound Channel

Bit     Function

7       F7

6       F6

5       F5

4       F4

3       F3

2       F2

1       F1

0       F0

Port H1E7 Read/Write Frequency MSB / Amplitude for DAC sound Channel

Bit     Function

7       Sound Amplitude 3   maximum level is 111X XXXX

6       Sound Amplitude 2   approximately 3 db per level

5       Sound Amplitude 1   000X XXXX will have no sound

4       Reserved

3       F11

2       F10

1       F9

0       F8

The actual frequency is 111.86 kHz divided by the number in the

sound frequency register(s).

Note also that the sound chip can be disabled by writing a 0 to

Bit 4 of i/o port H1F9. Before this bit can be written, the

configuration enable bit must be set in configuration register

H073 bit 6. There is also a miscellaneous register H1FB that will

disable the sound chip if bit 2 is 0.

The Tandy 2500SX has Bios support for the PSSJ which contains the

equivalent of a TI 76569 Complex Sound Generator Chip and a DAC/ADC

chip which can also be used as a tone generator. The DAC/ADC

supports DMA and interrupt operation for the transfering of 8 bit

data between memory and the speaker and microphone circuits.

Interrupt 1Ah   Function 7f00   Get Sound Chip ID

mov AX,07F00h   ;Get Sound Chip ID Info on stack

int 1ah         ;Time of day/sound interrupt

Returns

Interrupt 1Ah   Function 7f01   Get Sound Chip ID

mov Ax,07F01h   ;Get Sound Chip ID

mov dx,seg buffer

mov es,dx

mov bx,buffer

int 1ah         ;Time of Day/sound Interrupt

Returns string containing sound chip id to es:bx

Interrupt 1Ah   Function 80     Set Sound Source (Tandy 1000s PC Jr only)

mov AX,08000h   ;Select 8253 timer channel2

int 1Ah         ;Time of Day Interrupt

Parameters      AL selects the sound source

00 = 8253 timer chip channel 2

01 = Cassette interface

02 = Audio In on I/O board

03 = Sound Generator Chip

Returns Nothing

Interrupt 1Ah   Function 81     Get DAC starting address

mov AX,08100h   ;Get DAC starting Address

int 1Ah         ;Time of Day Interrupt

Returns the starting address of the DAC/ADC sound chip

Parameters      This function has no parameters.

Return Value    The AX register contains the dac address (1e4 for the

2500sx).

Interrupt 1Ah   Function 82     Record Sound

mov AX,08200h   ;Record sound

mov bx,buffer   ;Starting Address of buffer

mov cx,count    ;Number of bytes to record (max ffff)

mov dx,seg buffer

mov es,dx       ;Place Buffer segment in ES

mov dx,0145h    ;Count which determines sample frequency

int 1a          ;Time of Day/Sound Interrupt

Returns ?

Parameters      es:bx is the buffer address

cx is number of samples with 64k maximum

dx is the value required for the number of samples/second

dx = 3.58 Mhz / frequency of samples i.e. 145h for 11,000

samples per second.

Interrupt 1Ah   Function 83     Play Sound

mov AX,083h     ;Play Sound

mov al,07       ;Sound volume 0-7

mov bx,buffer   ;Starting Address of buffer

mov cx,count    ;Number of bytes to record (max ffff)

mov dx,seg buffer

mov es,dx       ;Place Buffer segment in ES

mov dx,0145h    ;Count which determines sample frequency

int 1a          ;Time of Day/Sound Interrupt

Returns ?

Parameters      es:bx is the buffer address

cx is number of samples with 64k maximum

dx is the value required for the number of samples/second

dx = 3.58 Mhz / frequency of samples i.e. 145h for 11,000

samples per second.

Interrupt 1Ah   Function 84     Stop Sound Playing/Recording

mov AX,08400h   ;Stops DMA output/input

int 1a          ;Time of Day/Sound Interrupt

Returns ?

Parameters      None known

Deskmate Sound/Music File Formats

Original version of Deskmate .SND files

00       1A used to terminate ASCII files

01       Compression 0 = none

02       Number of Instruments in file

03       Instrument Number

04 to 0D Instrument/Sound Name

0E to 0F Sampling Rate

10       Pitch A1 to    (01-  )

11       Pitch Flag (0 = no pitch set, -1 = pitch set)

12       Lower Range A1 (01-3F)

13       Upper Range B6 (01-3F)

14       Start of sound Block

18       Length of sound block

1C       Unknown

20       # of samples

24 to 27 Start of sustain region

28 to 2B End of sustain region

Last version of Deskmate .SND files

00 to 8 Sample Name

2C to 2D 1A 80 ?

2e       Number of Recordings in File

30       Voice (Sample Number)

42       Compression (0=none, 1=music, 2=speech)

58 to 59 Sampling Rate of Sound Clip i.e. 2AF8 for 11,000 samples

per second and 55F0 for 22,000

72       Link to next Sound Sample Record

78       Pitch A1 to    (01-  )

79       Pitch Flag (0 = no pitch set, -1 = pitch set)

7A       Lower Range A1 (01-3F)

7B       Upper Range B6 (01-3F)

7C to 7F Start of Data (A0 or FC)

80 to 83 Length of Sample in bytes

84 to 87 Number of 8-bit Samples (Same as Sample length for

uncompressed files).

88 to 8B Start of sustain region

8C to 8F End of sustain region

A0 to ?? Sound Samples with 80 being no sound (Sound Files Only)

Music Files

AA to AD Start of Second Block of Data

AE to B1 Length of Sample in bytes

B2 to B5 Number of Samples

D8 to DB Start of Third Block of Data

DC to DF Length of Sample in Bytes

E0 to E3 Number of Samples

FC to ?? Sound Samples

Errors or additional information should be sent to me at the above

address. Thanks and I hope this information helps your programming

efforts.

John A. Ball    (john.ball@two-t.com)

2398 Victoria Street

Prince George, B.C.

V2L 2M2

Canada

Sources of Information include

InterTAN Canada                 Radio Shack

Computer Support Services       Computer Services 7880

279 Bayview Drive               400 Atrium-One Tandy Center

Barrie, Ontario                 Fort Worth, Texas 76102

L4M 4W5

"Graphics and Sound for the Tandy 1000s and PC Compatibles"

by William Barden, JR.   cat no. 25-1512

Thanks to Elliott Godson, Rose Funari, George Good, and

Microsoft for including Debug with DOS